***** File HMCGUIDE.DOC

MPAE-W-55-90-19

The preparation of the Halley Multicolour Camera

encounter night data set for submission to the

International Halley Watch.

by

N. Thomas (1), W. Curdt (1), G. Schwarz (2), H.U. Keller (1).

1 Max-Planck-Institut fuer Aeronomie                    2 DLR-WT-DA-PK,

Postfach 20,                          Deutsche Forschungs- und

D-3411 Katlenburg-Lindau                 Versuchsanstalt fuer Luft- und

Federal Republic of Germany.                         Raumfahrt,

Oberpfaffenhofen,

D-8031 Wessling,

Federal Republic of Germany.

SUMMARY

Data obtained by the Halley Multicolour Camera (HMC) from on board the

European Space Agency's (ESA) spacecraft, Giotto, on the night of the 13th

March 1986 have been prepared for submission to the archive of the

International Halley Watch (IHW).  This document briefly describes the

calibration status of the submitted data, the form in which the data have been

submitted and the FITS header entries accompanying the data themselves.  Some

data acquired on encounter night have been omitted from the set submitted to

IHW.  A brief explanation and a description of these data are included.

INTRODUCTION

Purpose

The European Space Agency's (ESA) Giotto spacecraft made its closest

approach to comet P/Halley at 00:03:01.84 (+/-0.20) UT on March 14th 1986

(Curdt et al., 1988).  On board, the Halley Multicolour Camera (HMC) was the

only remote sensing experiment studying the nucleus and its environment.  HMC

operated for more than three hours prior to closest approach (CA) taking 2304

images before a malfunction occurred 9 seconds before CA.  It had previously

been agreed that the data obtained by the Giotto spacecraft during the fly-by

would be made available to the International Halley Watch (IHW) for inclusion

in its Halley archive.  The objectives and structure of IHW have been

described by the IHW staff (1985) and by Edberg et al. (1987).  This document

describes the HMC experimentors' contribution to the archive.

Instrumentation

The design of HMC has been described in depth by Schmidt et al. (1986)

and Keller et al.  (1987) and only details necessary for the evaluation and

accurate description of the data are presented here.  HMC was a modified

Ritchey-Chretien type telescope with 2 584x390 Texas Instruments charge-

coupled devices (CCDs) located in the focal plane.  Each CCD was divided into

2 sections.  CCD1 contained detectors B and C while CCD2 contained detectors D

and E.  HMC operated in a line scan mode from on board the spinning spacecraft

and could take an image every spin (approx. every 4s).  For the vast majority

of the encounter, HMC operated in single detector mode (SDM) during which all

transmitted images originated from detector C.  For the last 5 minutes before

closest approach, HMC operated in multi-detector mode (MDM) during which time

four images were made (one from each detector) every spin and subsequently

transmitted to ground.  Hence, for MDM the image identifier (image id) which

was incremented every spin does not provide a unique identification of the

image.  For images made during this period (from image id 3436 to 3503), the

detector must also be specified.

The analogue to digital (A/D) converter in HMC provided images with 4096

levels.  Three data compression techniques were adopted to bring the total

data transmission to within acceptable levels.  Firstly, all images were

"square-root encoded" which, in combination with a two level gain switch,

provided a dynamic range of 14 bits in 8 transmitted bits.  Secondly, specific

sections of each image could be selected by the onboard software for

transmission to ground resulting in image formats such as 98 x 98 or 74 x 74

(the latter in MDM for example).  Finally, images could be taken in various

"superpixel" formats.  In this case a group of pixels were summed on the CCD

to form one superpixel for subsequent transmission to ground.  Improved signal

to noise is achieved in this technique at the cost of inferior spatial

resolution.  Information is included in the FITS header which specifies the

data compression technique adopted for each individual image.

THE DATA SET

In total 2304 images were returned on encounter night between 20:50 and

00:03 UT.  Of these images, a total of 2017 are present in the data set

submitted to IHW.

Omitted data

Images taken in "photometer" mode have not been submitted.  These data

were obtained by using the spin of the spacecraft to scan the sky while the

CCD remained unclocked.  They therefore have one dimensional spatial

information but each pixel contains the integrated intensity from some portion

(depending upon the exposure time) of an annulus on the sky.  These data would

be useful for this purpose (particularly when taken through the narrow-band

filters because of the significantly higher exposure time) were it not for the

straylight entering the optics of the camera when HMC was on the sunward side

of the spacecraft.  No effort has been made to reduce this data and its

scientific usefulness is assumed to be negligible.

The last three image sets returned in multi-detector mode (MDM)

immediately prior to the power disturbance which terminated operations before

closest approach are also excluded.  Image set 3504 does contain useful data

but is corrupted and requires manual reduction.  This task has not been

completed at this time.  Image sets 3505 and 3506 are also corrupted and

probably do not contain useful image data.

Seven images taken at the beginning of the encounter sequence (image ids

674 to 680) were not correctly converted by the telemetry conversion routine.

These images are not currently in the HMC database system and are therefore

not included in the IHW data set.  The similarity between these data and the

subsequent data probably ensures that, for scientific evaluation of HMC data,

their omission is of little or no importance.

One image (3142) has been omitted because it does not have an associated

header.

Available data

The total numbers of images taken in each superpixel format (SPF) in the

IHW data set are shown in Table I together with the size of each superpixel

and the size of the output image in superpixels.  For the encounter night data

set, images taken in SPFs 2, 3, 4, and 5 showed the complete CCD at all times.

SPF1 images were only used in MDM and provided a 37 x 37 superpixel image.

For the MDM set, one image (usually detector C image) was in SPF0 while the

other 3 were in SPF1.   In SPF0, the image format varied as shown in Table II.

Red, blue and clear fixed filters covered detectors B, D, and E

respectively.  Detector C was exposed through a filter wheel with 11 filters

and polarizers.  The total numbers of images taken through each filter are

shown in Table III together with the effective wavelengths and bandwidths of

the configurations for single detector mode (from Thomas and Keller, 1989).

Table I

=========================================================================

| Superpixel | Superpixel size in | Number of available | Image size in |

|   format   |  original pixels   |       images        |  superpixels  |

-------------------------------------------------------------------------

|     0      |       1 x  1       |         882         |  see Table II |

|     1      |       2 x  2       |         204         |    37 x 37    |

|     2      |       4 x  4       |          38         |    98 x 73    |

|     3      |       8 x  8       |         171         |    49 x 36    |

|     4      |      16 x 16       |         394         |    25 x 18    |

|     5      |       4 x  3       |         328         |    98 x 97    |

=========================================================================

Table II

============================================

| Image format in SPF 0 | Number of images |

--------------------------------------------

|        74 x  74       |        68        |

|       392 x 292       |       131        |

|       368 x  26       |       210        |

|        34 x 276       |       209        |

|        36 x  36       |       132        |

|        98 x  98       |        93        |

|       196 x 196       |        39        |

============================================

Table III

=================================================================

| Filter | Number of | Effective  | Effective | Multiplication  |

|        |  images   | wavelength | bandwidth | factor for FITS |

|        |           |    [nm]    |    [nm]   |   conversion    |

-----------------------------------------------------------------

|  clear |    893    |   652.9    |   372.6   |        10       |

|   red  |    177    |   813.0    |   165.0   |        10       |

| orange |    109    |   645.4    |    94.0   |        10       |

|  blue  |    175    |   440.0    |   101.1   |        10       |

| cont.1 |    172    |   457.4    |    20.30  |       100       |

| cont.2 |    174    |   738.1    |    37.42  |       100       |

|   p-   |     42    |     -      |     -     |        10       |

|  pII   |     43    |     -      |     -     |        10       |

|   oh   |     83    |   314.8    |    12.25  |       100       |

|  c-2   |     74    |   408.4    |    16.63  |       100       |

|  c-3   |     75    |   509.5    |    20.90  |       100       |

=================================================================

REDUCTION

The method used to reduce the data will be fully described in an ESA

special publication (ESA SP-1127) by Keller et al. (1990).  The expansion from

square-root encoding has been completed for all images.  Subsequently, all

images were corrected for dark current and corrected for varying responsivity

across the CCD (flat-fielded).  Removal of energetic particle events has been

performed.  Coherent noise subtraction has been completed in detail for SPFs

0, 1, and 5.  Other images (particularly low signal SPF 2 data) do show

interference effects.  The scientific content of many of these images is

relatively low and as such, coherent noise removal from these images was

assigned a low priority.  All images have been converted into absolute units

of [mW m**(-2) sr**(-1)].

The geometrical correction of the data (necessary because of the spin of

the spacecraft) is a complex and computationally time consuming operation.

For submission of data to IHW, several decisions were taken to facilitate

interpretation of the data.  Firstly, all SPF images have been expanded and

geometrically corrected.  The expansion was performed using bilinear

interpolation.  Strictly speaking, the data should have been decompressed by

simple expansion of each individual superpixel, a process which would not

interpret the information content of each superpixel.  However, the

geometrical correction of the data (which itself involves a bilinear

interpolation) was considered a necessary part of the reduction for submission

to IHW.  The bilinear interpolation for the geometry combined with a simple

expansion of the SPF data would have resulted in non-physical rhomboid

structures (corresponding to each superpixel in the original CCD data) with

rounded edges in the images.  Consequently, bilinear interpolation for all

processing steps was selected.

Due to the geometrical correction, images are not rectangular but "pie-

shaped".  Some pixels in the rectangular arrays are therefore not image

pixels.  All non-image pixels have been set to -32768.  Similarly, image

pixels corresponding to areas on the CCD which contain erroneous data (e.g.

the unmasked area) have also been set to -32768.

The orientation of the images was the subject of considerable debate and

several conventions were proposed.  It was decided to leave the images in the

orientation generated by the geometrical correction procedure and to specify

the phase of the observation in the accompanying FITS header.  The spin phase

of the spacecraft at the time of image taking varied considerably through the

encounter and care should be taken during the interpretation process.

FITS FORMAT CONVERSION

Conversion in FITS format was performed by a batch program.  Data were

written onto 5 magnetic tapes at 6250 bpi.  A test was performed by reading

the data tapes using the Munich Image Data Analysis System (MIDAS) software

(November 88 version).  In all cases the first file is a null file.  Remaining

records appear to be correct.

INTENSITY SCALING

For the reduction and analysis of data at Lindau, all HMC data is

maintained in single precision floating point format.  For FITS format

however, data should be in integers.  To maintain the correct degree of

precision, the broad-band images have been multiplied by scaling factors

before conversion to integer format.  For broad-band filter images, the

background level has been estimated to be accurate to better than 0.2

mW m**(-2) sr**(-1) (the statistical noise is several fold higher, of course).

Thus, these images have been multiplied by 10 before conversion into FITS.

For the narrow band filter images, which contain little or no signal,

multiplication factors ensuring the required accuracy have been assumed.  The

multiplication factors adopted, which are filter dependent, are summarized in

Table III.

IMAGE ORIENTATION

In order to display the data in the correct orientation, the first

record in any FITS file should be at the bottom of the image display.  The

projected direction of the Sun in the image plane can be calculated from

the PHASE keyword in the FITS header as described below.  As a guide, the

direction of maximum emission at distances greater than about 100 km from

the nucleus was directed between 35 and 55 degrees south of the projected

direction to the Sun.

HEADER INFORMATION

There are 43 header entries in the FITS header accompanying each image.

The contents of the header has been designed to define unambigiously the

viewing geometry, resolution, and orientation of the images.  Each header

entry is described in detail in Table IV.

Table IV

=============================================================================

|    | Header entry |                      Description                      |

|    |     name     |                                                       |

-----------------------------------------------------------------------------

|  1 |   SIMPLE     |                                                       |

|  2 |   BITPIX     | Number of bits per pixel.                             |

|  3 |   NAXIS      | Number of dimensions of image.                        |

|  4 |   NAXIS1     | Number of columns.                                    |

|  5 |   NAXIS2     | Number of lines.                                      |

|  6 |   IMAGE-ID   | The original image identifier number.                 |

|  7 |   TIME-ENC   | The time to encounter in seconds.  The distance to    |

|    |              | the target plane and the spacecraft-comet distance    |

|    |              | can be calculated from this using the relative        |

|    |              | velocity (68.373 km s**(-1)) and the spacecraft-comet |

|    |              | distance at closest approach (596+/-2 km) (see Curdt  |

|    |              | et al., 1988).                                        |

|  8 |   SCALE      | The scale of the image in [km px**(-1)].              |

|  9 |   SENSOR     | The detector used to acquire the image.  This         |

|    |              | together with IMAGE-ID uniquely identifies each       |

|    |              | image.                                                |

| 10 |   FILTER     | The filter used.                                      |

| 11 |   SUPERPIX   | The superpixel format used for this image (see Table  |

|    |              | I).  This taken with SCALE defines the spatial        |

|    |              | resolution of the image.                              |

| 12 |   TDI        | The clocking rate for the CCD (TDI = time delay and   |

|    |              | integration).  The TDI time [microsec.] combined with |

|    |              | the number of exposed CCD lines in the line scan      |

|    |              | operation defines the exposure time for an extended   |

|    |              | source.  The number of exposed lines for each         |

|    |              | detector section is shown in Table V.                 |

| 13 |  PHASE       | Because of the spin of the spacecraft and the method  |

|    |              | of image taking, the projection of the sun vector in  |

|    |              | the image plane depends upon the precise timing of    |

|    |              | the clocking of the CCDs.   The timing of HMC image   |

|    |              | was expressed in terms of a phase which represents    |

|    |              | the spin phase of the spacecraft at the time of image.|

|    |              | This, in turn, can be related to the projection of    |

|    |              | the sun vector on the image by the formula            |

|    |              |        AZIMUTH = PHASE - 103.14  [deg]                |

|    |              | where AZIMUTH is the angle between the horizontal     |

|    |              | axis of the image and the direction of the sun vector.|

|    |              | The angle is measured clockwise from the left.        |

| 14 |   RIGHTAS    | The right ascension of the observation as seen from   |

|    |              | the spacecraft in the 1950.0 co-ordinate system.      |

|    |              | This refers to the central pixel (column 200, line    |

|    |              | 147) of detector C.  Entries 14 and 15 are based on   |

|    |              | the attitude and fly-by geometry determined from HMC  |

|    |              | data.                                                 |

| 15 |   DECLIN     | The declination of the observation as seen from the   |

|    |              | spacecraft in the 1950.0 co-ordinate system.  This    |

|    |              | refers to the central pixel (column 200, line 147) of |

|    |              | detector C.                                           |

| 16 |   POS-HAL1   | The x component of the position of the centre of the  |

|    |              | nucleus of comet Halley in barycentric equatorial     |

|    |              | 1950 co-ordinates (EME50) (Hechler and Jappe, 1986).  |

|    |              |                                                       |

|    |              | From this and the following components for the Earth, |

|    |              | Sun, Giotto, and comet Halley one can compute the     |

|    |              | viewing directions.  Note that the values RIGHTAS and |

|    |              | DECLIN refer to the centre of the CCD, not to the     |

|    |              | comet.  The position of the central pixel on the CCD  |

|    |              | can be estimated on full-frame images by knowing that |

|    |              | the central line (or row) is 375 pixels in length     |

|    |              | (after removal of invalid pixels) and that the first  |

|    |              | valid pixel is in column number 16.                   |

|    |              |                                                       |

|    |              | Entries 16 to 39 are original data sets provided by   |

|    |              | ESA. The approach geometry according to this data set |

|    |              | differs slightly from the HMC model.                  |

| 17 |   POS-HAL2   | The y component of the position of the centre of the  |

|    |              | nucleus of comet Halley in barycentric equatorial     |

|    |              | 1950 co-ordinates (EME50).                            |

| 18 |   POS-HAL3   | The z component of the position of the centre of the  |

|    |              | nucleus of comet Halley in barycentric equatorial     |

|    |              | 1950 co-ordinates (EME50).                            |

| 19 |   VEL-HAL1   | The x component of the velocity of the centre of the  |

|    |              | nucleus of comet Halley in barycentric equatorial     |

|    |              | 1950 co-ordinates (EME50).                            |

| 20 |   VEL-HAL2   | The y component of the velocity of the centre of the  |

|    |              | nucleus of comet Halley in barycentric equatorial     |

|    |              | 1950 co-ordinates (EME50).                            |

| 21 |   VEL-HAL3   | The z component of the velocity of the centre of the  |

|    |              | nucleus of comet Halley in barycentric equatorial     |

|    |              | 1950 co-ordinates (EME50).                            |

| 22 |   POS-GIO1   | The x component of the position of the Giotto         |

|    |              | spacecraft in barycentric equatorial 1950 co-         |

|    |              | ordinates (EME50).                                    |

| 23 |   POS-GIO2   | The y component of the position of the Giotto         |

|    |              | spacecraft in barycentric equatorial 1950 co-         |

|    |              | ordinates (EME50).                                    |

| 24 |   POS-GIO3   | The z component of the position of the Giotto         |

|    |              | spacecraft in barycentric equatorial 1950 co-         |

|    |              | ordinates (EME50).                                    |

| 25 |   VEL-GIO1   | The x component of the velocity of the Giotto         |

|    |              | spacecraft in barycentric equatorial 1950 co-         |

|    |              | ordinates (EME50).                                    |

| 26 |   VEL-GIO2   | The y component of the velocity of the Giotto         |

|    |              | spacecraft in barycentric equatorial 1950 co-         |

|    |              | ordinates (EME50).                                    |

| 27 |   VEL-GIO3   | The z component of the velocity of the Giotto         |

|    |              | spacecraft in barycentric equatorial 1950 co-         |

|    |              | ordinates (EME50).                                    |

| 28 |   POS-SUN1   | The x component of the position of the Sun in         |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 29 |   POS-SUN2   | The y component of the position of the Sun in         |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 30 |   POS-SUN3   | The z component of the position of the Sun in         |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 31 |   VEL-SUN1   | The x component of the velocity of the Sun in         |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 32 |   VEL-SUN2   | The y component of the velocity of the Sun in         |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 33 |   VEL-SUN3   | The z component of the velocity of the Sun in         |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 34 |   POS-EAR1   | The x component of the position of the Earth in       |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 35 |   POS-EAR2   | The y component of the position of the Earth in       |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 36 |   POS-EAR3   | The z component of the position of the Earth in       |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 37 |   VEL-EAR1   | The x component of the velocity of the Earth in       |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 38 |   VEL-EAR2   | The y component of the velocity of the Earth in       |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 39 |   VEL-EAR3   | The z component of the velocity of the Earth in       |

|    |              | barycentric equatorial 1950 co-ordinates (EME50).     |

| 40 |   FI-COL     | First image column on detector.  This and the         |

|    |              | following three entries identify the position of the  |

|    |              | image relative to the reference system based on       |

|    |              | column 200 line 147 of the CCD.                       |

| 41 |   LA-COL     | Last image column on detector.                        |

| 42 |   FI-LINE    | First image line on detector.                         |

| 43 |   LA-LINE    | Last image line on detector.                          |

=============================================================================

Table V

================================================

| Detector |    Filter     | Number of exposed |

|  section |               |     CCD lines     |

------------------------------------------------

|     B    |  red (fixed)  |         6         |

|     C    |    variable   |         6         |

|     D    |  blue (fixed) |         8         |

|     E    | clear (fixed) |         4         |

================================================

COPYRIGHT

The HMC images submitted to IHW are intended for use by the scientific

community.  Persons may use and study the data for scientific purposes free of

charge.  Copyright for the images remains with the Max-Planck-Institut fuer

Aeronomie and their publication or use for non-scientific purposes or for

financial gain without written permission from the experiment principal

investigator (Dr. H.U. Keller) is not permitted.

ACKNOWLEDGEMENT

The authors thank their co-investigators on the Halley Multicolour Camera

team and the many engineers involved in the design and development of the

instrument for their dedicated work during the course of the project.  We

should also like to acknowledge the contributions of the ESA project team and

the project scientist to the successful completion of the HMC experiment

within the Giotto mission.  The experiment was funded by BMFT, MPG, DFVLR,

CNES, SPPS, CNR/PSN and NASA.

REFERENCES

Curdt, W., Wilhelm, K., Craubner, H., Krahn, E., and Keller, H.U. (1988)

Astron. Astrophys. 191 L1.

Edberg, S.J., Newburn, R.L., and Rahe, J. (1987) in "Exploration of

Halley's Comet" ed. Grewing, M., Praderie, F., and Reinhard, R. pub. by

Springer- Verlag, Berlin.

Hechler, F. and Jappe, S. (1986) European Space Operations Centre/Orbit

and Attitude Division, EXSUP 5 Data Base.

IHW staff (1985) in "Space Missions to Halley's Comet", eds. Reinhard, R.

and Battrick, B. ESA SP-1066 p. 161.

Keller, H.U. and 24 others.  J. Phys. E: Sci. Instrumentation. 20 807.

Keller, H.U., Curdt, W., Kramm, J.-R., and Thomas, N. (1990) in "The

Halley Multicolour Camera Atlas" eds. Longdon, N. and Reinhard, R. ESA

SP-1127.

Schmidt, W.K.H., Keller, H.U., Wilhelm, K., Arpigny, C., Barbieri, C.,

Biermann, L., Bonnet, R.M., Cazes, S., Cosmovici, C.B., Delamere, W.A.,

Huebner, W.F., Hughes, D.W., Jamar, C., Malaise, D., Reitsema, H., Seige, P.,

and Whipple, F.L. (1986) in "The Giotto Mission - Its Scientific

Investigations" ed. Reinhard, R. ESA SP-1077 p. 149.

Thomas, N. and Keller, H.U. (1989) Astron. Astrophys. 213 487.

Thomas, N. and Keller, H.U. (1990) Appl. Optics, 29, No. 10, 1503.