Galileo Magnetometer, Gaspra Browse Data - Additional Documentation

===================================================================

[These data were extracted from the separate catalog files originally

provided with the BROWSE data file.]

Dataset Overview

================

This dataset contains data acquired by the Galileo Magnetometer

during the Gaspra flyby on Oct 29, 1991. The browse dataset has

been created by averaging samples from the  7.68 kB Low Rate

Science (LRS) tape record mode to 20 second sampling and merging

the data with the 8 RIM averages of the optimal averager to

provide a continuous dataset for the encounter day. The

magnetometer high time resolution recorded data include a few

short intervals in the solar wind prior to the encounter, plus

a main ~two hour recording near closest approach.  The optimal

averager section of the instrument (please see the instrument

description) was configured to acquired 8 RIM (~8 minute)

averages to fill in the gaps between the recorded intervals. All

of these data have been fully processed to remove instrument

response function characteristics and interference from magnetic

sources aboard the spacecraft. The magnetometer data are provided

in heliographic (RTN) coordinates. Trajectory data have been

provided as a separate archive product.

Primary Dataset Reference:

Kivelson, M. G., L. F. Bargatze,  K. K. Khurana, D. J. Southwood,

R. J. Walker, P. J. Coleman, 'Magnetic Field Signatures Near

Galileo's Closest Approach to Gaspra', Science, Vol. 261, p331-334,

16 July, 1993.

Primary Instrument Reference:

Kivelson, M. G., K. K. Khurana, J. D. Means, C. T. Russell, and

R. C. Snare, 'The Galileo magnetic field investigation', Space

Science Reviews, 60, 1-4, 357, 1992.

Data Columns:

time    S/C event time (UT) given in PDS time format

YYYY-MM-DDThh:mm:ss.sssZ

Br      Magnetic field radial component in RTN coordinates <nT>

Bt      Magnetic field tangential component in RTN coordinates <nT>

Bn      Magnetic field normal component in RTN coordinates <nT>

Bt      Average magnetic field magnitude <nT>

Missing data value = 99999.999

Fortran Format of the data file: (1X, A24, 4F11.3)

Data Acquisition:

The data for this dataset were acquired by the outboard magnetometer

(flip left position) in the low field (+/- 32 nT) mode (ULLR).

has a digitization step size of 0.0625 nanoTesla. However, these

data are acquired at 30 vectors/second and then recursively filtered

in the instrument. The high rate data that are recorded to tape have

a sample rate of 4.5 vectors/second. If there is sufficient variation

in the 6-7 input samples that make up a single output sample, then

the effective digitization step size becomes much smaller. The long

averages provided in the browse dataset should not show any

quantization effects.

Data Sampling:

The high rate recorded data are not evenly sampled within a minor

frame. However, these data have been averaged using an averaging

routine to produce evenly sampled data. The averaging process takes

all available data during a 20 second period centered on the output

time tag. Averages are not overlapping. The time tag gives the

spacecraft event time (SCET) of the center of the averaging interval

in universal time (UT).

The optimal averager section of the instrument 'averages' data by

applying a recursive filter to onboard processed and despun data

and then decimating the data at the appropriate sample times. This

process is explained in detail in the 'cruise' dataset that has

been included with this archive product. The final processed

product that has been incorporated into this dataset is sampled

about every 8 minutes where the time tag marks the center of the

averaging interval in SCET / UT.

Coordinate System

=================

The data are provided in heliographic  RTN (radial-tangential-normal)

coordinates. The radial direction is taken along the instantaneous

Sun->S/C line, positive away from the sun. The tangential direction

is found by taking the cross product of the sun spin axis with the

radial (T = Omega x R) direction. Finally, the normal direction is

the cross product of R and T (N = R x T).

CONFIDENCE_LEVEL_NOTE

The sources of error which are the most significant are those

associated with magnetic sources aboard the spacecraft, especially

those with temporal or spacecraft orientation variations. The next

greatest contributor of error is the data from the AACS which

affects our knowledge of the spacecraft orientation and hence

rotates the magnetic field vector. Lastly, telemetry or software

errors which produce 'spikes' or bit errors in the data are error

sources.

In this dataset the magnetic sources associated with the spacecraft

were fairly constant and magnetic field values are accurate to at

least 0.01 nT. The spacecraft orientation has been well determined

and is known to better the 0.1 degree.