Galileo Magnetometer, Gaspra High-Resolution Data - Additional Documentation

============================================================================

[These data were extracted from the separate catalog files originally

provided with the HRSC and HRGEOPH data files.]

Dataset Overview

================

This dataset contains data acquired by the Galileo Magnetometer

from the Gaspra encounter. The data are at the full instrument

resolution for the 7.68 kB Low Rate Science (LRS) real time

telemetry mode. These data have been fully processed to remove

instrument response function characteristics and interference

from magnetic sources aboard the spacecraft. The data are provided

in physical units (nanoTesla) in 4 coordinate systems. One of the

data files contains data in Inertial Rotor Coordinates (IRC = despun

spacecraft). This file includes many of the data processing

parameters from the AACS system as well as the sensor zero levels.

The other data file contains magnetic field data in Gaspra-centric

Solar Ecliptic (GaSE), Earth Mean Equatorial equinox 1950 (EME-50),

and finally in Heliographic (RTN) coordinates.

Trajectory data are provided as a separate dataset for each of the

geophysical coordinate systems.

Primary Dataset  Reference:

Kivelson, M. G., L. F. Bargatze,  K. K. Khurana, D. J. Southwood,

R. J. Walker, P. J. Coleman, 'Magnetic Field Signatures Near

Galileo's Closest Approach to Gaspra', Science, Vol. 261, p331-334,

16 July, 1993.

Primary Instrument Reference:

Kivelson, M. G., K. K. Khurana, J. D. Means, C. T. Russell, and

R. C. Snare, 'The Galileo magnetic field investigation', Space Science

Reviews, 60, 1-4, 357, 1992.

Data Acquisition:

The data were acquired by the Galileo Magnetometer in the normal

Low Rate Science (LRS) manner except that the data were recorded

to tape and played back during the Earth 2 encounter. The data

were acquired by the outboard magnetometer in the flip left

position and the low range (high gain) mode.

The Galileo magnetometer has 8 possible LRS acquisition

configurations (modes). There are two sensor triads mounted

7 and 11 meters from the rotor spin axis (inboard and outboard)

along the boom. Each of the sensor triads has two gain states

(high and low).  In addition, the sensor triads can be 'flipped'

to move the spacecraft spin-axis aligned sensor into the spin

plane and visa versa. Please see the instrument description for

full details on the instrument, sensors, and geometries. The

combinations of sensor, gain state, and flip direction form modes.

_____________________________________________________________________

Mode Characteristics

_____________________________________________________________________

Mode Name            Acronym         range      quantization

_____________________________________________________________________

Inboard,  left,  high range*    ILHR      +/- 16384 nT       8.0   nT

Inboard,  right, high range*    IRHR      +/- 16384 nT       8.0   nT

Inboard,  left,  low  range*    ILLR      +/-   512 nT       0.25  nT

Inboard,  right, low  range*    IRLR      +/-   512 nT       0.25  nT

Outboard, left,  high range*    ULHR      +/-   512 nT       0.25  nT

Outboard, right, high range*    URHR      +/-   512 nT       0.25  nT

Outboard, left,  low  range*    ULLR      +/-    32 nT       0.008 nT

Outboard, right, low  range*    URLR      +/-    32 nT       0.008 nT

* range is the opposite of gain

Data Sampling:

The Galileo magnetometer samples the magnetic field 30 times

per second. In order to reduce the data rate to the MAG LRS rate,

the instrument performs an onboard averaging process.  The high

rate samples are recursively filtered

Recursive Filter

B(t) = 1/4 Bs(t) + 3/4 B(t-1)

B  = output field

Bs = input field measured by the sensor

t  = sample time

and then resampled by the instrument at 4.5 vectors per second

using a 7,7,6 decimation pattern. The pattern is generated by

doubling the spacecraft clock modulo 10 counter and then applying

the decimation scheme. This gives 3 vectors every spacecraft minor

frame (about 2/3 second) which are sampled unevenly. The first

vector in a minor frame is sampled approximately 0.200 seconds

after the last vector in the preceding minor frame. The other two

samples are taken approximately 0.233 seconds apart. The time tag

associated with a sample is the decimation time.

Coordinate Systems:

The Galileo magnetometer data are being archived in 4 coordinate

systems. The first coordinate system is referred to as inertial

rotor coordinates (IRC). This coordinate system has the Z axis

along the rotor spin axis, positive away from the antenna, and

the X and Y axes lie in the rotor spin plane. In a crude sense,

when the spacecraft is far from Earth, positive X points south,

normal to the ecliptic plane, positive Y lies in the ecliptic

plane in the sense of Jupiter's orbital motion and positive Z is in

the anti-earth direction. The spacecraft antenna (negative Z

direction) is kept earthward pointing to about +/- 10 deg accuracy.

Gaspra-centric Solar Ecliptic (GaSE) is a Gaspra centered

coordinate system defined by the primary vector along the

instantaneous Gaspra->Sun (GSun) line and the Earth's ecliptic

north pole (ENP) as the secondary vector. In this coordinate system:

X is the GSun unit vector taken to be positive towards the sun.

Y is the vector formed by the unitized cross product ENP x GSun

Z completes the right-handed set (Z = X x Y)

such that the X-Z plane contains the ecliptic north pole.

The Earth Mean Equatorial equinox of 1950 (EME-50) coordinate system

is an inertial reference system. The primary vector in this system

points from the Earth towards Aries at the reference epoch. The

secondary vector is along the Earth's rotational axis positive

towards its north pole. In this coordinate system:

X is a unit vector in the direction of Aries at the equinox of 1950

Y is a unit vector in the direction of Z x X

Z is a unit vector in the direction of the Earth's equatorial north

pole at the equinox of 1950.

The EME-50 coordinate system is directly supported by SPICE as the

'FK4' inertial reference frame.

The heliographic (RTN) coordinate system is centered at the sun. The

primary vector in this system points from the center of the sun to the

spacecraft (R). The secondary vector in this system is the sun's north

rotational axis (Omega).

R is along R, positive away from the sun.

T is along the cross product Omega x R

N completes the right-handed set (R x T)

Data Processing:

These data have been processed from the PDS dataset:

'GO-E/V/A-MAG-2-RDR-RAWDATA-HIRES-V1.0'

The 'raw data' product was created from the EDR dataset by removing

the data processing done by the instrument in space. The raw data

product contains the raw instrument samples which have been recursively

filtered and decimated as described above. The processed data in IRC

coordinates were rotated into geophysical coordinates using the SPICE

libraries and kernels provided by NAIF.  In order to generate the IRC

processed data, the following procedure was followed:

1)  Sensor zero level corrections were subtracted from the raw data.

2)  Data were converted to nanoTesla.

3)  A coupling matrix which orthogonalizes the data and corrects for

gains was applied to the data (calibration applied).

4)  Magnetic sources associated with the spacecraft were subtracted

from the data.

5)  Data were 'despun' into inertial rotor coordinates.

1) Zero level determination: The zero levels of the two spin plane

sensors were determined by taking averages over a large number

(about 50) of integral spin cycles. The zero level of the spin

axis aligned sensor was determined by a variety of means. First,

since the spin axis aligned sensor can be flipped into the spin

plane, the value of the zero level determined in the spin plane

can be used in the other geometry. This works well if there are

no spacecraft fields and the zero level is stable. If there are

spacecraft fields present which remain constant over relatively

long time periods (many hours), then another method of zero level

determination is used. The spacecraft spin axis is along the Z

direction; the data in the X and Y directions have already had

zero level corrections applied.

Bm(z)   = B(z) + O(z)

|Bm|^2  = B(x)^2 + B(y)^2 + Bm(z)^2

= B(x)^2 + B(y)^2 + B(z)^2 + O(z)^2 + 2B(z)O(z)

= |B|^2 + O(z)^2 + 2B(z)O(z)

= |B|^2 + O(z)^2 + 2O(z)[Bm(z)  - O(z)]

= |B|^2 - O(z)^2 + 2O(z)Bm(z)

m = measured value    -    no subscript = true value

Now if |B| remains constant over a short interval and O(z) remains

constant over a much longer interval, we can take averages and

reduce this equation to:

|Bm|^2 - <|Bm|^2> =  2O(z)[Bm(z) - <Bm(z)>]

<> indicates average value

Data can be processed using short averages of |B| until many

points are accumulated and then fit with a line in a least

squares sense. The slope of this line is twice the required

offset. The scatter in the data give an indication of the

error in the assumption that |B| and O(z) have remained constant.

Intervals with large rms errors are not retained. The zero levels

removed from the data are given in the IRC data file.

2) Conversion to nanoTesla simply requires dividing the instrument

data numbers by a constant scale factor. For the inboard high

range (low gain) mode the scale factor is 2. For the inboard low

range and outboard high range, the scale factor is 64. The

outboard low range data has a scale factor of 1024.

3) Calibration matrix applied: The determination of a calibration

matrix is too complex to describe here. The method employed has

been well described in 'A Fourier Transform Based Method for

Intraspacecraft Magnetometer Calibration', K.K. Khurana, E.L.

Kepko, and M.G. Kivelson, (in press).

4) After the data were initially processed (calibrated and despun),

it was clear that there were still coherent noise sources remaining

in the data. Dynamic spectra of the magnetometer data revealed

coherent energy at high order (2nd, 3rd, 4th) harmonics of the spin

period. High order harmonics of the spin period can be generated by

spinning about a fixed dipole source such as a source on the despun

platform. The source of the high order harmonics was modeled using

1-D (spacecraft clock angle) Fourier transforms of high pass

filtered data. This allowed us to resolve the source in terms of

the relative spin phase of the scan platform. Model fields

associated with this source (approximately 0.15 nT at the inboard

sensors in the lowest harmonic) have been subtracted from the data.

5) Despinning: Data are despun and checked in inertial rotor

coordinates before transforming to geophysical coordinates. Any

errors in the processing will be most readily apparent in inertial

rotor coordinates. The  nominal transformation to IRC from SRC is

(Bx)   ( cos(theta)   -sin(theta)  0 ) (Bxs)

(By) = ( sin(theta)    cos(theta)  0 ) (Bys)

(Bz)   (   0               0       1 ) (Bzs)

Where s denotes spinning coordinates and theta is the rotor spin

angle.

Frequency dependent phase delays associated with the analog

anti-aliasing filter and the digital recursive filter have been

removed during the despinning of the data. The dominant frequency

in the spinning data is at the spacecraft spin frequency. The phase

angle delay associated from all known sources is computed at the spin

frequency and removed from the data during despinning.

Analog Filter:              Digital Filter (Nyquist Freq Fn = 15Hz)

1543                         1/3

__________________          _____________________

s^2 + 55.5s + 1543           4/3 - exp(-PI*i*f/Fn)

s = 2*PI*i*f

Imaginary = 55.5s          Imaginary = -sin(PI*f/Fn)

Real      = 1543 + s^2     Real      = 4/3 - cos(PI*f/FREQ_N)

f = frequency

delay = tan^-1(Im/Re)

In addition, there is an electrical delay associated with the A/D

conversion of about .037 milliseconds. This delay is converted to

an angle using the instantaneous spin frequency. These 3 sources

of delay are then summed in to the quantity 'phase' and then the

despinning matrix becomes:

(Bx)   ( cos(theta - phase)   -sin(theta - phase)  0 ) (Bxs)

(By) = ( sin(theta - phase)    cos(theta - phase)  0 ) (Bys)

(Bz)   (      0                          0         1 ) (Bzs)

6) Data was transformed to geophysical coordinates: Data are transformed

from inertial rotor coordinates to the Earth Mean Equatorial (equinox

1950) coordinate system. This system is directly supported by the

SPICE software provided by the Navigation and Ancillary Information

Facility (NAIF) at JPL as inertial coordinate system 'FK4'. The

angles required for this transformation come directly from the

Galileo Attitude and Articulation Control System (AACS) data. The

transformation matrix for this rotation is:

--                                                                   --

| (cosTsinDcosR - sinTsinR)  (-sinDsinTcosR - cosTsinR)      cosDcosR |

|                                                                     |

| (cosTsinDsinR + sinTcosR)  (-sinDsinTsinR + cosTcosR)      cosDsinR |

|                                                                     |

| -cosDcosT                  sinTcosD                        sinD     |

--                                                                   --

where

R                              = Rotor-Right Ascension

D                              = Rotor-Declination

T                          = Rotor-Twist - Rotor-Spin-angle  (despun data)

Once in an inertial coordinate system, SPICE software provides

subroutines which allow a user to construct coordinate system

transformation matrices for any ephemeris time. These matrices

were constructed from the SPICE kernels by directly or indirectly

extracting the primary and secondary vectors defining the

coordinate system and then following the procedure outlined in

the coordinate systems section of this document. The spacecraft/

planet (SPK) , leap second (TS), and planetary constants (PCK)

kernels required for these transformations have been archived in

the PDS by NAIF. These SPICE kernels are available on the CD_ROM

which contains the magnetometer data. The SPICE toolkit (software)

can be obtained from the NAIF node of the PDS for many different

platforms and operating systems. At the time of this archive, the

SPICE toolkit was available via an anonymous-ftp site at

naif.jpl.nasa.gov"

CONFIDENCE_LEVEL_NOTE

Data Confidence Level:

Data quality assessment is a rather vague concept which we

will try to address in a somewhat qualitative manner. Each

aspect of the data processing sequence can be analyzed to

determine its maximum possible error contribution. In theory,

these errors could be summed to provide estimates of the

maximum error for each data point. We have not taken our

error analysis to that level. We believe that our calibrations

(sensor geometry and gains) are good enough that they produce

a negligible source of data error. In addition, we believe that

the coordinate system transformations which are derived from

the SPICE kernels and Toolkit are negligible sources of error

in the magnetic field vectors. The sources of error which we

feel are the most significant are those associated with magnetic

sources aboard the spacecraft, especially those with temporal or

spacecraft orientation variations. The next greatest contributor

of error is the data from the AACS which affects our knowledge

of the spacecraft orientation and hence rotates the magnetic

field vector. Lastly, telemetry or software errors which produce

'spikes' or bit errors in the data are error sources.

In regions where the magnetic sources associated with the

spacecraft are fairly constant, magnetic interference is

probably reduced by data processing to better than 0.01 nT

at the inboard sensors. In these same regions, sensor zero

levels (offsets) are known equally well. The data processing

software does a fairly good job of removing all currently

identified sources of magnetic interference.

Our data processing software creates a data quality flag

(dqf) which is an assessment of AACS and telemetry error

source contamination of a given data point. This number

is a binary integer where each bit indicates the presence

or absence of some error source. The number 0 represents

the absence of all error sources which are tested. The higher

order bit (larger number) error sources are considered to be

more significant error sources. Data are examined for

gradients in the field which might be associated with telemetry

bit errors, for regions of bad AACS angles, and for completely

missing data. If the error is considered completely unrecoverable,

the data values are replaced with a missing data flag. In the

case of a flag in the rotor spin angle, the vector components

may be flagged but the magnitude is still valid. Here is a list

of all of the error checks and the bits they set in the dqf field.

DQF_GOOD_DATA                 0   Good data

DQF_BX_GRAD_WARNING         2^0   Component gradient warning

DQF_BY_GRAD_WARNING         2^1   Component gradient warning

DQF_BZ_GRAD_WARNING         2^2   Component gradient warning

DQF_INTERP_ROTATTR          2^3   Missing rotor RA interpolated

DQF_INTERP_ROTATTD          2^4   Missing rotor DEC interpolated

DQF_INTERP_SPINDELT         2^5   Missing rotor Spin Delta interpolated

DQF_INTERP_SCRELCON         2^6   Missing Relative Cone

angle interpolated

DQF_INTERP_SCRELCLK         2^7   Missing Relative Clock

angle interpolated

DQF_INTERP_ROTATTT          2^8   Missing rotor Twist interpolated

DQF_INTERP_SPINANGL         2^9   Missing rotor Spin interpolated

DQF_ROTATTR_FLAG            2^10  Missing rotor RA flagged

DQF_ROTATTD_FLAG            2^11  Missing rotor DEC flagged

DQF_SPINDELT_FLAG           2^12  Missing rotor Spin Delta flagged

DQF_SCRELCON_FLAG           2^13  Missing Relative Cone angle flagged

DQF_SCRELCLK_FLAG           2^14  Missing Relative Clock angle flagged

DQF_ROTATTT_FLAG            2^15  Missing rotor Twist flagged

DQF_AACS_TELEMETRY_HIT_FLAG 2^16  Telemetry hit in AACS record

DQF_MAG_TELEMETRY_HIT_FLAG  2^17  Telemetry hit in mag record

DQF_SPINANGL_FLAG           2^18  Missing rotor Spin flagged

DQF_BX_GRAD_ERROR           2^25  Component gradient error

DQF_BY_GRAD_ERROR           2^26  Component gradient error

DQF_BZ_GRAD_ERROR           2^27  Component gradient error

DQF_BX_FLAG                 2^28  Component flagged

DQF_BY_FLAG                 2^29  Component flagged

DQF_BZ_FLAG                 2^30  Component flagged

Magnetic field gradient warning or error levels are set during

the data processing according to expected variances depending on

the region of space. In the solar wind, gradient warnings are

usually issued at gradients of 10 nT/sec and errors at 15 nT/sec.

AACS angles are interpolated across gaps during the processing

if the gap length is relatively short (less than 10 minutes

typically). If the gaps in spacecraft attitude are long, the AACS

angles are flagged and not interpolated.

Errors associated with AACS angles have various effects on the

data. The rotor right ascension and declination are crucial to

the understanding of the spacecraft orientation. Fortunately,

these angles are slowly varying and can be interpolated to

better than 1 degree of accuracy for long (many hour) time

periods except near major spacecraft maneuvers. The relative

clock and cone angles are used to remove scan platform

interference. In their absence, no interference is removed

(+/- 0.15 nT error possible in each component). The rotor motion

spin delta is used to determine the instantaneous spin frequency

for the phase delay computation. In its absence, the last known

phase delay is applied to the current data point. The rotor spin

angle and twist angle must be present in order to despin the data.

These angles are generally not interpolated for more than ten

minutes because the rotor spin period drifts over time periods on

this order.