Galileo Magnetometer, Gaspra Trajectory Data - Additional Documentation

=======================================================================

[These data were extracted from the separate catalog files originally

provided with the TRAJECT data file.]

Dataset Overview

================

This dataset contains the Galileo spacecraft trajectory data sampled every

minute for Oct. 29 and 30, 1991. The spacecraft closest approach to the

asteroid was on October 29, 1991 at 22:36:40 at a range of about 1600 km.

The trajectory data

are provided in two coordinate systems, Gaspra-centric Solar Ecliptic (GaSE)

and Heliographic (HG) coordinates. The position of the spacecraft has been

extracted from the SPICE kernels provided by the project. The time tags are

the exact times for which the S/C position was calculated. The time tag is

spacecraft event time (SCET) given in universal time (UT).

Primary Dataset Reference:

Kivelson, M. G., L. F. Bargatze,  K. K. Khurana, D. J. Southwood,

R. J. Walker, P. J. Coleman, 'Magnetic Field Signatures Near

Galileo's Closest Approach to Gaspra', Science, Vol. 261, p331-334,

16 July, 1993.

Data Columns:

time    S/C event time (UT) given in PDS time format YYYY-MM-DDThh:mm:ss.sssZ

X       S/C distance from Gaspra in the X-direction in GaSE coordinates <km>

Y       S/C distance from Gaspra in the Y-direction in GaSE coordinates <km>

Z       S/C distance from Gaspra in the Z-direction in GaSE coordinates <km>

R       S/C radial distance from the Sun <AU>

LAT     S/C solar latitude <degrees>

LON     S/C solar (east) longitude <degrees>

Missing data values = 9999999.9 and 99999.999

Fortran Format of the data file: (1X, A24, 3F11.1,3F11.5)

Coordinate Systems:

The data are archived in two coordinate systems, GaSE and IHG (1950). GaSE

Gaspra-centric Solar Ecliptic (GaSE) is a Gaspra centered coordinate system

defined by the primary vector along the instantaneous Gaspra->Sun (GSun) line

and the Earth's ecliptic north pole (ENP) as the secondary vector. In this

coordinate system:

X is the GSun unit vector taken to be positive towards the Sun.

Y is the vector formed by the unitized cross product ENP x GSun

Z completes the right-handed set (Z = X x Y)

such that the X-Z plane contains the ecliptic north pole.

The data are also provided in inertial heliographic (epoch 1950)

coordinates. The radial direction is taken along the instantaneous Sun->S/C

line, positive away from the Sun. Latitude is the angle the radial vector

makes with the Sun's equatorial plane at the reference epoch, positive above

the equator. Longitude is measured from the longitude of the ascending node.

At the reference epoch, the inclination of the Sun's north pole was

7 deg 15 min and the longitude of the ascending node can be computed using the

formula:

Node =  74 deg 22 min + 0.84 min (year - 1900)

CONFIDENCE_LEVEL_NOTE

These data have been extracted from SPICE kernels that were created using

post encounter ephemeris data. The data should be of the highest possible

quality.