Galileo Magnetometer, Ida High-Resolution Data - Additional Documentation

=========================================================================

[These data were extracted from the separate catalog files originally

provided with the HRSC and HRGEOPH data files.]

Dataset Overview

================

This dataset contains data acquired by the Galileo Magnetometer during the

Ida flyby on Aug 28, 1993. Limited space on the tape recorder forced the

magnetometer team to limit their highest time resolution observations to

a ~30 minute interval beginning after closest approach to Ida.

In order to acquire the high time resolution mag data at Ida necessary

to look for magnetic signatures similar to those observed at Gaspra,

a new method of using the instrument optimal averager section was tested.

This method used the spacecraft Command and Data System (CDS) computer to

sample the MAG data buffer memory to retrieve vectors every 4/3 second.

CDS stored the data in internal data buffers (DBUM1A and DBUM1B). The data

were downlinked back to earth in near real-time via CDS memory readouts

(MROs). The data were highly over-filtered by the instrument but many

of the effects of over-filtering could be removed in ground processing by

knowing the filter response function. Details of this process are described

below in the data processing section.

High resolution magnetometer data are provided in four coordinate systems.

Of the two data files included, one contains data in Inertial Rotor

Coordinates (IRC = despun spacecraft). This file includes the spacecraft

attitude and spin phase angles from the AACS system and the sensor zero

levels. The other data file contains magnetic field data in Ida-centric

Solar Ecliptic (IdaSE), Earth Mean Equatorial equinox 1950 (EME-50), and

Heliographic (RTN) coordinates.

Trajectory data are provided as a separate dataset for each of the geophysical

coordinate systems.

Primary Dataset Reference:

Kivelson, M.G., Z. Wang, S. Joy, K.K. Khurana, C. Polanskey, D.J.

Southwood, and R.J. Walker, 'Solar Wind Interactions with Small Bodies: 2.

What Can Galileo's Detection of Magnetic Rotations Tell Us About GASPRA

and IDA', Advances in Space Research, 1994

Primary Instrument Reference:

Kivelson, M. G., K. K. Khurana, J. D. Means, C. T. Russell, and

R. C. Snare, 'The Galileo magnetic field investigation', Space Science

Reviews, 60, 1-4, 357, 1992.

Data Columns (Spacecraft coordinates IRC):

Fortran format (1X, A24, 11F11.3)

time    S/C event time (UT) given in PDS time format YYYY-MM-DDThh:mm:ss.sssZ

rotattt   Rotor twist angle (EME-50)

rotattd   Rotor attitude declination (EME-50)

rotattr   Rotor attitude right ascension (EME-50)

spinangl  Rotor spin angle - inertial S/C coordinates

spindelt  Rotor spin motion delta <radians/minor frame>

screlcon  Rotor-Platform relative cone angle

screlclk  Rotor-Platform relative clock angle

Bx_sc     Magnetic field X component in S/C (IRC) coordinates <nT>

By_sc     Magnetic field Y component in S/C (IRC) coordinates <nT>

Bz_sc     Magnetic field Z component in S/C (IRC) coordinates <nT>

Bmag      |B| Magnitude of B <nT>

Data Columns (Geophysical Coordinates):

Fortran format (1X, A24, 10F11.3)

time    S/C event time (UT) given in PDS time format YYYY-MM-DDThh:mm:ss.sssZ

Bx_IdaSE  X component of B in IdaSE coordinates (towards Sun) <nT>

By_IdaSE  Y component of B in IdaSE coordinates (towards dusk) <nT>

Bz_IdaSE  Z component of B in IdaSE coordinates (|| to ecliptic normal) <nT>

Bx_eme50  Magnetic field X component in EME-50 coordinates <nT>

By_eme50  Magnetic field Y component in EME-50 coordinates <nT>

Bz_eme50  Magnetic field Z component in EME-50 coordinates <nT>

Br        Magnetic field radial component in RTN coordinates <nT>

Bt        Magnetic field tangential component in RTN coordinates <nT>

Bn        Magnetic field normal component in RTN coordinates <nT>

Bmag      |B| Magnitude of B <nT>

Missing data value = 99999.999

Data Acquisition:

The data for this dataset were acquired by the outboard magnetometer (flip

left position) in the low field (+/- 32 nT) mode (ULLR). This mode

has a digitization step size of 0.0625 nanoTesla. However, these data are

acquired at 30 vectors/second and then recursively filtered in the

instrument. The high rate data that are recorded to tape have a sample rate of

4.5 vectors/second. If there is sufficient variation in the 6-7 input samples

that make up a single output sample, then the effective digitization step size

becomes much smaller. The data are next passed to an onboard processing

algorithm which corrects the data for offsets, gains, and geometry. The

data can now be sent to the tape recorder or passed to the optimal averager

section of the instrument.

Optimal averager data are decimated to 1 vector every minor frame such that

only the first sample of the minor frame is retained. These data are then

despun into Inertial Rotor Coordinates (IRC) and passed to another

recursive filter operation. The filter used in the Ida high resolution (4/3

sec) data was:

Bo(i) = (15/16) * Bo(i-1) + (1/16)*Bi(i)

where Bo(i) is the output of the i-th sample and Bi(i) is the new input vector

at the time of the i-th sample. Data were sampled by the CDS every 4/3 second.

Data are not averaged any further. The time tag associated with each data

record is the sample time (UT) on the spacecraft (SCET).

Coordinate System:

The Galileo magnetometer data are being archived in 4 coordinate systems.

The first coordinate system is referred to as inertial

rotor coordinates (IRC). This coordinate system has the Z axis along the

rotor spin axis, positive away from the antenna, and the X and Y axes in

the rotor spin plane. In a crude sense, when the spacecraft is far from Earth,

positive X points south, normal to the ecliptic plane, positive Y lies in the

ecliptic plane in the sense of Jupiter's orbital motion and positive Z is in

the anti-Earth direction. The spacecraft antenna (negative Z direction) is

kept earthward-pointing to about +/- 10 deg accuracy.

Ida-centric Solar Ecliptic (IdaSE) is an Ida-centered coordinate system

defined by the primary vector along the instantaneous Ida->Sun (GSun) line

with the Earth's ecliptic north pole (ENP) as the secondary vector. In this

coordinate system:

X is the GSun unit vector taken to be positive towards the Sun.

Y is the vector formed by the unitized cross product ENP x GSun

Z completes the right-handed set (Z = X x Y)

such that the X-Z plane contains the ecliptic north pole.

The Earth Mean Equatorial equinox of 1950 (EME-50) coordinate system is

an inertial reference system. The primary vector in this system points

from the Earth towards Aries at the reference epoch. The secondary vector

is along the Earth's rotational axis positive towards its north pole. In this

coordinate system:

X is a unit vector in the direction of Aries at the equinox of 1950

Y is a unit vector in the direction of Z x X

Z is a unit vector in the direction of the Earth's equatorial north pole

at the equinox of 1950.

The EME-50 coordinate system is directly supported by SPICE as the 'FK4'

inertial reference frame.

The heliographic (RTN) coordinate system is centered at the Sun. The primary

vector in this system points from the center of the Sun to the spacecraft (R).

The secondary vector in this system is the Sun's north rotational axis

(Omega).

R is along R, positive away from the Sun.

T is along the cross product Omega x R

N completes the right-handed set (R x T)

Data Processing:

The tape recorded portion of this dataset was processed according to the

standard methods employed for LRS data. These methods have been well described

in the Venus, Earth, and Gaspra documentation. A 1-D spacecraft interference

function was removed from the LRS data during their processing. The 1-D and

2-D methods and coefficients are described in the Earth 1 and 2 data

processing descriptions. Recorded data cover the time interval 16:53:20.026

through 17:21:36.691. Data sampling in this interval is the normal 3 vectors

per minor frame with sample times offset from the start of the minor frame

by (0.0, 0.2333, 0.2333) seconds.

The remainder of the dataset was acquired by using the optimal averager and

CDS in a special procedure used only at Ida. The highest normal data

acquisition rate of the optimal averager section of the instrument

is 1 vector per spacecraft frame (RIM). In order to achieve the higher desired

data rates, the spacecraft CDS reads out the instrument memory where the

recursive filter stores its vectors every other minor frame (4/3 sec). The

data were stored in CDS data buffers DBUM1A and DBUM1B and then returned by

CDS memory readouts (MRO). The readout of the CDS memory did not go smoothly

and had to be done several times due to telemetry errors. In the end, pieces

of memory were reconstructed by hand with various address ranges coming from

various MRO files. Even after hand splicing segments of data together, many

small segments of data contained dropouts and spurious values.

In order to recover from the effects of over-filtering of the data, the data

were deconvolved with the filter response function in the frequency domain.

This process is not tolerant of data spikes or gaps. In regions of lost or

contaminated data, data were interpolated from the nearest good values.

The interpolation scheme used was unable to seamlessly replace missing data.

The effect of this procedure is to introduce ringing in these areas during

the deconvolution.

Filter ringing also occurs at the ends of the interval in the FFT when the

data are transformed back to the time domain. The crossover from CDS DBUM1A

to DBUM1B occurred at closest approach. At the crossover, a minor frame timing

shift occurred. In other words, even though all of the data in DBUM1A and

DBUM1B are sampled at 4/3 second, the separation between the two buffers is

2 seconds. The FFT method does not tolerate uneven sampling. In order to get

around this problem, and not have FFT edge effects at closest approach, the

entire dataset was resampled to 2/3 second by linear interpolation for the

filter recovery and the data from both buffers were deconvolved together.

After the proper signal amplitude and phase were recovered, the offsets were

removed. The instrument removes the best guess of the sensor zero levels

prior to despinning the data and passing it to the optimal averager. Any

residual DC offset is then spun up to the spin frequency by the despinning

process. In order to remove the offsets from the despun data, the spin plane

components were notch filtered in the frequency domain. The data were

Fourier transformed and the background spectra was determined. In the

frequency band affected by spin tone, the real and imaginary components of

the transform were separately set to match background levels. The data were

then transformed back into the time domain. The spin aligned sensor offset

was determined and removed using the values determined from the cruise data.

After the data were transformed back into the time domain, the data that were

added during the interpolation to even 2/3 second sampling were removed. Only

values at times when data were actually take remain in the dataset. Data at

the ends of the dataset where strong FFT ringing occurs have been excluded.

Attitude control data necessary to transform the magnetometer data into

geophysical coordinates were acquired only for the few seconds during the

taking of each of the Ida images. These very sparse AACS data were linearly

interpolated between values. Since the data were already despun, spin phase

information was not required.

CONFIDENCE_LEVEL_NOTE

These data have been highly processed to obtain the final dataset. Both the

magnetometer data and the AACS data required to transform the mag data into

geophysical coordinates are of lower than average quality. The dataset should

not be used in wave studies. Much of the processing has been done in the

frequency domain and the spectra clearly show the effects. Overall, the field

values are probably correct to 0.01 nT and the AACS angles to better than

a degree. The segment of data that were recorded to tape (16:53 ->17:22) do

not carry as strong a warning but users should be wary of power at the spin

period or its harmonics. The recorded data interval provided a mechanism for

verifying the filter recovered data properly reflect the source signal at the

sampling resolution. Both types of data were simultaneously acquired for about

30 minutes. When superimposed on each other, the filter recovered data

accurately trace the high resolution data but they appear to have a high

frequency cutoff that is less than would be expected from 4/3 second samples.