Requirement Specifications For Level-2 Parameters Required Stored or Derived by

The Near Earth Asteroid Rendezvous (NEAR) Gamma-Ray Spectrometer  (GRS) Ground System

Irina Mikheeva (UA) and Tim McClanahan (GSFC)

Revision 1.5:  May 25, 2001

Table of Contents

- Introduction and Purpose..................................................................................................................…..1

- The UA-LPL Ground Processing Environment....................................................................................2

- Description of NEAR GRS Data File Standards...............................................................................….3

- Fractional Footprint Accounting............................................................................................................4

- Listings of Parameters for GRS Level-2 Integral Records...............................................................…5

E.1-E.7.  NEAR GRS Spectra...................................................................................................................5

E.8.        NEAR GRS Science Housekeeping (SCI HK) Parameters........................................................5

E.9.        NEAR GRS Engineering Housekeeping (ENG HK) Parameters...............................................6

E.10.      NEAR GRS Derived Engineering (DER ENG) Parameters.....................................................10

E.11.      NEAR GRS Spatial (SPATIAL) Parameters............................................................................10

- Listings of Parameters for GRS Level-2 Summary Record............................................................…11

F.1-F.7.  NEAR GRS Summarized Spectra............................................................................................11

F.8.        NEAR GRS Summarized Engineering (ENG) Parameters.......................................................11

F.9.        NEAR GRS Summarized Spatial (SPATIAL) Parameters.......................................................12

- Listings of Parameters for GRS Fractional Footprint Accounting System.................................…12

- Numerical Specifications for GRS Level-2 Integral Records............................................................12

H.1-H.7. NEAR GRS Spectra.................................................................................................................12

H.8.        NEAR GRS Science Housekeeping (SCI HK) Parameters......................................................13

H.9.        NEAR GRS Engineering Housekeeping (ENG HK) Parameters.............................................13

H.10.      NEAR GRS Derived Engineering (DER ENG) Parameters.....................................................13

H.11.      NEAR GRS Spatial (SPATIAL) Parameters............................................................................14

- Numerical Specifications for GRS Level-2 Summary Record...........................................................22

I.1-I.7.    NEAR GRS Summarized Spectra......................................................................................…...22

I.8.         NEAR GRS Summarized Engineering (ENG) Parameters.......................................................23

I.9.         NEAR GRS Summarized Spatial (SPATIAL) Parameters.......................................................24

- Numerical Specifications for GRS Fractional Footprint Accounting System.................................27

APPENDIX 1.    Definitions for:    Instr_Angular_Response_Fct_YYMeV

(where YY can be 0.3, 0.6, 1.0, 3.0 or 6.0);  NC_Angular_Fct;

NC_Geom_Weight_YYMeV;   NC_Weight_YYMeV;

NR_Geom_Weight_YYMeV;   NR_Weight_YYMeV.....................................................30

APPENDIX 2.    Approximations for:   Instr_Angular_Response_Fct_YYMeV,

(where YY can be 0.3, 0.6, 1.0, 3.0 or 6.0);   NC_Angular_Fct...................................…32

APPENDIX 3    Location of GRS Level-2 Parameters in the U of A Database....................................36

NEAR GRS Requirements Specs Revision Notes................................................................................…47

NEAR GRS Glossary and Acronym List..................................................................................................49

# Introduction and Purpose

The following document will act as one part of a two part requirement specification for all Level-2 parameters required to be stored or derived by the Near Earth Asteroid Rendezvous (NEAR) X-ray and Gamma-Ray spectrometer ground system (XGRS).  It will relate specifically to the Gamma-Ray spectrometer (GRS) requirements. The document will act as a primary common reference for both NEAR X-ray and Gamma-ray spectrometer (XGRS) science team members, systems development staff and future users of these data.  The goal is to assist in the development and understanding of parameters defined as key to the NEAR XGRS Science objectives.

A primary data server residing at the University of Arizona, Lunar and Planetary Laboratory (UA-LPL) is designated with the function of deriving and serving mission critical parameters during cruise and encounter phases of the NEAR mission scheduled from February 1996 through February 2001.   The intent of the first level product is to include all NEAR XGRS mission critical information needed for asteroid mapping, health of instrument, control, and science analysis systems.

Level -1 products will include integral spectral data in either raw or instrument calibrated format.  Bundled with the spectral data products will be scientific housekeeping and engineering housekeeping data sampled by the XGRS instrumentation at the same time as the integral spectra onboard the spacecraft.  This information is then telemetered through NASA’s Deep Network (DSN) managed by the Jet Propulsion Laboratory in Pasadena, California, and NEAR related telemetry is forwarded to the Applied Physics Laboratory (APL).  The Level-0 XGRS raw spectral and engineering data is then broken out of the NEAR mission specific data stream and stored online at the APL Science Data Center (SDC).  These data are stored in the Hierarchical Data Format (HDF) file standard.  These files are then picked up by the automated (UA-LPL) data processing ingest system.

Figure 1.0   NEAR XGRS OPERATIONAL FLOW OF INFORMATION

NEAR

SPACECRAFT

MAX PLANCK

INSTITUTE  FOR CHEMISTRY

DEEP SPACE NETWORK (DSN)

JET PROPULSION LABORATORY

NEAR Telemetry

Flow   											           MAX PLANCK

APL		    Meteorite Libraries

APPLIED  PHYSICS LABORATORY

Serve HDF         Science Analysis

Serve SPICE

Validate SC MACROS

UA-LPL

NEAR XGRS Data Server

NASA GODDARD SPACE  FLIGHT CENTER

UNIVERSITY OF ARIZONA LUNAR AND PLANETARY

LABORATORY

Level 1  Processing

Science Analysis

Raw Spectral Data Products

Calibration and Quicklook Info

GSFC

Instrument MACROS

CORNELL UNIVERSITY						                     XGRS health of mission

CORNELL

UNIVERSITY

Coverage Mapping						         Spacecraft Pointing

Instrument Pointing Plan					         Science Analysis

- The UA-LPL Ground Processing Environment

The core of the UA-LPL data processing system (Figure 2.0) consists of a Sybase Relational Database (UA-RDBMS) that stores tables of NEAR XGRS raw spectral, science housekeeping, engineering housekeeping, derived engineering and spatial data products.   XRS and GRS data processing systems are logically partitioned into separate processing environments on the UA-LPL server.    A scientific data processing system has been developed around the UA-RDBMS to provide automated data ingest and science team interaction. Critical spatial and derived engineering data products are generated either at ingest time or query time.  Core query time processing options include calibration and summation processing, compression and bundling of query results for return to the scientific user community over the Internet file transfer protocol (FTP).

Figure 2.0   NEAR XGRS Ground Processing System at the UA-LPL

Ingest  Proc

Spatial Process

Calibration

User Comm

Bundling

Summation

WWW Interface

UA-LPL

RDBMS

NASA Goddard

Calibrate

Quicklook

Inst. Health

UA-LPL  database ingest systems automatically monitor the APL SDC for new and updated postings of XGRS data sets.  These file sets are recovered via the file transfer protocol (FTP) to the UA-LPL data system.  Raw spectral and engineering data products are then loaded into XGRS RDBMS tables with Mission Elapsed Time (MET) acting as the primary key for database access.  Mission Elapsed Time is used with spacecraft ephemeris data SPICE system,  APL Science Data Center and JPL/NASA Ancillary Information Facility) to derive spatial data products.  Spacecraft navigation and pointing information is converted to SPICE data for the NEAR XGRS ground system and will be generated by participating scientists at APL/SDC. These products will then be maintained within the UA-RDBMS database system.

At Goddard Space Flight Center participating scientists will be charged with monitoring instrument health, calibration, and bad data flagging systems.  These data are then forwarded to on-line ingest processes at the UA-RDBMS.  The calibration system will be used to convert spectral data products to a common energy standard specified at query time.   Calibration processing is optionally selected at query time.  Bad flagging information is generated during instrument health checks as a function of identification of problem (MET) record sets.  These (MET)’s are forwarded to the UA-LPL RDBMS for tagging.

Query time summation processes will be used to sum multiple integral record sets into a single output record. This record will include one spectrum per spectral classification and selected engineering products.  Summed output will generally be specific to regional coverage and science analysis.  The primary motivation for summation processing is to limit large volume network transfers incurred in this distributed processing environment.  Results are then accumulated in the Xternal Data Representation (XDR) binary file standard (Sun Microsystems) then compressed, bundled and forwarded to the query initiator.

C. Description of NEAR GRS Data File Standards

In the NEAR GRS data processing system there are 3 data file standards possible for output.   These different file standards reflect the different UA-LPL ground processes selectable at query time.   Output format type is selected at query time through the UA-LPL WWW query interface.    Integral raw GRS products will be used primarily in the calibration, instrument health and mission planning activities.  Calibrated data will be used primarily in the science analysis activity.   Formats for the three record standards are as follows.

Figure 3.0

FORMAT 1:     GRS Integral Raw:

NAI    BGO   ANTICO   NAI1ESC    BGO1ESC   NAI2ESC   BGO2ESC     SCI HK    ENG HK     DER ENG   SPATIAL   SPARE

Description:

- NAI 		1024 R*4	Sodium Iodide  (NaI) raw spectrum				E.1., H.1.

- BGO		1024 R*4	Bismuth Germanate (BGO) raw spectrum 			E.2., H.2.

- ANTICO   		1024 R*4	AntiCoincidence raw spectrum				E.3., H.3.

- NAI1ESC		1024 R*4	NaI single escape raw spectrum				E.4., H.4.

- BGO1ESC	    	   21  R*4	BGO single escape raw spectrum				E.5., H.5.

- NAI2ESC		1024 R*4	NaI double escape raw spectrum				E.6., H.6.

- BGO2ESC	    	    21 R*4	BGO double escape raw spectrum				E.7., H.7

- SCI HK		    31 I*4		Science Housekeeping sampled at integration time		E.8., H.8.

- ENG HK		  152 R*4	Engineering Housekeeping sampled at integration time	E.9., H.9.

- DER ENG	    	    17 R*4	Derived Engineering: only live times, galactic cosmic 	E.10., H.10.

ray (GCR) flux, real gain zero** estimates and position

of escape windows are valid ELSE Default value

5  I*4	 	1 longword for BadFlag and 4 longwords for QueryID

- SPATIAL	     	     1  I*4		Mission Elapsed Time (MET)				E.11., H.11.

32  R*4	Spatial derivations, positional and pointing information

- SPARE		  25  R*4	To Be Decided (TBD)

FORMAT 2:     GRS Integral Calibrated:

NAI     BGO   ANTICO    NAI1ESC   BGO1ESC   NAI2ESC  BGO2ESC    SCI HK   ENG HK    DER ENG    SPATIAL   SPARE

Description:

- NAI 		1024 R*4	Sodium Iodide  (NaI)  calibrated spectrum			E.1., H.1.

- BGO		1024 R*4	Bismuth Germanate (BGO) calibrated spectrum 		E.2., H.2.

- ANTICO   		1024 R*4	AntiCoincidence calibrated  spectrum			E.3., H.3.

- NAI1ESC		1024 R*4	NaI single escape calibrated  spectrum			E.4., H.4.

- BGO1ESC	    	    21 R*4	BGO single escape raw spectrum				E.5., H.5.

- NAI2ESC		1024 R*4	NaI double escape calibrated spectrum			E.6., H.6.

- BGO2ESC	    	    21 R*4	BGO double escape raw spectrum				E.7., H.7.

- SCI HK		    31  I*4	Science Housekeeping sampled at integration time 		E.8., H.8.

- ENG HK		  152 R*4	Engineering Housekeeping sampled at integration time 	E.9., H.9.

- DER ENG	    	    17 R*4	Derived Engineering: All information is valid 		E.10., H.10.

5 I*4	1 longword for BadFlag and 4 longwords for QueryID

- SPATIAL	      	      1  I*4	Mission Elapsed Time (MET)				E.11., H.11.

32  R*4	Spatial derivations, positional and pointing information

- SPARE		   25  R*4 	To Be Decided  (TBD)

Nomenclature for GRS integral data products:  FORMATS 1 and 2  (Calibrated and Raw)

g(yyyy)(ddd)(hh)(mm)(ss).xdr                          Example:  g1998234010159.xdr

NEAR GRS Summation Processing:  (Format 3)

The NEAR XGRS data processing ground system is a distributed system that currently spans institutions in the United States and Europe.  The summation processing system has been specified to render the possibly very large volumes of data (> Gbytes) returned from a single query down to a single summary record.  The intention of summation processing will be to minimize the per query network data transfer size (bytes), by rendering a single summed spectrum from a relationally defined set.  Correlated parameters will be bundled with this summed set.   The summation ground processing step will be supported only on calibrated spectral data products.

FORMAT 3:      GRS Summed:

NAI         BGO        ANTICO        NAI1ESC         BGO1ESC         NAI2ESC         BGO2ESC             ENG          SPATIAL

Description:

- NAI 		1024 R*4	Sodium Iodide  (NaI)  calibrated summed spectrum		F.1., I.1.

- BGO		1024 R*4	Bismuth Germanate (BGO) calibrated summed spectrum	F.2., I.2.

- ANTICO   	1024 R*4	AntiCoincidence calibrated  summed spectrum		F.3., I.3.

- NAI1ESC	1024 R*4	NaI single escape calibrated  summed spectrum		F.4., I.4.

- BGO1ESC	    21 R*4	BGO single escape raw summed spectrum			F.5., I.5.

- NAI2ESC	1024 R*4	NaI double escape calibrated summed spectrum		F.6., I.6.

- BGO2ESC	    21 R*4	BGO double escape raw summed spectrum			F.7., I.7

- ENG 		    13 R*4	Engineering  and Derived Eng				F.8., I.8.

- SPATIAL	    13 R*4	Averaged Spatial parameters 				F.9., I.9.

Nomenclature for GRS integral data products:  FORMATS 3  (Summed)

h(yyyy)(ddd)(hh)(mm)(ss).xdr                          Example:  h1998234010159.xdr

Summation processing will be query specific and all derived engineering tracking is done internal to each query process.  All (XDR) file sets returned from the query will be run through the summation processing system. Integral calibrated spectra will be summed into single GRS specific output sets.  Selected engineering and spatial parameters are also averaged into values representative of the parameter over the range of output (MET) records and specific to each query.  Parameter rendering algorithms are listed in the numerical specifications (Section I).

# Fractional Footprint Accounting

The 433Eros asteroid is highly irregular in shape and deviates significantly from a spherical geometry.  A system was developed by NEAR XGRS staff to partition a virtual representation of the 433Eros asteroid into surface regions (bins) of roughly equal surface area, not based on spherical geometry.  The bin system is internal to the UA-RDBMS ground processing system.

Individual spectral accumulations will most likely span multiple asteroid surface bins.   This system will be used to discriminate and identify the applicability of integral footprints, based on their fractional coverage of the asteroid surface bin being queried. During UA-RDBMS ingest processing each integral footprint will be broken down into a set of records, one bin record for each bin subtended by the footprint.  The current plan does not include the possibility of using the fractional footprint as a weighting function when automatically summing spectral data at query time.   The fractional footprint accounting will be used as a query time threshold criteria for inclusion of the integral MET to summation processing.

##### Figure 3.0

FOOTPRINT 	      BIN       MET  = Single Integration Mission Elapsed Time (MET)

Fractional Footprint Accounting (MET in Spatial Bins)

A                 B

C                 D

BIN  A		BIN  B 	BIN  C		BIN  D

#### MET  0.2	MET  0.1	MET  0.4	MET 0.3

1 Integral Footprint  at MET

Fraction totals = 1.0

4 total records generated Asteroid Spatial Bins A,B,C,D

# A query time specification requesting all integrations (MET records) from Bin C with a fractional footprint area

>= 0.4  would return the example integration and others whose fractional coverage are at least 0.4.

Information related to the bin processing is included in sections G and J.

#### Listings of Parameters for GRS level–2 Integral Records

FORMATS  1,2 - List of RDBMS parameters for Single Raw and Calibrated GRS Level-2 record (output)

###### E.1. – E.7.   NEAR GRS Spectra:  4 spectra*1024 channels*(Real*4), 1 spectrum*21 channels(Real*4), 1 spectrum*1024 channels*(Real*4), 1 spectrum*21 channels(Real*4)

0 – 1023  Sodium Iodide (NAI) spectrum

0 – 1023  Bismuth Germanate (BGO) spectrum

0 – 1023  Anti-Coincidence (ANTICO) spectrum

0 – 1023  Sodium Iodide (NAI1ESC)  single escape spectrum

0 – 20  Bismuth Germanate (BGO1ESC) single escape spectrum

0 – 1023  Sodium Iodide (NAI2ESC) double escape spectrum

0 – 20  Bismuth Germanate (BGO2ESC) double escape spectrum

## E.8.   NEAR GRS Science Housekeeping (SCI HK) Parameters:  31*(Integer*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.

- MET

- UTC

- DQI  			*  Data Quality Index

- TIME_TAG

- INTEGRATION_TIME

- BGO_PROTON_FLARE_FLG

- BGO_LEVEL_1_SAFE_FLG 							Version 7

- BGO_LEVEL_2_SAFE_FLG

- NAI_LEVEL_1_SAFE_FLG

- NAI_LEVEL_2_SAFE_FLG

- NAI_INT_OVERFLOW_FLG

- BGO_INT_OVERFLOW_FLG

- NAI_ANTICOINCID_OVFL_FLG

- NAI_SINGLE_ESCP_OVFL_FLG

- NAI_DOUBLE_ESCP_OVFL_FLG

- BGO_SINGLE_ESCP_OVFL_FLG

- BGO_DOUBLE_ESCP_OVFL_FLG

- NAI_RAW_EVENT_RATE

- BGO_RAW_EVENT_RATE

- COINCIDENCE_COUNTER 							Version 7

- COMMAND_COUNTER 							Version 7

- PROBLEM_COUNTER 								Version 7

- PROBLEM_CODE 								Version 7

- NUM_NAI_ANTICOINCID_EVENTS

- NUM_NAI_VALID_EVENTS

- NUM_BGO_VALID_EVENTS

- NAI_DC_OFFSET 						Downlinked value.  No conversion to energy

bins is performed. ( no devision by 2 as required by NEAR XFRS DPU Software Requirements Specifications JHU/APL 7358-9002 Version 7)

- BGO_DC_OFFSET 								- “ -

- NAI_STAND_DEV								- “ -

- BGO_STAND_DEV								- “ -

# 30  GRAY_DATA_QUALITY_BIT 							Version 7

## E.9. NEAR GRS Engineering Housekeeping (ENG HK) Parameters: 152*(REAL*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.

# ENG_HK Parameters 0:137 are common to XRS and GRS.  Parameters are sampled at XRS and GRS

# Resolutions.

- DETECTOR_ELECTR_CURR

- XRAY_PIN_DIODE_FET_BIAS

- HVCE_TEMPERATURE

- UNF_XRAY_HVPS_VOLT

- MG_XRAY_HVPS_VOLT

- AL_XRAY_HVPS_VOLT

- XRAY_GAS_HVPS_VOLT

- XRAY_PIN_HVPS_VOLT

- NAI_HVPS_VOLT

- BGO_HVPS_VOLT

- DPU_DC_DC_TEMP

- XGRS_DC_DC_TEMP

- UNF_XRAY_HVPS_TEMP

- MG_XRAY_HVPS_TEMP

- AL_XRAY_HVPS_TEMP

- XRAY_GAS_HVPS_TEMP

- XRAY_PIN_HVPS_TEMP

- NAI_HVPS_TEMP

- BGO_HVPS_TEMP

- TEC_CURRENT

- NAI_HVPS_VOLT_CMD

- BGO_HVPS_VOLT_CMD

- UNF_HVPS_VOLT_CMD

- MG_HVPS_VOLT_CMD

- AL_HVPS_VOLT_CMD

- HVPS_POWER_ON_OFF

- XRAY_CALIB_HOME_FLG

- XRAY_CALIB_MOTOR_ON_OFF

- XRAY_CALIB_MOTOR_ENABLE

- GAS_HVPS_VOLT_CMD

- PIN_HVPS_VOLT_CMD

- GAMMA_RAY_HEATER_CURR

- spare0

- XRAY_CALIB_MOTOR_DIR

- XRAY_CALIB_MOTOR_CURR

- XRAY_CALIB_MOTOR_GOAL

- XRAY_CALIB_MOTOR_POS

- XRAY_CALIB_MOTOR_FID_SENS

- XRAY_CALIB_MOTOR_FID_BRIT

- XRAY_MOTOR_IN_CALIB_POSITION

- XRAY_MOTOR_IN_NORM_POSITION

- ACTIVE_SOLAR_TOGGLE_MODE

- XRAY_CALIB_MAX_STEPS

- Spare

- TEC_ENABLE

- XRAY_CALIB_CUM_MOTOR_STPS

- GRAY_BURST_THRESH_VAL

- XRAY_SAFING_THRESH

- XRAY_CALIB_MOTOR_DIAG

- TEC_POWER_ON_OFF

- TEC_MODE

- TEC_COOL_HEAT_FLG

- TEC_TEMPERATURE

- TEC_COOL_MODE_TEMP_UP_LMT

- TEC_COOL_MODE_TEMP_LOW_LMT

- TEC_HEAT_MODE_TEMP_UP_LMT

- TEC_HEAT_MODE_TEMP_LOW_LMT

- GAMMA_RAY_TEMP_HYSTERESIS

- GAMMA_RAY_HEAT_TEMP_LOW_LMT

- FULL_GAMMA_RAY_SCIENCE_MODE

- FULL_XRAY_SCIENCE_MODE

- SUMM_GRAY_XRAY_SCIENCE_MODE

- GRAY_BURST_SCI_REC_MODE

- HVCE_TEMP_SET_POINT

- CMDED_END_XRAY_INTEG_PER

- CMDED_GAMMA_RAY_INTEG_PER

- BGO_PROTON_FLARE_FLG

- GAMMA_RAY_BIN_OVER_FLG_CMD

- UNF_XRAY_BIN_OVER_FLG_CMD

- MG_BIN_OVER_FLG_CMD

- AL_BIN_OVER_FLG_CMD

- ACT_BIN_OVER_FLG_CMD

- NAI_BIN_OVERFLOW_FLG

- BGO_BIN_OVERFLOW_FLG

- NAI_ANTICOIN_OVERFLOW_FLG

- NAI_SING_ESC_OVERFLOW_FLG

- NAI_DOUB_ESC_OVERFLOW_FLG

- BGO_SING_ESC_OVERFLOW_FLG

- BGO_DOUB_ESC_OVERFLOW_FLG

- UNF_XRAY_BIN_OVERFLOW_FLG

- MG_BIN_OVERFLOW_FLG

- AL_BIN_OVERFLOW_FLG

- ACT_BIN_OVERFLOW_FLG

- GAMMA_RAY_HEAT_ON_OFF

- GAMMA_RAY_HEAT_MODE

- BGO_LEVEL_1_SAFING_FLG

- BGO_LEVEL_2_SAFING_FLG

- Spare

- XRAY_INTEGR_TIME

- HVPS_ON_OFF

- PIN_SENSOR_MASK_FLG

- GAS_SENSOR_MASK_FLG

- AL_SENSOR_MASK_FLG

- MG_SENSOR_MASK_FLG

- UNF_SENSOR_MASK_FLG

- BGO_SENSOR_MASK_FLG

- NAI_SENSOR_MASK_FLG

- UNF_XRAY_RISE_TIME_THRESH

- MG_RISE_TIME_THRESH

- AL_RISE_TIME_THRESH

- GAS_RISE_TIME_THRESH

- UNF_RISE_TIME_VALID_THRESH 			  Downlinked value. No ground conversion

specified in NEAR XFRS DPU Software Requirements Specifications JHU/APL 7358-9002 Version 7 is performed.

- MG_RISE_TIME_VALID_THRESH  				- “ -

- AL_RISE_TIME_VALID_THRESH  				- “ -

- GAS_RISE_TIME_VALID_THRESH  			- “ -

- GAMMA_RAY_INTEGR_TIME

- GAMMA_SING_ESC_WIND_CENTER			- “ -

- GAMMA_SING_ESC_WIND_WIDTH

- GAMMA_DOUB_ESC_WIND_CENTER			- “ -

- GAMMA_DOUB_ESC_WIND_WIDTH

- NAI_LOW_LEVEL_AMPL_THRESH 	  	Ground conversion is: (downlinked value * 3.413 +

3.0), which is different from specified in NEAR XFRS DPU Software Requirements Specifications JHU/APL 7358-9002 Version 7

- BGO_LOW_LEVEL_AMPL_THRESH  			- “ -

- UNF_LOW_LEVEL_AMPL_THRESH

- MG_LOW_LEVEL_AMPL_THRESH

- AL_LOW_LEVEL_AMPL_THRESH

- GAS_LOW_LEVEL_AMPL_THRESH

- PIN_LOW_LEVEL_AMPL_THRESH

- GAMMA_RAY_SENSOR_TEMP

- NAI_LEVEL_1_SAFING_FLG

- UNF_LEVEL_1_SAFING_FLG

- MG_LEVEL_1_SAFING_FLG

- AL_LEVEL_1_SAFING_FLG

- GAS_LEVEL_1_SAFING_FLG

- PIN_LEVEL_1_SAFING_FLG

- NAI_LEVEL_2_SAFING_FLG

- UNF_LEVEL_2_SAFING_FLG

- MG_LEVEL_2_SAFING_FLG

- AL_LEVEL_2_SAFING_FLG

- GAS_LEVEL_2_SAFING_FLG

- PIN_LEVEL_2_SAFING_FLG

- XRAY_SENS_SAFING_REST_LEVEL

- GAMMA_SENS_SAFING_REST_LEVEL

- XRAY_SAFING_REST_MAX_RETRY

- GAMMA_SAFING_REST_MAX_RETRY

- GAMMA_HEATER_DUTY_CYCLE

- GAMMA_SENS_SAFING_THRESH

- HVPS_CURRENT

- BGO_PROTON_FLARE_THRESH

- VERSION_NUMBER								Version 7

- NAI_SPECTRUM_0 								Version 7

- HW_NAI_RAW 								Version 7

- NAI_HIGH_ENERGY 								Version 7

- BGO_SPECTRUM_0 								Version 7

- HW_BG 										Version 7

- BGO_HIGH_ENERGY 								Version 7

- NAI_ANTICOIN_SPECTRUM_0 						Version 7

- HW_NAI_ANTICOIN 								Version 7

- NAI_ANTICOIN_HIGH_ENERGY 						Version 7

- SINGLE_ESC_SPECTRUM_0 							Version 7

- SINGLE_ESC_HIGH_ENERGY 						Version 7

- DOUBLE_ESC_SPECTRUM_0 						Version 7

- DOUBLE_ESC_HIGH_ENERGY 						Version 7

### E.10. NEAR GRS Derived Engineering (DER ENG) Parameters:  17*(Real*4), 1*(Integer*4), 1*(4*Integer*4)

These parameters are not stored in the University of Arizona  RDBMS but derived at a query time.

# “Not valid” – is designated as = -1

0     GAIN_STANDARD_BGO

1     GAIN_STANDARD_NAI

2     ZERO_STANDARD_BGO

3     ZERO_STANDARD_NAI

4     REAL_GAIN_BGO

5     REAL_GAIN_NAI

6     REAL_ZERO_BGO

7     REAL_ZERO_NAI

8     LIVE_TIME_BGO

9     LIVE_TIME_NAI

10   BGO_VALID_CHANNEL_HI

11   NAI_VALID_CHANNEL_HI

12   BGO_VALID_CHANNEL_LOW

13   NAI_VALID_CHANNEL_LOW

14   Pos_ESC_1

15   Pos_ESC_2

16   GCR_FLUX

17   Bad_Flag

18   Query_ID

E.11.  NEAR GRS Spatial (SPATIAL) Parameters:  1*(Integer*4), 32*(Real*4)

These parameters are derived in the University of Arizona at the ingest phase.

- “Not valid” – is designated as = -1

- MET

- PLATEID_BORESITE_INTERSECT				NOT VALID IF FOV_STATUS=0

- BINID_BORESIGHT_INTERSECT				NOT VALID IF FOV_STATUS=0

- SC_POSITION, X

- SC_POSITION, Y

- SC_POSITION, Z

- BS_VECTOR, X

- BS_VECTOR, Y

- BS_VECTOR, Z

- FOV_STATUS

- DOWNLINK_STATUS

- AVERAGE_SC_DISTANCE					NOT VALID IF FOV_STATUS=0

- AVERAGE_EMISSION_ANGLE				NOT VALID IF FOV_STATUS=0

- TOTAL_EFF_SOLID_ANGLE					=0  IF FOV_STATUS=0

- TOTAL_AREA_FOOTPRINT					NOT VALID

- TOTAL_VISIBLE_AREA					=0  IF FOV_STATUS=0

- NC_WEIGHT_0.3MEV						=0  IF FOV_STATUS=0

- NR_WEIGHT_0.6MEV						=0  IF FOV_STATUS=0

- NC_WEIGHT_0.6MEV						=0  IF FOV_STATUS=0

- NR_WEIGHT_1.0MEV						=0  IF FOV_STATUS=0

- NC_WEIGHT_1.0MEV						=0  IF FOV_STATUS=0

- NR_WEIGHT_3.0MEV						=0  IF FOV_STATUS=0

- NC_WEIGHT_3.0MEV						=0  IF FOV_STATUS=0

- NC_WEIGHT_6.0MEV						=0  IF FOV_STATUS=0

- BORESIGHT_INTERSECT_LAT                                              NOT VALID IF FOV STATUS = 0

- BORESIGHT_INTERSECT_LON                                             NOT VALID IF FOV STATUS = 0

- BORESIGHT_INTERSECT_R                                                   NOT VALID IF FOV STATUS = 0

- SPICE_KERNEL_ID

- PLATE_MODEL_ID

- PROCESSING_SOFTWARE_ID

- DATABASE_VERSION_ID

#### F. Listings of Parameters for GRS Level–2 Summary Record

FORMAT  3 - List of RDBMS parameters for Summarized GRS Level-2 record (output)

###### F.1. – F.7.   NEAR GRS Summarized Spectra: 4 spectra*1024 channels*(Real*4), 1 spectrum*21 channels(Real*4), 1 spectrum*1024 channels*(Real*4), 1 spectrum*21 channels(Real*4)

0 – 1023  Sodium Iodide (NAI) spectrum

0 – 1023  Bismuth Germanate (BGO) spectrum

0 – 1023  Anti-Coincidence (ANTICO) spectrum

0 – 1023  Sodium Iodide (NAI1ESC)  single escape spectrum

0 – 20  Bismuth Germanate (BGO1ESC) single escape spectrum

0 – 1023  Sodium Iodide (NAI2ESC) double escape spectrum

0 – 20  Bismuth Germanate (BGO2ESC) double escape spectrum

F.8.   NEAR GRS Summarized Engineering (ENG) Parameters:  13*(Real*4)

- GAIN_STANDARD_BGO

- GAIN_STANDARD_NAI

- ZERO_STANDARD_BGO

- ZERO_STANDARD_NAI

- TOTAL_INTEG_TIME

- TOTAL_LIVE_TIME_BGO

- TOTAL_LIVE_TIME_NAI

- BGO_VALID_CHANNEL_HI

- NAI_VALID_CHANNEL_HI

- BGO_VALID_CHANNEL_LOW

- NAI_VALID_CHANNEL_LOW

- AVG_GCR_FLUX

- NUMBER_MET

F.9.  NEAR GRS Summarized Spatial (SPATIAL) Parameters:  13*(Real*4)

“Not valid” – is designated as = -1

- AVG_SC_DISTANCE						NOT VALID IF FOV_STATUS=0

- AVG_EMI_ANGLE						NOT VALID IF FOV_STATUS=0

- AVG_TOTAL_EFF_SOLID_ANGLE				=0  IF FOV_STATUS=0

- NR_WEIGHT_0.3MEV						=0  IF FOV_STATUS=0

- NC_WEIGHT_0.3MEV						=0  IF FOV_STATUS=0

- NR_WEIGHT_0.6MEV						=0  IF FOV_STATUS=0

- NC_WEIGHT_0.6MEV						=0  IF FOV_STATUS=0

- NR_WEIGHT_1.0MEV						=0  IF FOV_STATUS=0

- NC_WEIGHT_1.0MEV						=0  IF FOV_STATUS=0

- NR_WEIGHT_3.0MEV						=0  IF FOV_STATUS=0

- NC_WEIGHT_3.0MEV						=0  IF FOV_STATUS=0

11  NC_WEIGHT_6.0MEV						=0  IF FOV_STATUS=0

12  ESTIMATED_COVERED_AREA				=0  IF FOV_STATUS=0

G.  Listing of Parameters for GRS Fractional Footprint Accounting System

Internal to the University of Arizona RDBMS information for accounting purposes only.  This information will not be output at the query time.

- MET						INTEGER*4

- FOOTPRINT_BIN_TOTAL_AREA		REAL*4		NOT VALID

- FOOTPRINT_BIN_VISIBLE_AREA		REAL*4

- TOTAL_EFF_SOLID_ANGLE_FRACTION	REAL*4

- NR_SPECTRUM_FRACTION			REAL*4

- NC_SPECTRUM_FRACTION			REAL*4

- BINID						REAL*4

### Numerical Specifications for GRS level–2 Integral Records

FORMATS  1,2 – Numerical specifications for Single Raw and Calibrated GRS Level-2 record (output)

###### H.1. – H.7.   NEAR GRS Spectra:  4 spectra*1024 channels*(Real*4), 1 spectrum*21 channels(Real*4), 1 spectrum*1024 channels*(Real*4), 1 spectrum*21 channels(Real*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.

## H.8. NEAR GRS Science Housekeeping (SCI HK) Parameters:  31*(Integer*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.

## H.9. NEAR GRS Engineering Housekeeping (ENG HK) Parameters: 152*(REAL*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.

H.10. NEAR GRS Derived Engineering (DER ENG) Parameters:  17*(Real*4), 1*(Integer*4), 1*(4*Integer*4)

PARAMETER NAME                  	   UNITS

0	GAIN_STANDARD_BGO	         	keV/channel

1	GAIN_STANDARD_NAI		keV/channel

2	ZERO_STANDARD_BGO		      keV

3	ZERO_STANDARD_NAI		      keV

4     	REAL_GAIN_BGO	            	keV/channel 		a1+a2*x+a3*x^2+a4*x^3+...,

where x is day since launch,

a1..an are derived coefficients,

provided by GSFC.

5     	REAL_GAIN_NAI 			keV/channel 			 - “ -

6     	REAL_ZERO_BGO		         	     keV                     		 - “ -
7     	REAL_ZERO_NAI                    	     keV                     		- “ -

8    	LIVE_TIME_BGO                  	Unitless

LIVE_TIME_BGO =

NUM_BGO_VALID_EVENTS(SCI_HK:25)/(BGO_RAW_EVENT_RATE(SCI_HK:18) –

0.5*BGO_HIGH_ENERGY(ENG_HK:144))

9	LIVE_TIME_NAI			 Unitless

LIVE_TIME_NAI =

NUM_NAI_VALID_EVENTS(SCI_HK:24) / (NAI_RAW_EVENT_RATE(SCI_HK:17) -

0.2*NAI_HIGH_ENERGY(ENG_HK:141))

10	BGO_VALID_CHANNEL_HI     	  Channel		    Highest channel of the spectrum

in new (after calibration) energy scale.

=N(1021), if N(1021)<=1023

=1023,    if N(1021)>1023,

N(1021): channel number in the

new energy scale, corresponding

to 1021 channel of the original spectrum.

N(1021)=1021⋅REALGAIN...+REALZERO...−ZEROSTANDARD...GAINSTANDARD...

11    	NAI_VALID_CHANNEL_HI    	  Channel		            - “ -

12	BGO_VALID_CHANNEL_LOW    	  Channel		    Lowest channel of the spectrum

in new (after calibration) energy scale.

=N(LLD), if N(LLD)>=0

=0,    if N(LLD)<0,

N(LLD): channel number in the

new energy scale, corresponding

to low level discriminator setting of the

original spectrum N(LLD_0)=

BGO_LOW_LEVEL_AMPL_THRESH

(Eng_HK:111) or 10, whatever is higher.

- NAI_VALID_CHANNEL_LOW     	  Channel		    - “ -      N(LLD_0)=

NAI_LOW_LEVEL_AMPL_THRESH

(Eng_HK:110)

14     Pos_ESC_1                       		   keV			    Energy, corresponding to the

middle of 1-st escape window

(Eng_HK:106)

PosEsc1=(EngHK:106)⋅RealGainBGO+RealZeroBGO

15     Pos_ESC_2                       		   keV			    Energy, corresponding to the

middle of 2-nd escape window

(Eng_HK:108)

16      GCR_Flux                        		Counts/Seconds         	    Parameter reflects changes in

galactic cosmic ray flux.

Updated after calibration

GCRFLUX=I1I2NAI(I)LIVETIMENAI⋅INTEGRATIONTIME,NAI(I)− raw (uncalibrated) spectrumI1=(5000−RealZeroNaI)/RealGainNaII2=(9000−RealZeroNaI)/RealGainNaI

17    Bad_Flag

18    Query_ID

H.11.  NEAR GRS Spatial (SPATIAL) Parameters:  1*(Integer*4), 32*(Real*4)

All spatial derivations, unless specified differently, are made for the middle of integration period.

*  ET – ephemeris time corresponding to clock fixed Mission Elapsed Time in the middle of the integration

period.  Measuring units are seconds before January 1, 2000.

PARAMETER NAME           	UNITS

0  MET					Seconds		Mission Elapsed Time at start of integration

1  PlateID_Boresite_Intersect  		Plate Index    		ID of the plate that boresight intersects

in the middle of integration period

2  BinID_Boresite_Intersect    		Bin   Index    		ID of the plate that boresight intersects

in the middle of integration period

3  SC_Position, X              			Coord X, km    	Position of spacecraft (ET*) in Eros fixed

4  SC_Position, Y              			Coord Y, km    	coord. system (ET*)

5  SC_Position, Z              			Coord Z, km

6  BS_Vector, X                			Vector X, normal    	Boresight vector of GRS (ET*) in Erosfixed

7  BS_Vector, Y                			Vector Y, normal    	coord. system (ET*)

8  BS_Vector, Z                			Vector Z, normal

9  FOV_Status                  			Status value   		= 0:     Field of view (FOV) is completely

off  the asteroid (if there are no

plates in the field of view).

= 1:     Field of view  is on the asteroid

(totally or partially).

10  Downlink_Status            			Status Value    	= 0:     No downlink

= 1:     Yes downlink (The angle between

antenna direction and vector

spacecraft(ET*)→Earth(ET*)

is less than 2 degrees)

11 Average_SC_Distance	       		Kilometers     		Average distance from the spacecraft

to the surface of the asteroid

in the middle of integration period.

visibleplates  footprint,(Fractions of the plates are considered);SCDistance=Distance from the spacecraft to the midpoint of the plateEffPlateSolidAngle=PlateSolidAngle⋅InstrAngular_ResponseFct1.0MeVPlateSolidAngle=|Cos(emiangle)|⋅PlateAreaSCDistance2InstrAngular_ResponseFct1.0MeV:  see Appendix 1 for definition and Appendix 2                                                                       for approximation.

12  Avg_Emission_Angle             		Degrees   		Average emission angle

visibleplates  footprint,(Fractions of the plates are considered);EffPlateSolidAngle :    definition is given in specifications for parameter 11                                              of the present section.

13  Total_Eff_Solid_Angle          		Steradians 		Overall solid angle of all plates

in the footprint weighted by the instrument angular response function for the 1 MeV radiation.

visibleplates  footprint,(Fractions of the plates are considered)EffPlateSolidAngle  :    definition is given in specifications for parameter 11                                              of the present section.

14  Total_Area_Footprint		           Sq. Kilometers      	NOT VALID

15  Total_Visible_Area			Sq. Kilometers 	Total area of plates in the

footprint visible from the spacecraft

visibleplatesfootprint,footprint(PlateArea∗VISAMT),                                where VISAMT: fraction of the plate that is located within the field of view                                                       and is visible to the spacecraft.

16  Footprint_Solid_Angle          		Steradians 		Overall solid angle of all plates in the

footprint as seen from the spacecraft.

visibleplates   footprint,(Fractions of the plates are considered)

17  NR_Geom_Weight_0.3MeV          	Steradians		Normalization coefficient.  It relates the

gamma-ray flux at the detector and the intensity of gamma-ray radiation from the asteroid surface in the direction normal to the surface (for  0.3 MeV natural radioactivity radiation).

For complete definition see Appendix 1.

visible platesfootprint,(Fractions of the plates are considered)InstrAngular_ResponseFct0.3MeV:   see Appendix 1 for definition and Appendix 2                                                                       for approximation.PlateSolidAngle :    definition is given in specifications for parameter 11                                    of the present section.

18  NC_Geom_Weight_0.3MeV           	Steradians 		Normalization coefficient.  It relates the

gamma-ray flux at the detector and the intensity of gamma-ray radiation from the asteroid surface in the direction normal to the surface (for  0.3 MeV non-elastic scattering and thermal neutron capture radiation).

For complete definition see Appendix 1.

visible platesfootprint,(Fractions of the plates are considered)NCAngularFct  :  see Appendix 1 for definition and Appendix 2 for approximationInstrAngular_ResponseFct0.3MeV:   see Appendix 1 for definition and Appendix 2                                                                       for approximation.PlateSolidAngle  :   definition is given in specifications for parameter 11                                    of the present section.

19  NR_Geom_Weight_0.6MeV             	Steradians	 	Normalization coefficient.  It relates the

gamma-ray flux at the detector and the intensity of gamma-ray radiation from the asteroid surface in the direction normal to the surface (for  0.6 MeV natural radioactivity radiation).

For complete definition see Appendix 1.

visible platesfootprint,(Fractions of the plates are considered)InstrAngular_ResponseFct0.6MeV:   see Appendix 1 for definition and Appendix 2                                                                       for approximation.PlateSolidAngle  :   definition is given in specifications for parameter 11                                    of the present section.

20  NC_Geom_Weight_0.6MeV 		Steradians	 	Normalization coefficient.  It relates the

gamma-ray flux at the detector and the intensity of gamma-ray radiation from the asteroid surface in the direction normal to the surface (for  0.6 MeV non-elastic scattering and thermal neutron capture radiation).

For complete definition see Appendix 1.

visible platesfootprint,(Fractions of the plates are considered)NCAngularFct  :   see Appendix 1 for definition and Appendix 2 for approximationInstrAngular_ResponseFct0.6MeV:   see Appendix 1 for definition and Appendix 2                                                                       for approximation.PlateSolidAngle  :   definition is given in specifications for parameter 11                                    of the present section.

21  NR_Geom_Weight_1.0MeV                 Steradians 		Normalization coefficient.  It relates the

gamma-ray flux at the detector and the intensity of gamma-ray radiation from the asteroid surface in the direction normal to the surface (for  1.0 MeV natural radioactivity radiation).

For complete definition see Appendix 1.

visible platesfootprint,(Fractions of the plates are considered)InstrAngular_ResponseFct1.0MeV:   see Appendix 1 for definition and Appendix 2                                                                       for approximation.PlateSolidAngle  :    definition is given in specifications for parameter 11                                    of the present section.

22  NC_Geom_Weight_1.0MeV       		Steradians 		Normalization coefficient.  It relates the

gamma-ray flux at the detector and the intensity of gamma-ray radiation from the asteroid surface in the direction normal to the surface (for  1.0 MeV non-elastic scattering and thermal neutron capture radiation).

For complete definition see Appendix 1.

visible platesfootprint,(Fractions of the plates are considered)NCAngularFct  :  see Appendix 1 for definition and Appendix 2 for approximationInstrAngular_ResponseFct1.0MeV:   see Appendix 1 for definition and Appendix 2                                                                       for approximation.PlateSolidAngle  :    definition is given in specifications for parameter 11                                    of the present section.

23  NR_Geom_Weight_3.0MeV                   Steradians 		Normalization coefficient.  It relates the

gamma-ray flux at the detector and the intensity of gamma-ray radiation from the asteroid surface in the direction normal to the surface (for  3.0 MeV natural radioactivity radiation).

For complete definition see Appendix 1.

visible platesfootprint,(Fractions of the plates are considered)InstrAngular_ResponseFct3.0MeV:   see Appendix 1 for definition and Appendix 2                                                                       for approximation.PlateSolidAngle  :   definition is given in specifications for parameter 11                                    of the present section.

24  NC_Geom_Weight_3.0MeV           	Steradians 		Normalization coefficient.  It relates the

gamma-ray flux at the detector and the intensity of gamma-ray radiation from the asteroid surface in the direction normal to the surface (for  3.0 MeV non-elastic scattering and thermal neutron capture radiation).

For complete definition see Appendix 1.

visible platesfootprint,(Fractions of plates are considered)NCAngularFct  :   see Appendix 1 for definition and Appendix 2 for approximationInstrAngular_ResponseFct3.0MeV:        see Appendix 1 for definition and Appendix 2                                                                       for approximation.PlateSolidAngle  :     definition is given in specifications for parameter 11                                    of the present section.

25  NC_Geom_Weight_6.0MeV	           Steradians 		Normalization coefficient.  It relates the

gamma-ray flux at the detector and the intensity of gamma-ray radiation from the asteroid surface in the direction normal to the surface (for  6.0 MeV non-elastic scattering and thermal neutron capture radiation).

For complete definition see Appendix 1.

visible platesfootprint,(Fractions of the plates are considered)NCAngularFct  :   see Appendix 1 for definition and Appendix 2 for approximationInstrAngular_ResponseFct6.0MeV:        see Appendix 1 for definition and Appendix 2                                                                       for approximation.PlateSolidAngle  :     definition is given in specifications for parameter 11                                    of the present section.

26  Boresight_Intersect_Lat		Degrees 		Latitude of boresight intersection

in the middle of integration period

Coordinate transformation Asteroid Body Fixed, Cartesian to

Asteroid Body Fixed, Spherical. Intersection point (x,y,z) of

boresight with asteroid surface converted to Latitude.

LAT = Degrees(ARC_SIN(Z/R))            (R solved for in eqn 28)

27   Boresight_Intersect_Lon   	Degrees 		Longitude of boresight intersection

in the middle of integration period

Coordinate transformation Asteroid Body Fixed, Cartesian to

Asteroid Body Fixed, Spherical. Intersection point (x,y,z) of

boresight with asteroid surface converted to Longitude.

1.    LON = -1.*Degrees(ARC_TAN(Y/X))

2.    LON = 270				for x = 0 AND y > 0

3.    LON = 90					for x = 0 AND y < 0

28   Boresight_Intersect_R		Kilometers		Radius-vector of boresight intersection

in the middle of integration period

Coordinate transformation Asteroid Body Fixed, Cartesian to

Asteroid Body Fixed, Spherical. Intersection point (x,y,z) of

boresight with asteroid surface converted to Radius from

center of asteroid mass.

R =    SQRT( X^2 + Y^2 + Z^2)         (R is needed in sol’n 26)

29   SPICE_Kernel_ID             	Version Number 			Specified by NAIF

30   Plate_Model_ID              		Version Number

31   Processing_Software_ID         	Version Number

32   DataBase_Version_ID         	Version Number

#### I. Numerical Specifications for GRS Level–2 Summary Record

FORMAT  3 :  Numerical specifications for summarized GRS Level-2 record (output)

I.1. – I.7.   NEAR GRS Summarized Spectra: 4 spectra*1024 channels*(Real*4), 1 spectrum*21 channels(Real*4), 1 spectrum*1024 channels*(Real*4), 1 spectrum*21 channels(Real*4)

Measuring units:   Counts/Channel

Sodium Iodide summed spectrum:                		NAI(I)=METNAI(I,MET);   I=0..1023−channel

Bismuth Germanate summed spectrum:                	BGO(I)=METBGO(I,MET);    I=0,,1023−channel

Anti-Coincidence summed spectrum:                ANTICO(I)=METANTICO(I,MET);   I=0..1023−channel

Sodium Iodide single escape summed spectrum:

NAI1ESC(I)=METNAI1ESC(I,MET);   I=0..1023 - channel

Bismuth Germanate single escape summed spectrum:

BGO1ESC(I)=METBGO1ESC(I,MET);   I – channel, 0-20

Sodium Iodide double escape summed spectrum:

NAI2ESC(I)=METNAI2ESC(I,MET);    I=0..1023 - channel

Bismuth Germanate double escape summed spectrum:

BGO2ESC(I)=METBGO2ESC(I,MET); I – channel, 0-20

I.8.   NEAR GRS Summarized Engineering (ENG) Parameters:  13*(Real*4)

PARAMETER NAME                   	    UNITS

0	GAIN_STANDARD_BGO	         	  keV/Channel			User specified or default

1	GAIN_STANDARD_NAI		  keV/Channel				- “ -

2	ZERO_STANDARD_BGO		    keV		     			 - “ -

3	ZERO_STANDARD_NAI		    keV					- “ -

4	TOTAL_INTEG_TIME  	         	  Seconds       		      METIntegrationTime(MET)

5	TOTAL_LIVE_TIME_BGO               Seconds

MET[LIVETIMEBGO(MET)⋅INTEGRATIONTIME]

6	TOTAL_LIVE_TIME_NAI                Seconds

MET[LIVETIMENAI(MET)⋅INTEGRATIONTIME]

7	BGO_VALID_CHANNEL_HI     	   Channel		Highest channel of summed spectrum

uncorrupted by energy scale adjustments.

Lowest BGO_VALID_CHANNEL_HI among all MET records in a sum.

8	NAI_VALID_CHANNEL_HI    	   Channel	      		- “ –

Lowest NAI_VALID_CHANNEL_HI

among all MET records in a sum

9	BGO_VALID_CHANNEL_LOW    	   Channel		Lowest channel of summed spectrum

uncorrupted by energy scale adjustments.

Highest BGO_VALID_CHANNEL_LOW among all MET records in a sum.

10	NAI_VALID_CHANNEL_LOW     	   Channel	      		- “ –

Highest NAI_VALID_CHANNEL_LOW

among all MET records in a sum.

11	Avg_GCR_FLUX                     	   Counts/Second	Characteristics of average galactic cosmic

ray flux

METGCRFlux(MET)⋅LIVETIMENAI⋅INTEGRATIONTIMEMETLIVETIMENAI⋅INTEGRATIONTIME

12        Number_MET                     					Number of MET records in a query

I.9.  NEAR GRS Summarized Spatial (SPATIAL) Parameters:  13*(Real*4)

PARAMETER NAME             		UNITS

0	AVG_SC_DISTANCE      		       Kilometers 	Distance from the spacecraft to the surface

of the asteroid averaged over MET records in a sum.

Weight(MET)=TotalEffSolidAngle(MET)⋅LIVETIMENAI(MET)⋅INTEGRATIONTIMEAvgSCDISTANCE=MET[AvgSCDistance(MET)⋅Weight(MET))]MET[Weight(MET)]

1	AVG_EMI_ANGLE     		 Degrees 		Emission angle averaged over MET records

in a sum.

AVGEMIANGLE=MET[AvgEmissionAngle(MET)⋅Weight(MET))]MET[Weight(MET)]Weight(MET)  :    definition is given in specifications for parameter 0                          of the present section.

2     AVG_TOTAL_EFF_SOLID_ANGLE      Steradians 		Total effective solid angle averaged over

MET records in a sum.

MET[TotalEffSolidAngle(MET)⋅LIVETIMENAI(MET)⋅INTEGRATIONTIME]MET[LIVETIMENAI(MET)⋅INTEGRATIONTIME]

3     NR_Weight_0.3MeV              		Steradians*Seconds	Normalization coefficient.  It relates the

number of counts in summed spectrum gamma-line to the intensity of gamma-ray radiation of corresponding energy from the asteroid surface in the direction normal to the surface (for natural radioactivity radiation and energy range < 0.45 MeV).

For complete definition see Appendix 1.

MET[NRGeomWeight0.3MeV(MET)⋅LIVETIMENAI⋅INTEGRATIONTIME]

4     NC_Weight_0.3MeV    		Steradians*Counts 		Normalization coefficient.  It relates the

number of counts in summed spectrum gamma-line to the intensity of gamma-ray radiation of corresponding energy from the asteroid surface in the direction normal to the surface normalized for GCR flux = 1 proton/(s*cm2) (for non-elastic scattering and thermal neutron capture radiation and energy range < 0.45 MeV).

For complete definition see Appendix 1.

MET[NCGeomWeight0.3MeV(MET)⋅LIVETIMENAI⋅INTEGRATIONTIME⋅GCRFLUX]

5     NR_Weight_0.6MeV              	Steradians*Seconds		Normalization coefficient.  It relates the

number of counts in summed spectrum gamma-line to the intensity of gamma-ray radiation of corresponding energy from the asteroid surface in the direction normal to the surface (for natural radioactivity radiation and energy range 0.45..0.8 MeV).

For complete definition see Appendix 1.

MET[NRGeomWeight0.6MeV(MET)⋅LIVETIMENAI⋅INTEGRATIONTIME]

6     NC_Weight_0.6MeV		    Steradians*Counts 		Normalization coefficient.  It relates the

number of counts in summed spectrum gamma-line to the intensity of gamma-ray radiation of corresponding energy from the asteroid surface in the direction normal to the surface normalized for GCR flux = 1 proton/(s*cm2) (for non-elastic scattering and thermal neutron capture radiation and energy range 0.45..0.8 MeV).

For complete definition see Appendix 1.

MET[NCGeomWeight0.6MeV(MET)⋅LIVETIMENAI⋅INTEGRATIONTIME⋅GCRFLUX]

7     NR_Weight_1.0MeV         	Steradians*Seconds		Normalization coefficient.  It relates the

number of counts in summed spectrum gamma-line to the intensity of gamma-ray radiation of corresponding energy from the asteroid surface in the direction normal to the surface (for natural radioactivity radiation and energy range 0.8..2.0 MeV).

For complete definition see Appendix 1.

MET[NRGeomWeight1.0MeV(MET)⋅LIVETIMENAI⋅INTEGRATIONTIME]

8     NC_Weight_1.0MeV          Steradians*Counts 			Normalization coefficient.  It relates the

number of counts in summed spectrum gamma-line to the intensity of gamma-ray radiation of corresponding energy from the asteroid surface in the direction normal to the surface normalized for GCR flux = 1 proton/(s*cm2) (for non-elastic scattering and thermal neutron capture radiation and energy range 0.8..2.0 MeV).

For complete definition see Appendix 1.

MET[NCGeomWeight1.0MeV(MET)⋅LIVETIMENAI⋅INTEGRATIONTIME⋅GCRFLUX]

9     NR_Weight_3.0MeV         	Steradians*Seconds		Normalization coefficient.  It relates the

number of counts in summed spectrum gamma-line to the intensity of gamma-ray radiation of corresponding energy from the asteroid surface in the direction normal to the surface (for natural radioactivity radiation and energy range 2.0..4.5 MeV).

For complete definition see Appendix 1.

MET[NRGeomWeight3.0MeV(MET)⋅LIVETIMENAI⋅INTEGRATIONTIME]

10     NC_Weight_3.0MeV          	Steradians*Counts 		Normalization coefficient.  It relates the

number of counts in summed spectrum gamma-line to the intensity of gamma-ray radiation of corresponding energy from the asteroid surface in the direction normal to the surface normalized for GCR flux = 1 proton/(s*cm2) (for non-elastic scattering and thermal neutron capture radiation and energy range 2.0..4.5 MeV).

For complete definition see Appendix 1.

MET[NCGeomWeight3.0MeV(MET)⋅LIVETIMENAI⋅INTEGRATIONTIME⋅GCRFLUX]

11     NC_Weight_6.0MeV          Steradians*Counts			Normalization coefficient.  It relates the

number of counts in summed spectrum gamma-line to the intensity of gamma-ray radiation of corresponding energy from the asteroid surface in the direction normal to the surface normalized for GCR flux = 1 proton/(s*cm2) (for non-elastic scattering and thermal neutron capture radiation and energy range > 4.5 MeV).

For complete definition.. see Appendix 1.

MET[NCGeomWeight6.0MeV(MET)⋅LIVETIMENAI⋅INTEGRATIONTIME⋅GCRFLUX]

12    ESTIMATED_COVERED_AREA  	Sq. Kilometers		Estimated area of the asteroid surface

covered by MET records in a sum

Bins in a region BinArea(MET) 		For a spatial query

Different bins covered by all footprints in a sumBinArea 		For a temporal query

J.  Numerical Specifications for GRS Fractional Footprint Accounting System

PARAMETER NAME                      UNITS

0  	MET					Seconds		Mission Elapsed Time at start of integartion.

1  	Footprint_Bin_Total_Area	        Sq. Kilometers 		NOT VALID

2  	Footprint_Bin_Visible_Area	        Sq. Kilometers		Total area of all plates in the

footprint that belong to the bin, visible from the spacecraft.

visible from the spacecraf                plates of the footprint, that belong to the BIN,                   (Fractions of the plates are considered) 			Fractions are considered as in H11 param.15

3  	Total_Eff_Solid_Angle_Fraction      Unitless		Fraction of the total effective

solid angle that can be attributed to the bin.

visibleplates       footprint,   (Fractions of the plates are considered)EffPlateSolidAngle=PlateSolidAngle⋅InstrAngular_ResponseFct1.0MeVPlateSolidAngle=|Cos(emiangle)|⋅PlateAreaSCDistance2SCDistance= Distance from the spacecraft to the midpoint of the plateInstrAngular_ResponseFct1.0MeV:   see Appendix 1 for definition and Appendix 2                                                                       for approximation.

4  	NR_Spectrum_Fraction		Unitless		Fraction of the spectrum that can

be attributed to the bin

(for natural radioactivity radiation).

visible platesfootprint,(Fractions of the plates are considered)EffPlateSolidAngle)EffPlateSolidAngle :   definition is given in specifications for parameter 3                                            of the present section.

5	NC_Spectrum_Fraction		Unitless 		Fraction of the spectrum that can

be attributed to the bin

(for non-elastic scattering and thermal neutron capture radiation).

visible platesfootprint,(Fractions of the plates are considered)NCAngularFct  :   see Appendix 1 for definition and Appendix 2 for approximationEffPlateSolidAngle :   definition is given in specifications for parameter 3                                                of the present section.

6         Binid					Value			Bin identifier generated by the Bin

generation system

APPENDIX 1	  Definitions for:

Instr_Angular_Response_Fct_YYMeV,

where YY can be 0.3, 0.6, 1.0, 3.0 or 6.0;

NC_Angular_Fct;

NC_Geom_Weight_YYMeV;

NC_Weight_YYMeV;

NR_Geom_Weight_YYMeV;

NR_Weight_YYMeV

NjNC[counts]=Tlive[s]⋅Sdetector[cmdetector2]⋅effγ[countsphotons]⋅GammaFluxj[photons(s⋅cmdetector2)]GammaFluxj[photonss⋅cmdetector2]=I0[GCR_protonss⋅cmsurface2]⋅footprint(FjNC(β)4π[photonssr⋅GCRproton]⋅f(YY)CosβdΩ[sr⋅cmsurface2cmdetector2]),where     NjNC -  number of counts at the NaI detector in j energy line (nonelastic scattering or thermal neutron capture)                                      Ω   -    solid angle of an area on the asteroid surface as seen from the spacecraft             f(YY) -  angular response function of NaI detector (response, normalised to 1 at normal direction)                        for YY MeV photons, where YY = 0.3, 0.6, 1.0, 3.0 or 6.0.                 β  -      emission angle              Tlive-   livetime            Sdetector  -  area of the detector            effγ -    intrisic gamma-ray efficiency of the detector                    FjNC(β)4π=1I0⋅14π⋅0∞SjNC(x)exp(−μjxCosβ)dx    [photonssr⋅(GCRprotonsin a..b energy range)] - intensity of gamma-ray radiation,                                                                                         normalized to galactic-cosmic-ray flux=1 GCRprotonsin a..b energy ranges⋅cm2              μj- linear attenuation coefficient for j gamma line              SjNC(x)  - source function for galactic-cosmic-ray-produced j line photons  [photonss⋅cm3]  at the depth x cm              I0=ab(dNGCRdE)dE=GCRFlux[countss]Sdetector⋅effp[countsprotons];               a..b - energy range responsible for counts in 5MeV..9MeV energy window of the detector            effp -  intrisic efficiency of the detector for protons in a..b energy range.                        As galactic-cosmic-ray flux is isotropic I0=[GCRprotons(a..b)s⋅cmdet2]=[GCRprotons(a..b)s⋅cmsurf2]

GammaFluxj=I0⋅FjNC(0)⋅14π⋅footprint(FjNC(β)FjNC(0)⋅f(YY)CosβdΩ)=(I0⋅FjNC(0)4π)[photonssr⋅s⋅cmsurface2]⋅NCGeomWeightYYMeV,where     NCGeomWeightYYMeV=footprint(FjNC(β)FjNC(0)⋅f(YY)Cosβ)dΩ     [sr⋅cmsurface2cmdetector2],             Instr_Angular_Response_Fct_YYMeV =   f(YY),             NC_Angular_Fct =FjNC(β)FjNC(0)   , which is a weak function of energy j.

METNjNC=Sdetector[cmdetector2]⋅effγ[countsphotons]⋅FjNC(0)4π[photonssr⋅GCRproton]⋅METTlive[s]⋅I0[GCR_protons in a..b energy ranges⋅cmsurface2]⋅NCGeomWeightYYMeV[sr⋅cmsurface2cmdetector2]=effγeffp⋅FjNC(0)4π⋅NCWeightYYMeV,    where  NCWeightYYMeV=METTlive⋅GCRFlux⋅NCGeomWeightYYMeV

NjNR[counts]=Tlive[s]⋅Sdetector[cmdetector2]⋅effγ[countsphotons]⋅GammaFluxj[photons(s⋅cmdetector2)]GammaFluxj[photonss⋅cmdetector2]=footprint(FjNR(β)4π[photonssr⋅s⋅cmsurface2]⋅f(YY)CosβdΩ[sr⋅cmsurface2cmdetector2]),where    NjNR -  number of counts at the NaI detector in j energy line (natural radioactivity)                                      Ω ;  f(YY);  β;  Tlive;  Sdetector;  effγ -    same as above                            FjNR(β)4π=14π⋅0∞SjNR(x)exp(−μjxCosβ)dx           ≈            14π⋅SjNR⋅0∞exp(−μjxCosβ)dx=SjNR⋅Cosβ4π⋅μj                                                                         (assume SjNR=Const(x))              SjNR(x)  - source function for natural radioactivity photons  [photonss⋅cm3]  at the depth x cm

GammaFluxj=FjNR(0)⋅14π⋅footprint(FjNR(β)FjNR(0)⋅f(YY)CosβdΩ)=FjNR(0)4π[photonssr⋅s⋅cmsurface2]⋅NRGeomWeightYYMeV,where     NRGeomWeightYYMeV=footprint(Cosβ⋅f(YY)CosβdΩ)=footprintf(YY)dΩ     [sr⋅cmsurface2cmdetector2],                               Instr_Angular_Response_Fct_YYMeV=f(YY)            METNjNR=Sdetector[cmdetector2]⋅effγ[countsphotons]⋅FjNR(0)4π[photonssr⋅s⋅cmsurface2]⋅METTlive[s]⋅NRGeomWeightYYMeV[sr⋅cmsurface2cmdetector2]=Sdetector⋅effγ⋅FjNR(0)4π⋅NRWeightYYMeV,where   NRWeightYYMeV=METTlive⋅NRGeomWeightYYMeV

APPENDIX 2	 	Approximations for:

Instr_Angular_Response_Fct_YYMeV,

where YY can be 0.3, 0.6, 1.0, 3.0 or 6.0;

NC_Angular_Fct

InstrAngular_ResponseFct0.3MeV:3.905641⋅10−6⋅ϕ5−2.474219⋅10−4⋅ϕ4+5.405967⋅10−3⋅ϕ3−5.037063⋅10−2⋅ϕ2++1.953235⋅10−1⋅ϕ+1.003473 ,     when   0<=ϕ<=20 −9.74584⋅10−6⋅ϕ4+1.21503⋅10−3⋅ϕ3−5.52397⋅10−2⋅ϕ2+1.04087⋅ϕ−5.96365 ,   when   20<ϕ<=40 0.1 ,     when    ϕ>40InstrAngular_ResponseFct0.6MeV:    -1.94686⋅10−7⋅ϕ6+1.38911⋅10−5⋅ϕ5−3.70203⋅10−4⋅ϕ4++4.43077⋅10−3⋅ϕ3−2.32768⋅10−2⋅ϕ2+5.08814⋅10−2⋅ϕ+1.00741 ,     when   0<=ϕ<=20 1.11855⋅10−3⋅ϕ2−9.48114⋅10−2⋅ϕ+2.36814 ,     when   20<ϕ<=40 0.365 ,     when    ϕ>40 InstrAngular_ResponseFct1.0MeV:    5.94557⋅10−6⋅ϕ5−2.79531⋅10−4⋅ϕ4+4.37304⋅10−3⋅ϕ3−2.60801⋅10−2⋅ϕ2++5.28193⋅10−2⋅ϕ+1.00055 ,     when   0<=ϕ<=15 −1.44762⋅10−6⋅ϕ4+1.89333⋅10−4⋅ϕ3−8.0781⋅10−3⋅ϕ2+1.11267⋅10−1⋅ϕ+0.632857 ,   when   15<ϕ<=50 0.62 ,     when    ϕ>50InstrAngular_ResponseFct3.0MeV:   −8.333⋅10−7⋅ϕ3−8.65⋅10−4⋅ϕ2+1.2333⋅10−2⋅ϕ+1.0000 ,     when   0<=ϕ<=20 7.4⋅10−4⋅ϕ2−6.95⋅10−2⋅ϕ+1.988,     when   20<ϕ<=40 0.392,     when    ϕ>40 InstrAngular_ResponseFct6.0MeV:−1.36667⋅10−5⋅ϕ3−3.7⋅10−4⋅ϕ2+8.3666⋅10−3⋅ϕ+1.0000 ,     when   0<=ϕ<=20 9.85⋅10−4⋅ϕ2−8.535⋅10−2⋅ϕ+2.223,     when   20<ϕ<=40 0.385,     when    ϕ>40    where     ϕ[Degrees] - angle between the boresight and the direction from the spacecraft                                     to the midpoint of the plate                                 NC_Angular_Fct =     2.1568⋅10−8⋅β4−1.4891⋅10−6⋅β3−1.5412⋅10−4⋅β2−1.0227⋅10−3⋅β+1.0031,    0 <β<=87                            = 0,   β> 87where     β[Degrees] - emission angle

Figure 1.  GRS angular response function for radiation 0.3 MeV

Figure 2.  GRS angular response function for radiation 0.6 MeV

Figure 3.  GRS angular response function for radiation 1.0 MeV

Figure 4.  GRS angular response function for radiation 3.0 MeV

Figure 5.  GRS angular response function for radiation 6.0 MeV

Figure 6.  Angular dependence of non-elastic scattering and thermal neutron capture

gamma-ray emission from the surface (approximation).

## APPENDIX 3    Location of GRS Level-2 Parameters in the U of A Database

## E.8.   NEAR GRS Science Housekeeping (SCI HK) Parameters:  31*(Integer*4)

| #### No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) Gamma-Ray Spectrometer (GRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 0 | MET | ACQ_MET_TIME | All tables | All databases |
| 1 | UTC | ACQ_UTC_TIME | g_nai_spec_tab, g_bgo_spec_tab, g_anti_coin_spec_tab, g_sgl_esc_sft_nai_spec_tab, g_dbl_esc_sft_nai_spec_tab, g_sgl_esc_pk_bgo_spec_tab, g_dbl_esc_pk_bgo_spec_tab | near_grs_db |
|  |  |  | grs_hk_tab | near_hk_db |
| 2 | DQI | DQI | index_tab | near_grs_db |
| 3 | TIME_TAG | ACQ_MET_TIME | - All tables | - All databases |
| 4 | INTEGRATION_TIME | INT_TIME | g_nai_spec_tab, g_bgo_spec_tab, g_anti_coin_spec_tab, g_sgl_esc_sft_nai_spec_tab, g_dbl_esc_sft_nai_spec_tab, g_sgl_esc_pk_bgo_spec_tab, g_dbl_esc_pk_bgo_spec_tab | - near_grs_db |
| 5 | BGO_PROTON_FLARE_FLG | BGO_PROTON_FLARE_FLG | - grs_hk_tab | - near_hk_db |
| 6 | BGO_LEVEL_1_SAFE_FLG | LVL1_SENSOR_SAFING_FLAG | - g_bgo_spec_tab | - near_grs_db |
| 7 | BGO_LEVEL_2_SAFE_FLG | LVL2_SENSOR_SAFING_FLAG | - g_bgo_spec_tab | - near_grs_db |
| 8 | NAI_LEVEL_1_SAFE_FLG | LVL1_SENSOR_SAFING_FLAG | - g_nai_spec_tab | - near_grs_db |
| 9 | NAI_LEVEL_2_SAFE_FLG | LVL2_SENSOR_SAFING_FLAG | - g_nai_spec_tab | - near_grs_db |
| 10 | NAI_INT_OVERFLOW_FLG | SPEC_BOF | - g_nai_spec_tab | - near_grs_db |
| 11 | BGO_INT_OVERFLOW_FLG | SPEC_BOF | - g_bgo_spec_tab | - near_grs_db |
| 12 | NAI_ANTICOINCID_OVFL_FLG | SPEC_BOF | - g_anti_coin_spec_tab | - near_grs_db |
| 13 | NAI_SINGLE_ESCP_OVFL_FLG | SPEC_BOF | - g_sgl_esc_sft_nai_spec_tab | - near_grs_db |
| 14 | NAI_DOUBLE_ESCP_OVFL_FLG | SPEC_BOF | - g_dbl_esc_sft_nai_spec_tab | - near_grs_db |
| 15 | BGO_SINGLE_ESCP_OVFL_FLG | SPEC_BOF | - g_sgl_esc_pk_bgo_spec_tab | - near_grs_db |
| 16 | BGO_DOUBLE_ESCP_OVFL_FLG | SPEC_BOF | - g_dbl_esc_pk_bgo_spec_tab | - near_grs_db |
| 17 | NAI_RAW_EVENT_RATE | TOTAL_RAW_EVENT_RATE | - g_nai_spec_tab | - near_grs_db |
| 18 | BGO_RAW_EVENT_RATE | TOTAL_RAW_EVENT_RATE | - g_bgo_spec_tab | - near_grs_db |
| 19 | COINCIDENCE_COUNTER | COIN_COUNTER | - g_nai_spec_tab | - near_grs_db |
| 20 | COMMAND_COUNTER | COMMAND_COUNTER | - g_bgo_spec_tab | - near_grs_db |
| 21 | PROBLEM_COUNTER | PROBLEM_COUNTER | - g_bgo_spec_tab | - near_grs_db |
| 22 | PROBLEM_CODE | PROBLEM_CODE | - g_bgo_spec_tab | - near_grs_db |
| 23 | NUM_NAI_ANTICOINCID_EVENTS | NUM_NAI_EVENTS_WITH_ANTI_COIN | - g_nai_spec_tab, g_bgo_spec_tab | - near_grs_db |
| 24 | NUM_NAI_VALID_EVENTS | NUM_VALID_EVENTS_PROCESSED | - g_nai_spec_tab | - near_grs_db |
| 25 | NUM_BGO_VALID_EVENTS | NUM_VALID_EVENTS_PROCESSED | - g_bgo_spec_tab | - near_grs_db |
| 26 | NAI_DC_OFFSET | DC_OFFSET_EST | - g_nai_spec_tab | - near_grs_db |
| 27 | BGO_DC_OFFSET | DC_OFFSET_EST | - g_bgo_spec_tab | - near_grs_db |
| 28 | NAI_STAND_DEV | STD_DEV | - g_nai_spec_tab | - near_grs_db |
| 29 | BGO_STAND_DEV | STD_DEV | - g_bgo_spec_tab | - near_grs_db |
| 30 | # GRAY_DATA_QUALITY_BIT | # GRAY_DQI | - g_nai_spec_tab | - near_grs_db |

## E.9. NEAR GRS Engineering Housekeeping (ENG HK) Parameters: 152*(REAL*4)

| #### No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) Gamma-Ray Spectrometer (GRS) Ground System” | # PARAMETER NAME IN # UA DATABASE |  | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- | --- |
| 0 | DETECTOR_ELECTR_CURR | DETECTOR_ELECTR_CURR |  | grs_hk_tab | near_hk_db |
| 1 | XRAY_PIN_DIODE_FET_BIAS | XRAY_PIN_DIODE_FET_BIAS |  | grs_hk_tab | near_hk_db |
| 2 | HVCE_TEMPERATURE | HVCE_TEMPERATURE |  | grs_hk_tab | near_hk_db |
| 3 | UNF_XRAY_HVPS_VOLT | UNF_XRAY_HVPS_VOLT |  | grs_hk_tab | near_hk_db |
| 4 | MG_XRAY_HVPS_VOLT | MG_XRAY_HVPS_VOLT |  | grs_hk_tab | near_hk_db |
| 5 | AL_XRAY_HVPS_VOLT | AL_XRAY_HVPS_VOLT |  | grs_hk_tab | near_hk_db |
| 6 | XRAY_GAS_HVPS_VOLT | XRAY_GAS_HVPS_VOLT |  | grs_hk_tab | near_hk_db |
| 7 | XRAY_PIN_HVPS_VOLT | XRAY_PIN_HVPS_VOLT |  | grs_hk_tab | near_hk_db |
| 8 | NAI_HVPS_VOLT | NAI_HVPS_VOLT |  | grs_hk_tab | near_hk_db |
| 9 | BGO_HVPS_VOLT | BGO_HVPS_VOLT |  | grs_hk_tab | near_hk_db |
| 10 | DPU_DC_DC_TEMP | DPU_DC_DC_TEMP |  | grs_hk_tab | near_hk_db |
| 11 | XGRS_DC_DC_TEMP | XGRS_DC_DC_TEMP |  | grs_hk_tab | near_hk_db |
| 12 | UNF_XRAY_HVPS_TEMP | UNF_XRAY_HVPS_TEMP |  | grs_hk_tab | near_hk_db |
| 13 | MG_XRAY_HVPS_TEMP | MG_XRAY_HVPS_TEMP |  | grs_hk_tab | near_hk_db |
| 14 | AL_XRAY_HVPS_TEMP | AL_XRAY_HVPS_TEMP |  | grs_hk_tab | near_hk_db |
| 15 | XRAY_GAS_HVPS_TEMP | XRAY_GAS_HVPS_TEMP |  | grs_hk_tab | near_hk_db |
| 16 | XRAY_PIN_HVPS_TEMP | XRAY_PIN_HVPS_TEMP |  | grs_hk_tab | near_hk_db |
| 17 | NAI_HVPS_TEMP | NAI_HVPS_TEMP |  | grs_hk_tab | near_hk_db |
| 18 | BGO_HVPS_TEMP | BGO_HVPS_TEMP |  | grs_hk_tab | near_hk_db |
| 19 | TEC_CURRENT | TEC_CURRENT |  | grs_hk_tab | near_hk_db |
| 20 | NAI_HVPS_VOLT_CMD | NAI_HVPS_VOLT_CMD |  | grs_hk_tab | near_hk_db |
| 21 | BGO_HVPS_VOLT_CMD | BGO_HVPS_VOLT_CMD |  | grs_hk_tab | near_hk_db |
| 22 | UNF_HVPS_VOLT_CMD | UNF_HVPS_VOLT_CMD |  | grs_hk_tab | near_hk_db |
| 23 | MG_HVPS_VOLT_CMD | MG_HVPS_VOLT_CMD |  | grs_hk_tab | near_hk_db |
| 24 | AL_HVPS_VOLT_CMD | AL_HVPS_VOLT_CMD |  | grs_hk_tab | near_hk_db |
| 25 | HVPS_POWER_ON_OFF | HVPS_POWER_ON_OFF |  | grs_hk_tab | near_hk_db |
| 26 | XRAY_CALIB_HOME_FLG | XRAY_CALIB_HOME_FLG |  | grs_hk_tab | near_hk_db |
| 27 | XRAY_CALIB_MOTOR_ON_OFF | XRAY_CALIB_MOTOR_ON_OFF |  | grs_hk_tab | near_hk_db |
| 28 | XRAY_CALIB_MOTOR_ENABLE | XRAY_CALIB_MOTOR_ENABLE |  | grs_hk_tab | near_hk_db |
| 29 | GAS_HVPS_VOLT_CMD | GAS_HVPS_VOLT_CMD |  | grs_hk_tab | near_hk_db |
| 30 | PIN_HVPS_VOLT_CMD | PIN_HVPS_VOLT_CMD |  | grs_hk_tab | near_hk_db |
| 31 | GAMMA_RAY_HEATER_CURR | GAMMA_RAY_HEATER_CURR |  | grs_hk_tab | near_hk_db |
| 32 | spare | spare |  | grs_hk_tab | near_hk_db |
| 33 | XRAY_CALIB_MOTOR_DIR | XRAY_CALIB_MOTOR_DIR |  | grs_hk_tab | near_hk_db |
| 34 | XRAY_CALIB_MOTOR_CURR | XRAY_CALIB_MOTOR_CURR |  | grs_hk_tab | near_hk_db |
| 35 | XRAY_CALIB_MOTOR_GOAL | XRAY_CALIB_MOTOR_GOAL |  | grs_hk_tab | near_hk_db |
| 36 | XRAY_CALIB_MOTOR_POS | XRAY_CALIB_MOTOR_POS |  | grs_hk_tab | near_hk_db |
| 37 | XRAY_CALIB_MOTOR_FID_SENS | XRAY_CALIB_MOTOR_FID_SENS |  | grs_hk_tab | near_hk_db |
| 38 | XRAY_CALIB_MOTOR_FID_BRIT | XRAY_CALIB_MOTOR_FID_BRIT |  | grs_hk_tab | near_hk_db |
| 39 | XRAY_MOTOR_IN_CALIB_POSITION | XRAY_MOTOR_IN_CALIB_POSITION |  | grs_hk_tab | near_hk_db |
| 40 | XRAY_MOTOR_IN_NORM_POSITION | XRAY_MOTOR_IN_NORM_POSITION |  | grs_hk_tab | near_hk_db |
| 41 | ACTIVE_SOLAR_TOGGLE_MODE | ACTIVE_SOLAR_TOGGLE_MODE |  | grs_hk_tab | near_hk_db |
| 42 | XRAY_CALIB_MAX_STEPS | XRAY_CALIB_MAX_STEPS |  | grs_hk_tab | near_hk_db |
| 43 | spare | spare |  | grs_hk_tab | near_hk_db |
| 44 | TEC_ENABLE | TEC_ENABLE |  | grs_hk_tab | near_hk_db |
| 45 | XRAY_CALIB_CUM_MOTOR_STPS | XRAY_CALIB_CUM_MOTOR_STPS |  | grs_hk_tab | near_hk_db |
| 46 | GRAY_BURST_THRESH_VAL | GRAY_BURST_THRESH_VAL |  | grs_hk_tab | near_hk_db |
| 47 | XRAY_SAFING_THRESH | XRAY_SAFING_THRESH |  | grs_hk_tab | near_hk_db |
| 48 | XRAY_CALIB_MOTOR_DIAG | XRAY_CALIB_MOTOR_DIAG |  | grs_hk_tab | near_hk_db |
| 49 | TEC_POWER_ON_OFF | TEC_POWER_ON_OFF |  | grs_hk_tab | near_hk_db |
| 50 | TEC_MODE | TEC_MODE |  | grs_hk_tab | near_hk_db |
| 51 | TEC_COOL_HEAT_FLG | TEC_COOL_HEAT_FLG |  | grs_hk_tab | near_hk_db |
| 52 | TEC_TEMPERATURE | TEC_TEMPERATURE |  | grs_hk_tab | near_hk_db |
| 53 | TEC_COOL_MODE_TEMP_UP_LMT | TEC_COOL_MODE_TEMP_UP_LMT |  | grs_hk_tab | near_hk_db |
| 54 | TEC_COOL_MODE_TEMP_LOW_LMT | TEC_COOL_MODE_TEMP_LOW_LMT |  | grs_hk_tab | near_hk_db |
| 55 | TEC_HEAT_MODE_TEMP_UP_LMT | TEC_HEAT_MODE_TEMP_UP_LMT |  | grs_hk_tab | near_hk_db |
| 56 | TEC_HEAT_MODE_TEMP_LOW_LMT | TEC_HEAT_MODE_TEMP_LOW_LMT |  | grs_hk_tab | near_hk_db |
| 57 | GAMMA_RAY_TEMP_HYSTERESIS | GAMMA_RAY_TEMP_HYSTERESIS |  | grs_hk_tab | near_hk_db |
| 58 | GAMMA_RAY_HEAT_TEMP_LOW_LMT | GAMMA_RAY_HEAT_TEMP_LOW_LMT |  | grs_hk_tab | near_hk_db |
| 59 | FULL_GAMMA_RAY_SCIENCE_MODE | FULL_GAMMA_RAY_SCIENCE_MODE |  | grs_hk_tab | near_hk_db |
| 60 | FULL_XRAY_SCIENCE_MODE | FULL_XRAY_SCIENCE_MODE |  | grs_hk_tab | near_hk_db |
| 61 | SUMM_GRAY_XRAY_SCIENCE_MODE | SUMM_GRAY_XRAY_SCIENCE_MODE |  | grs_hk_tab | near_hk_db |
| 62 | GRAY_BURST_SCI_REC_MODE | GRAY_BURST_SCI_REC_MODE |  | grs_hk_tab | near_hk_db |
| 63 | HVCE_TEMP_SET_POINT | HVCE_TEMP_SET_POINT |  | grs_hk_tab | near_hk_db |
| 64 | CMDED_END_XRAY_INTEG_PER | CMDED_END_XRAY_INTEG_PER |  | grs_hk_tab | near_hk_db |
| 65 | CMDED_GAMMA_RAY_INTEG_PER | CMDED_GAMMA_RAY_INTEG_PER |  | grs_hk_tab | near_hk_db |
| 66 | BGO_PROTON_FLARE_FLG | BGO_PROTON_FLARE_FLG |  | grs_hk_tab | near_hk_db |
| 67 | GAMMA_RAY_BIN_OVER_FLG_CMD | GAMMA_RAY_BIN_OVER_FLG_CMD |  | grs_hk_tab | near_hk_db |
| 68 | UNF_XRAY_BIN_OVER_FLG_CMD | UNF_XRAY_BIN_OVER_FLG_CMD |  | grs_hk_tab | near_hk_db |
| 69 | MG_BIN_OVER_FLG_CMD | MG_BIN_OVER_FLG_CMD |  | grs_hk_tab | near_hk_db |
| 70 | AL_BIN_OVER_FLG_CMD | AL_BIN_OVER_FLG_CMD |  | grs_hk_tab | near_hk_db |
| 71 | ACT_BIN_OVER_FLG_CMD | ACT_BIN_OVER_FLG_CMD |  | grs_hk_tab | near_hk_db |
| 72 | NAI_BIN_OVERFLOW_FLG | NAI_BIN_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 73 | BGO_BIN_OVERFLOW_FLG | BGO_BIN_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 74 | NAI_ANTICOIN_OVERFLOW_FLG | NAI_ANTICOIN_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 75 | NAI_SING_ESC_OVERFLOW_FLG | NAI_SING_ESC_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 76 | NAI_DOUB_ESC_OVERFLOW_FLG | NAI_DOUB_ESC_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 77 | BGO_SING_ESC_OVERFLOW_FLG | BGO_SING_ESC_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 78 | BGO_DOUB_ESC_OVERFLOW_FLG | BGO_DOUB_ESC_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 79 | UNF_XRAY_BIN_OVERFLOW_FLG | UNF_XRAY_BIN_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 80 | MG_BIN_OVERFLOW_FLG | MG_BIN_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 81 | AL_BIN_OVERFLOW_FLG | AL_BIN_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 82 | ACT_BIN_OVERFLOW_FLG | ACT_BIN_OVERFLOW_FLG |  | grs_hk_tab | near_hk_db |
| 83 | GAMMA_RAY_HEAT_ON_OFF | GAMMA_RAY_HEAT_ON_OFF |  | grs_hk_tab | near_hk_db |
| 84 | GAMMA_RAY_HEAT_MODE | GAMMA_RAY_HEAT_MODE |  | grs_hk_tab | near_hk_db |
| 85 | BGO_LEVEL_1_SAFING_FLG | BGO_LEVEL_1_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 86 | BGO_LEVEL_2_SAFING_FLG | BGO_LEVEL_2_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 87 | spare | spare |  | grs_hk_tab | near_hk_db |
| 88 | XRAY_INTEGR_TIME | XRAY_INTEGR_TIME |  | grs_hk_tab | near_hk_db |
| 89 | HVPS_ON_OFF | HVPS_ON_OFF |  | grs_hk_tab | near_hk_db |
| 90 | PIN_SENSOR_MASK_FLG | PIN_SENSOR_MASK_FLG |  | grs_hk_tab | near_hk_db |
| 91 | GAS_SENSOR_MASK_FLG | GAS_SENSOR_MASK_FLG |  | grs_hk_tab | near_hk_db |
| 92 | AL_SENSOR_MASK_FLG | AL_SENSOR_MASK_FLG |  | grs_hk_tab | near_hk_db |
| 93 | MG_SENSOR_MASK_FLG | MG_SENSOR_MASK_FLG |  | grs_hk_tab | near_hk_db |
| 94 | UNF_SENSOR_MASK_FLG | UNF_SENSOR_MASK_FLG |  | grs_hk_tab | near_hk_db |
| 95 | BGO_SENSOR_MASK_FLG | BGO_SENSOR_MASK_FLG |  | grs_hk_tab | near_hk_db |
| 96 | NAI_SENSOR_MASK_FLG | NAI_SENSOR_MASK_FLG |  | grs_hk_tab | near_hk_db |
| 97 | UNF_XRAY_RISE_TIME_THRESH | UNF_XRAY_RISE_TIME_THRESH |  | grs_hk_tab | near_hk_db |
| 98 | MG_RISE_TIME_THRESH | MG_RISE_TIME_THRESH |  | grs_hk_tab | near_hk_db |
| 99 | AL_RISE_TIME_THRESH | AL_RISE_TIME_THRESH |  | grs_hk_tab | near_hk_db |
| 100 | GAS_RISE_TIME_THRESH | GAS_RISE_TIME_THRESH |  | grs_hk_tab | near_hk_db |
| 101 | UNF_RISE_TIME_VALID_THRESH | UNF_RISE_TIME_VALID_THRESH |  | grs_hk_tab | near_hk_db |
| 102 | MG_RISE_TIME_VALID_THRESH | MG_RISE_TIME_VALID_THRESH |  | grs_hk_tab | near_hk_db |
| 103 | AL_RISE_TIME_VALID_THRESH | AL_RISE_TIME_VALID_THRESH |  | grs_hk_tab | near_hk_db |
| 104 | GAS_RISE_TIME_VALID_THRESH | GAS_RISE_TIME_VALID_THRESH |  | grs_hk_tab | near_hk_db |
| 105 | GAMMA_RAY_INTEGR_TIME | GAMMA_RAY_INTEGR_TIME |  | grs_hk_tab | near_hk_db |
| 106 | GAMMA_SING_ESC_WIND_CENTER | GAMMA_SING_ESC_WIND_CENTER |  | grs_hk_tab | near_hk_db |
| 017 | GAMMA_SING_ESC_WIND_WIDTH | GAMMA_SING_ESC_WIND_WIDTH |  | grs_hk_tab | near_hk_db |
| 108 | GAMMA_DOUB_ESC_WIND_CENTER | GAMMA_DOUB_ESC_WIND_CENTER |  | grs_hk_tab | near_hk_db |
| 109 | GAMMA_DOUB_ESC_WIND_WIDTH | GAMMA_DOUB_ESC_WIND_WIDTH |  | grs_hk_tab | near_hk_db |
| 110 | NAI_LOW_LEVEL_AMPL_THRESH | NAI_LOW_LEVEL_AMPL_THRESH |  | grs_hk_tab | near_hk_db |
| 111 | BGO_LOW_LEVEL_AMPL_THRESH | BGO_LOW_LEVEL_AMPL_THRESH |  | grs_hk_tab | near_hk_db |
| 112 | UNF_LOW_LEVEL_AMPL_THRESH | UNF_LOW_LEVEL_AMPL_THRESH |  | grs_hk_tab | near_hk_db |
| 113 | MG_LOW_LEVEL_AMPL_THRESH | MG_LOW_LEVEL_AMPL_THRESH |  | grs_hk_tab | near_hk_db |
| 114 | AL_LOW_LEVEL_AMPL_THRESH | AL_LOW_LEVEL_AMPL_THRESH |  | grs_hk_tab | near_hk_db |
| 115 | GAS_LOW_LEVEL_AMPL_THRESH | GAS_LOW_LEVEL_AMPL_THRESH |  | grs_hk_tab | near_hk_db |
| 116 | PIN_LOW_LEVEL_AMPL_THRESH | PIN_LOW_LEVEL_AMPL_THRESH |  | grs_hk_tab | near_hk_db |
| 117 | GAMMA_RAY_SENSOR_TEMP | GAMMA_RAY_SENSOR_TEMP |  | grs_hk_tab | near_hk_db |
| 118 | NAI_LEVEL_1_SAFING_FLG | NAI_LEVEL_1_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 119 | UNF_LEVEL_1_SAFING_FLG | UNF_LEVEL_1_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 120 | MG_LEVEL_1_SAFING_FLG | MG_LEVEL_1_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 121 | AL_LEVEL_1_SAFING_FLG | AL_LEVEL_1_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 122 | GAS_LEVEL_1_SAFING_FLG | GAS_LEVEL_1_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 123 | PIN_LEVEL_1_SAFING_FLG | PIN_LEVEL_1_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 124 | NAI_LEVEL_2_SAFING_FLG | NAI_LEVEL_2_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 125 | UNF_LEVEL_2_SAFING_FLG | UNF_LEVEL_2_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 126 | MG_LEVEL_2_SAFING_FLG | MG_LEVEL_2_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 127 | AL_LEVEL_2_SAFING_FLG | AL_LEVEL_2_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 128 | GAS_LEVEL_2_SAFING_FLG | GAS_LEVEL_2_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 129 | PIN_LEVEL_2_SAFING_FLG | PIN_LEVEL_2_SAFING_FLG |  | grs_hk_tab | near_hk_db |
| 130 | XRAY_SENS_SAFING_REST_LEVEL | XRAY_SENS_SAFING_REST_LEVEL |  | grs_hk_tab | near_hk_db |
| 131 | GAMMA_SENS_SAFING_REST_LEVEL | GAMMA_SENS_SAFING_REST_LEVEL |  | grs_hk_tab | near_hk_db |
| 132 | XRAY_SAFING_REST_MAX_RETRY | XRAY_SAFING_REST_MAX_RETRY |  | grs_hk_tab | near_hk_db |
| 133 | GAMMA_SAFING_REST_MAX_RETRY | GAMMA_SAFING_REST_MAX_RETRY |  | grs_hk_tab | near_hk_db |
| 134 | GAMMA_HEATER_DUTY_CYCLE | GAMMA_HEATER_DUTY_CYCLE |  | grs_hk_tab | near_hk_db |
| 135 | GAMMA_SENS_SAFING_THRESH | GAMMA_SENS_SAFING_THRESH |  | grs_hk_tab | near_hk_db |
| 136 | HVPS_CURRENT | HVPS_CURRENT |  | grs_hk_tab | near_hk_db |
| 137 | BGO_PROTON_FLARE_THRESH | BGO_PROTON_FLARE_THRESH |  | grs_hk_tab | near_hk_db |
| 138 | VERSION_NUMBER | VERSION_NUMBER |  | grs_hk_tab | near_hk_db |
| 139 | NAI_SPECTRUM_0 | SPECTRUM_0 |  | g_nai_spec_tab | near_grs_db |
| 140 | HW_NAI_RAW | HW_RAW |  | g_nai_spec_tab | near_grs_db |
| 141 | NAI_HIGH_ENERGY | HIGH_ENERGY |  | g_nai_spec_tab | near_grs_db |
| 142 | BGO_SPECTRUM_0 | SPECTRUM_0 |  | g_bgo_spec_tab | near_grs_db |
| 143 | HW_BG | HW_RAW | g_bgo_spec_tab |  | near_grs_db |
| 144 | BGO_HIGH_ENERGY | HIGH_ENERGY | g_bgo_spec_tab |  | near_grs_db |
| 145 | NAI_ANTICOIN_SPECTRUM_0 | SPECTRUM_0 | g_anti_coin_spec_tab |  | near_grs_db |
| 146 | HW_NAI_ANTICOIN | HW_RAW | g_anti_coin_spec_tab |  | near_grs_db |
| 147 | NAI_ANTICOIN_HIGH_ENERGY | HIGH_ENERGY | g_anti_coin_spec_tab |  | near_grs_db |
| 148 | SINGLE_ESC_SPECTRUM_0 | SPECTRUM_0 | g_sgl_esc_sft_nai_spec_tab |  | near_grs_db |
| 149 | SINGLE_ESC_HIGH_ENERGY | HIGH_ENERGY | g_sgl_esc_sft_nai_spec_tab |  | near_grs_db |
| 150 | DOUBLE_ESC_SPECTRUM_0 | SPECTRUM_0 | g_dbl_esc_sft_nai_spec_tab |  | near_grs_db |
| 151 | DOUBLE_ESC_HIGH_ENERGY | HIGH_ENERGY | g_dbl_esc_sft_nai_spec_tab |  | near_grs_db |

### E.10. NEAR GRS Derived Engineering (DER ENG) Parameters:  17*(Real*4), 1*(Integer*4), 1*(4*Integer*4)

| #### No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) Gamma-Ray Spectrometer (GRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 4 | REAL_GAIN_BGO | BGO_GAIN | grs_calib_tab | near_grs_db |
| 5 | REAL_GAIN_NAI | NAI_GAIN | grs_calib_tab | near_grs_db |
| 6 | REAL_ZERO_BGO | BGO_ZERO | grs_calib_tab | near_grs_db |
| 7 | REAL_ZERO_NAI | NAI_ZERO | grs_calib_tab | near_grs_db |
| 17 | BAD_FLAG | BAD_CODE | grs_calib_tab | near_grs_db |

E.11.  NEAR GRS Spatial (SPATIAL) Parameters:  1*(Integer*4), 32*(Real*4)

| No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) Gamma-Ray Spectrometer (GRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 0 | #### MET | ACQ_MET_TIME | s_grs_spatial_tab | near_spatial_db |
| 1 | #### PLATEID_BORESITE_INTERSECT | BSIGHT_PLATE_ID | s_grs_spatial_tab | near_spatial_db |
| 2 | #### BINID_BORESIGHT_INTERSECT | BSIGHT_BIN_ID | s_grs_spatial_tab | near_spatial_db |
| 3 | #### SC_POSITION, X | SC_POS_X | s_grs_spatial_tab | near_spatial_db |
| 4 | #### SC_POSITION, Y | SC_POS_Y | s_grs_spatial_tab | near_spatial_db |
| 5 | #### SC_POSITION, Z | SC_POS_Z | s_grs_spatial_tab | near_spatial_db |
| 6 | #### BS_VECTOR, X | BSIGHT_X | s_grs_spatial_tab | near_spatial_db |
| 7 | #### BS_VECTOR, Y | BSIGHT_Y | s_grs_spatial_tab | near_spatial_db |
| 8 | #### BS_VECTOR, Z | BSIGHT_Z | s_grs_spatial_tab | near_spatial_db |
| 9 | #### FOV_STATUS | FOV_STATUS | s_grs_spatial_tab | near_spatial_db |
| 10 | #### DOWNLINK_STATUS | DOWNLINK_FLAG | s_grs_spatial_tab | near_spatial_db |
| 11 | #### AVERAGE_SC_DISTANCE | AVG_SC_DISTANCE | s_grs_spatial_tab | near_spatial_db |
| 12 | #### AVERAGE_EMISSION_ANGLE | AVG_EMI_ANGLE | s_grs_spatial_tab | near_spatial_db |
| 13 | #### TOTAL_EFF_SOLID_ANGLE | TOT_EFF_SOLID_ANGLE | s_grs_spatial_tab | near_spatial_db |
| 14 | #### TOTAL_AREA_FOOTPRINT | TOT_AREA | s_grs_spatial_tab | near_spatial_db |
| 15 | #### TOTAL_VISIBLE_AREA | TOT_VIS_AREA | s_grs_spatial_tab | near_spatial_db |
| 16 | #### FOOTPRINT_SOLID_ANGLE | TOT_SOLID_ANGLE | s_grs_spatial_tab | near_spatial_db |
| 17 | #### NR_WEIGHT_0.3MEV | NR_WEIGHT_03 | s_grs_spatial_tab | near_spatial_db |
| 18 | #### NC_WEIGHT_0.3MEV | NC_WEIGHT_03 | s_grs_spatial_tab | near_spatial_db |
| 19 | #### NR_WEIGHT_0.6MEV | NR_WEIGHT_06 | s_grs_spatial_tab | near_spatial_db |
| 20 | #### NC_WEIGHT_0.6MEV | NC_WEIGHT_06 | s_grs_spatial_tab | near_spatial_db |
| 21 | #### NR_WEIGHT_1.0MEV | NR_WEIGHT_10 | s_grs_spatial_tab | near_spatial_db |
| 22 | #### NC_WEIGHT_1.0MEV | NC_WEIGHT_10 | s_grs_spatial_tab | near_spatial_db |
| 23 | #### NR_WEIGHT_3.0MEV | NR_WEIGHT_30 | s_grs_spatial_tab | near_spatial_db |
| 24 | #### NC_WEIGHT_3.0MEV | NC_WEIGHT_30 | s_grs_spatial_tab | near_spatial_db |
| 25 | #### NC_WEIGHT_6.0MEV | NC_WEIGHT_60 | s_grs_spatial_tab | near_spatial_db |
| 26 | #### BORESIGHT_INTERSECT_LAT | BSIGHT_INTSECT_LAT | s_grs_spatial_tab | near_spatial_db |
| 27 | #### BORESIGHT_INTERSECT_LON | BSIGHT_INTSECT_LON | s_grs_spatial_tab | near_spatial_db |
| 28 | #### BORESIGHT_INTERSECT_R | BSIGHT_INTSECT_R | s_grs_spatial_tab | near_spatial_db |
| 29 | #### SPICE_KERNEL_ID | SPICE_ID | s_grs_spatial_tab | near_spatial_db |
| 30 | #### PLATE_MODEL_ID | PLATE_MODEL_ID | s_grs_spatial_tab | near_spatial_db |
| 31 | #### PROCESSING_SOFTWARE_ID | SPATIAL_SOFTWARE_ID | s_grs_spatial_tab | near_spatial_db |
| 32 | #### DATABASE_VERSION_ID | DB_VERSION | s_grs_spatial_tab | near_spatial_db |

G.  Listing of Parameters for GRS Fractional Footprint Accounting System

| No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) Gamma-Ray Spectrometer (GRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 0 | #### MET | ACQ_MET_TIME | s_grs_overlap_tab | near_spatial_db |
| 1 | #### FOOTPRINT_BIN_TOTAL_AREA | TOT_AREA | s_grs_overlap_tab | near_spatial_db |
| 2 | #### FOOTPRINT_BIN_VISIBLE_AREA | TOT_VIS_AREA | s_grs_overlap_tab | near_spatial_db |
| 3 | #### TOTAL_EFF_SOLID_ANGLE_FRACTION | TOT_EFF_SOLID_ANGLE_FRACTION | s_grs_overlap_tab | near_spatial_db |
| 4 | #### NR_SPECTRUM_FRACTION | NR_SPECTRUM_FRACTION | s_grs_overlap_tab | near_spatial_db |
| 5 | #### NC_SPECTRUM_FRACTION | NC_SPECTRUM_FRACTION | s_grs_overlap_tab | near_spatial_db |
| 6 | #### BINID | BIN_ID | s_grs_overlap_tab | near_spatial_db |

NEAR GRS Requirements Specs Revision Notes

NEAR XGRS Team Meeting,  September 15, 1999,  (Version 7 Upgrades)

1. Pg.3     Removed 2 Eng_hk redundant parameters from record:

Formats 1 and 2:

Eng_HK changed from 154 to 152 total parameters.

Two parameters:

gray_burst_thresh_val:   Partition Offset (46,139) kept at 46

gray_burst_sci_rec_mode: positions (62,140) kept at 62

Related: Forced changes in E.9 title (pg.6) and list

(Pg.9), H.9 (title) pg.13

Modified SCIHK for new V7 totals

2. Pg.6     Added 6 SCI_HK parameters E.8 and labeled for

version 7 sofware upgrade.

3. Pg.9     Added 14 ENG_HK parameters E.9 and labeled for

version 7 software upgrade.

4. Pg.9     E.10 BADID and QUERYID is a total 5*(I*4) field in Title

5. Pg.12    Added elaboration of 6 BINID to section G.

(Fractional Footprint Accounting System)

6. Pg.13    Modified Live Time Eqns for NAI, BGO

7. Pg.21	Corrected Boresite intersection point for Lat-lon-R eqns

Spatial Partition Offsets (32,33,34) Resp.

-1. designates 180 degree rotation in longitude to match

Cornell from test data

8. Pg.15    Changed Bsite km to normal

9. Pg.29    Added elaboration of 6 BINID to section J. See item 5.

(Fractional Footprint Accounting System)

10.Pg.32    * Need to make eqn mods for Instrument_Angular_Response_Fct_0.3Mev,

20<phi<=40.  Modification is not made. (Appendix 2)

11.         Modified all occurences of NAL_ to NAI_

October 18, 1999  Following revision is suggested.

1.  Pg.3			FORMAT 1: Position of escape windows is added to the

description of valid Derived Eng. Record parameters.

2.  Pg.4			FORMAT 3: “summed” is added to the description of spectra.

3.  Pg.10	   E.10.:	Param.14-15: “NOT VALID IF NOT CALIBRATED” note is removed.

4.  Pg.13	H.9,title:	Number of parameters in Eng.HK is changed from 154 to 152

5.  Pg.13	   H.10.: 	In description of param.8-9 eng.param.names are adjusted.

6.  Pg.13-14   H.10.: 	Param.10-13 definition is modified in accordance with new

spectrum limits. Derivation algorithm is added.

7.  Pg.14      H.10.:	Param.14-15 (Pos_Esc_1,Pos_Esc_2): units are changed to keV

Derivation algorithm is added.

8.  Pg.14      H.10.: 	Param.16: “NAI_INTEGRAL(I)” is replaced by “NAI(I)”

9.  Pg.15	   H.11.: 	In param.3-8 description light time correction NEAR-Eros is

removed.

10. Pg.15	   H.11:	Param.9: Derivation algorithm is added.

11. Pg.15-21   H.11.: 	Param.11-13,15-25: Added comment “Fractions of the plates

are considered”.

12. Pg.21	   H.11.: 	Param.27: specifications for Boresight_Intersection_Lon

are changed.

13. Pg.23	    I.8.: 	Insignificant changes in description of param.7-10.

14. Pg.28-29      J.: 	Param.2-5: Added comment “Fractions of the plates are

considered”.

15. Pg.32      App.2:	Instrument_Angular_Response_Fct_0.3Mev, 20<phi<=40:

1.04087*10^(-1) is changed to 1.04087 (in accordance with

revision note 10 of September 15, 1999)

16. Pg.36			Revision notes are added

17. Pg.37			Pending issues are added

18. Pg.i			Table of contents is added

January 25, 2000.  Following revision is suggested.

- H10, param.12:  (Eng_HK:111)*3.4+3 is replaced with (Eng_HK:111)

Param.13:  (Eng_HK:110)*3.4+3 is replaced with (Eng_HK:110)

- H10, param.16:  integration is changed to energy range instead of channel range

- H11, param.14-15: discription of fractions added

- H11, param.26-27: “degrees” added

- J,   param.1-2:   description of fractions added

- Appendix 1.:     some symbols definitions are added

April 6-7, 2000.  Following revision is suggested.

1. E8, param.26-29:		description is clarified

2. E9, param.101-104,106,108,110,111:  description is clarified.

3. E11, H11, param.14:   	set to –1 all the time.

4. G, J, param.1:		 	set to –1 all the time.

5. H10, param.16:        	description is clarified.

- Appendix 3 is added

May 25, 2001  	Revisions

1. Renamed all Level-1 references to Level-2.

NEAR GRS Glossary and Acronym List

AL		X-ray Spectrometer: Aluminum Detector

ANTICO	Gamma-ray Spectrometer: Anti-Coincidence

APL		Applied Physics Laboratory

AVG		Average

BGO		Bismuth Germanate

CH		Channel

CMD		Command

CTP		Command and Telemetry Processing

DER		Derived Engineering

DPU		Data Processing Unit

DSN		Deep Space Network

DQI		Data Quality Index

ENG		Engineering

ESC		Escape

ET		Ephemeris Time

FCT		Function

FLG		Flag

FOV		Field of View

FTP		File Transfer Protocol

GCR		Galactic Cosmic Rays

GAS PC	Gas Proportional Counter Detector

GRS		Gamma Ray Spectrometer

GSFC		Goddard Space Flight Center

LT		Light Time

HDF		Hierarchical Data Format

HK		Space Craft or Instrument House Keeping Parameters

HVPS		High Voltage Power Supply

I*4		Signed Integer 4 byte parameter = 32 bits

JPL		Jet Propulsion Laboratory

keV		Kilo Electron Volts

LLD		Lower Level Discriminator

MET		Mission Elapsed Time

MeV		Mega Electron Volts

MG		X ray Spectrometer: Magnesium Detector

NAI		Sodium Iodide

NAIF		Nasa Ancillary Information Facility

NASA		National Aeronautics and Space Administration

NC		Neutron Capture Radiation

NR		Natural Radioactivity Radiation

NEAR		Near Earth Asteroid Rendezvous

POS		Position

PIN		X-ray Spectrometer:  Silicon PIN Detector

QueryID	Query IDentifier file served by Relational Database Management

System

R*4	Floating Point 4 byte Parameter = 32 bits

RDBMS	Relational Database Management System

RT	Rise Time

SC		Space Craft

SCI		Science

GRS Glossary and NEAR Acronym List Cont.

SDC		Applied Physics Laboratory Science Data Center

SPICE	Spacecraft – Planetary – Instrument – C-matrix – Ephemerides

TBD		To Be Decided

UA		University of Arizona

UA-LPL	University of Arizona – Lunar and Planetary Laboratory

UA-RDBMS	University of Arizona – Relational Database Management System

UNF		X-ray Spectrometer:  Unfiltered Detector

WWW		World Wide Web

XDR		eXternal Data Representation

XGRS		X-ray and Gamma-ray Spectrometer

XRS		X-ray Spectrometer