Requirement Specifications for Level-2 Parameters

Required Stored or Derived by

The Near Earth Asteroid Rendezvous (NEAR) X-Ray Spectrometer  (XRS) Ground System

Irina Mikheeva (UA) and Tim McClanahan (GSFC)

Revision 1.5:  May 25,  2001

Table of Contents

- Introduction and Purpose.....................................................................................................................1

- The UA-LPL Ground Processing Environment.................................................................................2

- Description of NEAR GRS Data File Standards................................................................................3

- Fractional Footprint Accounting.........................................................................................................5

- Listings of Parameters for XRS Level-2 Integral Records................................................................5

E.1-E.4.  NEAR XRS Spectra.................................................................................................................5

E.5.        NEAR XRS Science Housekeeping (SCI HK) Parameters......................................................6

E.6.        NEAR XRS Engineering Housekeeping (ENG HK) Parameters.............................................7

E.7.        NEAR XRS Derived Engineering (DER ENG) Parameters...................................................10

E.8.        NEAR XRS Spatial (SPATIAL) Parameters..........................................................................11

E.9.        NEAR XRS Solar (SOLAR) Parameters................................................................................12

- Listings of Parameters for XRS Level-2 Summary Record.............................................................12

F.1-F.8.  NEAR XRS Summarized Spectra...........................................................................................12

F.9.        NEAR XRS Summarized Engineering (ENGINEERING) Parameters...................................13

F.10.      NEAR XRS Summarized Spatial (SPATIAL) Parameters......................................................14

F.11.      NEAR XRS Summarized Solar (SOLAR) Parameters............................................................14

- Listings of Parameters for XRS Fractional Footprint Accounting System....................................14

- Numerical Specifications for XRS Level-2 Integral Records...........................................................15

H.1-H.4. NEAR XRS Spectra................................................................................................................15

H.5.        NEAR XRS Science Housekeeping (SCI HK) Parameters.....................................................15

H.6.        NEAR XRS Engineering Housekeeping (ENG HK) Parameters............................................15

H.7.        NEAR XRS Derived Engineering (DER ENG) Parameters....................................................15

H.8.        NEAR XRS Spatial (SPATIAL) Parameters...........................................................................17

H.9.        NEAR XRS Solar (SOLAR) Parameters.................................................................................23

- Numerical Specifications for XRS Level-2 Summary Record..........................................................25

I.1-I.8.    NEAR XRS Summarized Spectra.......................................................................................… 25

I.9.          NEAR XRS Summarized Engineering (ENGINEERING) Parameters..................................28

I.10.        NEAR XRS Summarized Spatial (SPATIAL) Parameters.....................................................31

I.11.        NEAR XRS Summarized Solar (SOLAR) Parameters...........................................................36

- Numerical Specifications for XRS Fractional Footprint Accounting System.................................36

APPENDIX 1.    Definitions for:   Collimator_Fct;    Spatial_Fluorescence_Norm;

Angular_Fluor_Fct;   Total_Weight_Fluor;   Spatial_Scattering_Norm;

Angular_Scatter_Fct;   Total_Weight_Scatt.....................................................................39

APPENDIX 2.    Approximations for:   Collimator_Fct;    Angular_Fluor_Fct;

Angular_Scatter_Fct;   FOV_Status..................................................................................43

APPENDIX 3    Location of XRS Level-2 Parameters in the U of A Database...........................................45

NEAR XRS Requirements Specs Revision Notes.................................................................................….57

NEAR XRS Glossary and Acronym List……………..........................................................................….59

# Introduction and Purpose

The following document will act as one part of a two part requirement specification for all Level-2 parameters required to be stored, or derived by the Near Earth Asteroid Rendezvous (NEAR) X-ray and Gamma-Ray spectrometer ground systems (XGRS). It will relate specifically to the X-Ray spectrometer (XRS) requirements. The document will act as a primary common reference for both NEAR X-ray and Gamma-ray spectrometer (XGRS) science team members, systems development staff and future users of these data.  The goal is to assist in the development and understanding of parameters defined as key to the Level-2 NEAR XGRS science objectives.

A primary data server residing at the University of Arizona, Lunar and Planetary Laboratory (UA-LPL) is designated with the function of deriving and serving these mission critical parameters during cruise and encounter phases of the NEAR mission scheduled from February 1996 through February 2001.   The intent of the first level product is to include all NEAR XRS mission critical information needed for asteroid mapping, health of instrument, control, and science analysis systems.

Level -2 products will include integral spectral data in either raw or instrument calibrated format.  Bundled with the spectral data products will be scientific housekeeping and engineering housekeeping data sampled by the XGRS instrumentation at the same time as the integral spectra onboard the spacecraft.  This information is then telemetered through NASA’s Deep Network (DSN) managed by the Jet Propulsion Laboratory in Pasadena, California, and NEAR related telemetry is forwarded to the Applied Physics Laboratory (APL).  The Level-0 XGRS raw spectral and engineering data is then broken out of the NEAR mission specific data stream and stored online at the APL Science Data Center (SDC).  These data are stored in the Hierarchical Data Format (HDF) file standard.   Theses files are then picked up by the automated (UA-LPL) data processing ingest system.

Figure 1.0   NEAR XGRS OPERATIONAL FLOW OF INFORMATION

NEAR

SPACECRAFT

MAX PLANCK

INSTITUTE  FOR CHEMISTRY

DEEP SPACE NETWORK (DSN)

JET PROPULSION LABORATORY

NEAR Telemetry

Flow   											           MAX PLANCK

APL		    Meteorite Libraries

APPLIED  PHYSICS LABORATORY

Serve HDF         Science Analysis

Serve SPICE

Validate SC MACROS

UA-LPL

NEAR XGRS Data Server

NASA GODDARD SPACE  FLIGHT CENTER

UNIVERSITY OF ARIZONA LUNAR AND PLANETARY

LABORATORY

Level 1  Processing

Science Analysis

Raw Spectral Data Products

Calibration and Quicklook Info

GSFC

Instrument MACROS

CORNELL UNIVERSITY						                     XGRS health of mission

CORNELL

UNIVERSITY

Coverage Mapping						         Spacecraft Pointing

Instrument Pointing Plan					         Science Analysis

- The UA-LPL Ground Processing Environment

The core of the UA-LPL data processing system (Figure 2.0) consists of a Sybase Relational Database (UA-RDBMS) that stores tables of NEAR XGRS raw spectral, science housekeeping, engineering housekeeping, derived engineering and spatial data products.   XRS and GRS data processing systems are logically partitioned into separate processing environments on the UA-LPL server.    A scientific data processing system has been developed around the UA-RDBMS to provide automated data ingest and science team interaction. Critical spatial and derived engineering data products are generated either at ingest time or query time.  Core query time processing options include calibration and summation processing, compression and bundling of query results for return to the scientific user community over the Internet file transfer protocol (FTP).

Figure 2.0   NEAR XGRS Ground Processing System at the UA-LPL

Ingest  Proc

Spatial Process

Calibration

User Comm

Bundling

Summation

WWW Interface

UA-LPL

RDBMS

NASA Goddard

Calibrate

Quicklook

Inst. Health

UA-LPL  database ingest systems automatically monitor the APL SDC for new and updated postings of XGRS data sets.  These file sets are recovered via the file transfer protocol (FTP) to the UA-LPL data system.  Raw spectral and engineering data products are then loaded into XGRS RDBMS tables with Mission Elapsed Time (MET) acting as the primary key for database access.  Mission Elapsed Time is used with spacecraft ephemeris data (SPICE system, APL Science Data Center and JPL/NASA Ancillary Information Facility) to derive spatial data products.   Spacecraft navigation and pointing information is converted to SPICE data for the NEAR XGRS ground system and will be generated by participating scientists at APL/SDC. Software systems have been developed at the UA-LPL and GSFC to generate both spatial and derived engineering products. These products will then be maintained within the UA-RDBMS database system.

At Goddard Space Flight Center participating scientists will be charged with monitoring instrument health, calibration, and bad flagging systems.  A feedback system is employed to move derived information to on-line ingest processes at the UA-RDBMS.  The calibration system will be used to convert spectral data products to a common energy standard specified at query time.   Calibration processing is optionally selected at query time. Bad flagging information is generated during instrument health checks as a function of identification of problem (MET) record sets.  These (MET)’s are forwarded to the UA-LPL RDBMS for tagging.

Query time summation processes will be used to sum multple integral record sets into a single output record. This record will include one spectrum per spectral classification and selected engineering products.  Summed output will generally be specific to regional coverage and science analysis.  The primary motivation for summation processing is to limit large volume network transfers incurred in this distributed processing environment.  Results are then accumulated in the Xternal Data Representation (XDR) binary file standard (Sun Microsystems) then compressed, bundled and forwarded to the query initiator.

C. Description of NEAR XRS Data File Standards

In the NEAR XRS data processing system there are 3 data file standards possible for output.   These different file standards reflect the different UA-LPL ground processes selectable at query time. Output format type is selected at query time through the UA-LPL WWW query interface.    Integral raw XRS products will be used primarily in the calibration, instrument health and mission planning activities.  Calibrated data will be used primarily in the science analysis activity.   Formats for the three record standards are as follows.

Figure 3.0

FORMAT 1:    XRS Integral Raw:

UNF	MG	AL       SOLAR_SP   SCI  HK	ENG HK      DER ENG    SPATIAL     SOLAR        SPARE

Description:

- UNF 		256 R*4		Unfiltered raw spectrum					E.1., H.1.

- MG 		256 R*4		Magnesium raw spectrum 					E.2., H.2.

- AL   		256 R*4		Aluminum raw spectrum					E.3., H.3.

- SOLAR_SP	256 R*4		Solar raw spectrum:   GAS PC or PIN			E.4., H.4.

- SCI HK		  53  I*4		Science Housekeeping sampled at integration time		E.5., H.5.

- ENG HK		163 R*4		Engineering Housekeeping sampled at integration time	E.6., H.6.

- DER ENG*	  28 R*4		Derived Engineering:  only live times, tailing		E.7., H.7.

and real gain zero** estimates are valid ELSE Default value

5   I*4		1 longword for BadFlag and 4 longwords for QueryID

- SPATIAL	   1   I*4		Mission Elapsed Time (MET) in Spatial Section 		E.8., H.8.

38 R*4		Spatial derivations, positional and pointing information

- SOLAR		   2  R*4		Solar level and Solar Integral				E.9., H.9.

- SPARE		 25  R*4		Spare Record (TBD)

*     DER ENG total number of values may change if calibration system is not implemented.

**   There is no gain and zero tracking currently

*** TBD is To Be Decided

FORMAT 2:     XRS Integral Calibrated:

UNF	 MG	AL     SOLAR_SP      SCI  HK      ENG HK       DER ENG     SPATIAL      SOLAR         SPARE

Description:

- UNF 		256 R*4		Unfiltered Calibrated spectrum				E.1., H.1.

- MG 		256 R*4		Magnesium Calibrated spectrum				E.2., H.2.

- AL   		256 R*4		Aluminum Calibrated spectrum				E.3., H.3.

- SOLAR_SP	256 R*4		Solar raw OR Calibrated (TBD) spectrum: PC or PIN		E.4., H.4.

- SCI HK		  53  I*4		Science Housekeeping sampled at integration time		E.5., H.5.

- ENG HK		163 R*4		Engineering Housekeeping sampled at integration time	E.6., H.6.

- DER ENG*	  28 R*4		Derived Engineering:  All information is valid		E.7., H.7.

5  I*4		1 longword for BadFlag and 4 longwords for QueryID

- SPATIAL  	    1  I*4		Mission Elapsed Time (MET) in Spatial Section 		E.8., H.8.

38 R*4		Spatial derivations, positional and pointing information

- SOLAR		    2 R*4		Solar level and Solar Integral				E.9., H.9.

- SPARE		  25 R*4		Spare Record  (TBD)

* DER ENG total number of values may change if calibration system is not implemented.

Nomenclature for XRS integral data products:  FORMATS 1 and 2  (Calibrated and Raw)

Nomenclature for XRS integral data products:  FORMATS 1 and 2  (Calibrated and Raw)

x(yyyy)(ddd)(hh)(mm)(ss).xdr                          Example:  x1998234010159.xdr

There currently is no planned calibration processing for the integral XRS data products.  This may have an impact on the total number of parameters in the derived engineering sections of the XRS integral records.

NEAR XRS Summation Processing:  (Format 3)

The NEAR XRS data processing ground system is a distributed system that currently spans institutions in the United States and Europe.  The summation processing system has been specified to render potentially very large volumes of data (> Gbytes) returned from a single query down to a single summary record. The intention of summation processing will be to minimize the per query network data transfer size (bytes), by rendering a single summed spectrum from a relationally defined set.  Correlated parameters will be bundled with this summed set.   The summation ground processing step will be supported only on calibrated spectral data products.

Both Solar detector types (GAS PC and PIN) will be returned by the summation query and selected parameters will be summarized in the Format 3 product standard.

FORMAT 3:      XRS Summed:

4  SPECTRA, GAS_PC                  4  SPECTRA, PIN

UNF    MG    AL    GAS_PC         UNF    MG    AL    PIN       ENGINEERING          SPATIAL           SOLAR

Description:

4 SPECTRA, GAS_PC

- UNF	 	256 R*4		Unfiltered Calibrated  Summed, GAS PC		F.1., I.1.

- MG		256 R*4		Magnesium Calibrated  Summed, GAS PC		F.2., I.2.

- AL	   	256 R*4		Aluminum Calibrated  Summed, GAS PC		F.3., I.3.

- GAS_PC  	256 R*4		Summed GAS PC				F.4., I.4.

4 SPECTRA, PIN

- UNF	 	256 R*4		Unfiltered Calibrated  Summed, PIN		F.5., I.5.

- MG 		256 R*4		Magnesium Calibrated  Summed, PIN		F.6., I.6.

- AL 	  	256 R*4		Aluminum Calibrated  Summed, PIN		F.7., I.7.

- PIN		256 R*4		Summed PIN					F.8., I.8.

- ENGINEERING	  48 R*4		Averaged Engineering  and Derived Eng		F.9., I.9.

- SPATIAL	  17 R*4		Averaged Spatial Parameters			F.10., I.10.

- SOLAR		   4  R*4		Solar statistics					F.11., I.11.

Nomenclature for XRS integral data products:  FORMATS 3  (Summed)

y(yyyy)(ddd)(hh)(mm)(ss).xdr                          Example:  y1998234010159.xdr

Summation processing will be query specific and all derived engineering tracking is done internal to each query process.  All (XDR) file sets returned from the query will be run through the summation processing system. Integral calibrated spectra will be summed into single XRS specific output sets.  Selected engineering and spatial parameters are also averaged into values representative of the parameter over the range of output (MET) records and specific to each query.  Parameter rendering algorithms are listed in the numerical specifications (Section I).

# Fractional Footprint Accounting

The 433Eros asteroid is highly irregular in shape and deviates significantly from a spherical geometry.  A system was developed by NEAR XGRS staff to partition the virtual 433Eros asteroid into surface regions (bins) of roughly equal surface area, not based on spherical geometry.  The bin system is internal to the UA-RDBMS ground processing system.

Individual spectral accumulations will most likely span multiple asteroid surface bins.   This system will be used to discriminate and identify the applicability of integral footprints, based on their fractional coverage of the asteroid surface bin being queried. During UA-RDBMS ingest processing each integral footprint will be broken down into a set of records, one bin record for each bin subtended by the footprint.  The current plan does not include the possibility of using the fractional footprint as a weighting function when automatically summing spectral data at query time.   The fractional footprint accounting will be used as a query time threshold criteria for inclusion of the integral MET to summation processing.

##### Figure 3.0

FOOTPRINT 	      BIN       MET  = Single Integration Mission Elapsed Time (MET)

Fractional Footprint Accounting (MET in Spatial Bins)

A                 B

C                 D

BIN  A		BIN  B 	BIN  C		BIN  D

#### MET  0.2	MET  0.1	MET  0.4	MET 0.3

1 Integral Footprint  at MET

Fraction totals = 1.0

4 total records generated Asteroid Spatial Bins A,B,C,D

# A query time specification requesting all integrations (MET records) from Bin C with a fractional footprint area

>= 0.4  would return the example integration and others whose fractional coverage are at least 0.4.

Information related to the bin processing is included in sections G and J.

#### Listings of Parameters for XRS Level–2 Integral Records

FORMATS  1,2 - List of RDBMS parameters for Single Raw and Calibrated XRS Level-2 record (output)

###### E.1. – E.4.   NEAR XRS Spectra:  4 spectra*256 channels*(Real*4)

0 – 255  Unfiltered spectrum (UNF)

0 – 255  Mg filtered spectrum (MG)

0 – 255  Al filtered spectrum   (AL)

0 – 255  Solar spectrum (proportional counter GAS_PC or PIN)  (SOLAR_SP)

## E.5.   NEAR XRS Science Housekeeping (SCI HK) Parameters:  53*(Integer*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.

0    MET (Mission Elapsed Time) Primary Key to RDBMS

1    EPHEMERIS_TIME

2    DQI  			           - Data Quality Indicator

3    TIME_TAG

4    UNF_MON_INTEG_TIME

5    MG_MON_INTEG_TIME

6    AL_MON_INTEG_TIME

7    ACTV_SOL_MON_INTEG_TIME

8    UNF_XRAY_1_SAFE_FLG

9    MG_XRAY_1_SAFE_FLG

10  AL_XRAY_1_SAFE_FLG

11  GAS_XRAY_1_SAFE_FLG

12  PIN_XRAY_1_SAFE_FLG

13  UNF_XRAY_2_SAFE_FLG

14  MG_XRAY_2_SAFE_FLG

15  AL_XRAY_2_SAFE_FLG

16  GAS_XRAY_2_SAFE_FLG

17  PIN_XRAY_2_SAFE_FLG

18  UNF_XRAY_OVERFLOW_FLG

19  MG_XRAY_OVERFLOW_FLG

20  AL_XRAY_OVERFLOW_FLG

21  ACTIVE_XRAY_OVERFLOW_FLG

22  UNF_XRAY_RAW_EVENT_RATE

23  MG_XRAY_RAW_EVENT_RATE

24  AL_XRAY_RAW_EVENT_RATE

25  GAS_XRAY_RAW_EVENT_RATE

26  PIN_XRAY_RAW_EVENT_RATE

27  UNF_XRAY_VALID_EVENTS

28  MG_XRAY_VALID_EVENTS

29  AL_XRAY_VALID_EVENTS

30  GAS_XRAY_VALID_EVENTS 						Version 7

31  UNF_XRAY_EVENTS_REJ_BY_RT

32  MG_XRAY_EVENTS_REJ_BY_RT

33  AL_XRAY_EVENTS_REJ_BY_RT

34  GAS_XRAY_EVENTS_REJ_BY_RT

35  PIN_SOLAR_PROCESS_EVENT 						Version 7

36  XTALK_COUNTER 								Version 7

37  CMD_OPCODE 									Version 7

38  CMD_ARGUMENT 								Version 7

39  CMDS_RECEIVED 								Version 7

40  PROBLEM_COUNTER 								Version 7

41  PROBLEM_CODE 								Version 7

42  UNF_XRAY_DC_OFFSET					Downlinked value.  No conversion to energy

bins is performed. ( no devision by 8 as required by NEAR XFRS DPU Software Requirements Specifications JHU/APL 7358-9002 Version 7)

43  MG_XRAY_DC_OFFSET							- “ -

44  AL_XRAY_DC_OFFSET							- “ -

45  GAS_XRAY_DC_OFFSET							- “ -

46  PIN_DC_OFFSET								- “ -

47  UNF_STAND_DEV								- “ -

48  MG_STAND_DEV								- “ -

49  AL_STAND_DEV								- “ -

50  GAS_STAND_DEV								- “ -

51  PIN_STAND_DEV								- “ -

52  XRAY_DATA_QUALITY_BIT 							Version 7

## E.6. NEAR XRS Engineering Housekeeping (ENG HK) Parameters: 163*(REAL*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.   The Gamma –Ray Spectrometer ENG_HK paramaters (0:137) are identical to the XRS, but sampled at the GRS sample rate.

- DETECTOR_ELECTR_CURR

- XRAY_PIN_DIODE_FET_BIAS

- HVCE_TEMPERATURE

- UNF_XRAY_HVPS_VOLT

- MG_XRAY_HVPS_VOLT

- AL_XRAY_HVPS_VOLT

- XRAY_GAS_HVPS_VOLT

- XRAY_PIN_HVPS_VOLT

- NAI_HVPS_VOLT

- BGO_HVPS_VOLT

- DPU_DC_DC_TEMP

- XGRS_DC_DC_TEMP

- UNF_XRAY_HVPS_TEMP

- MG_XRAY_HVPS_TEMP

- AL_XRAY_HVPS_TEMP

- XRAY_GAS_HVPS_TEMP

- XRAY_PIN_HVPS_TEMP

- NAI_HVPS_TEMP

- BGO_HVPS_TEMP

- TEC_CURRENT

- NAI_HVPS_VOLT_CMD

- BGO_HVPS_VOLT_CMD

- UNF_HVPS_VOLT_CMD

- MG_HVPS_VOLT_CMD

- AL_HVPS_VOLT_CMD

- HVPS_POWER_ON_OFF

- XRAY_CALIB_HOME_FLG

- XRAY_CALIB_MOTOR_ON_OFF

- XRAY_CALIB_MOTOR_ENABLE

- GAS_HVPS_VOLT_CMD

- PIN_HVPS_VOLT_CMD

- GAMMA_RAY_HEATER_CURR

- Spare0

- XRAY_CALIB_MOTOR_DIR

- XRAY_CALIB_MOTOR_CURR

- XRAY_CALIB_MOTOR_GOAL

- XRAY_CALIB_MOTOR_POS

- XRAY_CALIB_MOTOR_FID_SENS

- XRAY_CALIB_MOTOR_FID_BRIT

- XRAY_MOTOR_IN_CALIB_POSITION

- XRAY_MOTOR_IN_NORM_POSITION

- ACTIVE_SOLAR_TOGGLE_MODE

- XRAY_CALIB_MAX_STEPS

- Spare1

- TEC_ENABLE

- XRAY_CALIB_CUM_MOTOR_STPS

- GRAY_BURST_THRESH_VAL

- XRAY_SAFING_THRESH

- XRAY_CALIB_MOTOR_DIAG

- TEC_POWER_ON_OFF

- TEC_MODE

- TEC_COOL_HEAT_FLG

- TEC_TEMPERATURE

- TEC_COOL_MODE_TEMP_UP_LMT

- TEC_COOL_MODE_TEMP_LOW_LMT

- TEC_HEAT_MODE_TEMP_UP_LMT

- TEC_HEAT_MODE_TEMP_LOW_LMT

- GAMMA_RAY_TEMP_HYSTERESIS

- GAMMA_RAY_HEAT_TEMP_LOW_LMT

- FULL_GAMMA_RAY_SCIENCE_MODE

- FULL_XRAY_SCIENCE_MODE

- SUMM_GRAY_XRAY_SCIENCE_MODE

- GRAY_BURST_SCI_REC_MODE

- HVCE_TEMP_SET_POINT

- CMDED_END_XRAY_INTEG_PER

- CMDED_GAMMA_RAY_INTEG_PER

- BGO_PROTON_FLARE_FLG

- GAMMA_RAY_BIN_OVER_FLG_CMD

- UNF_XRAY_BIN_OVER_FLG_CMD

- MG_BIN_OVER_FLG_CMD

- AL_BIN_OVER_FLG_CMD

- ACT_BIN_OVER_FLG_CMD

- NAI_BIN_OVERFLOW_FLG

- BGO_BIN_OVERFLOW_FLG

- NAI_ANTICOIN_OVERFLOW_FLG

- NAI_SING_ESC_OVERFLOW_FLG

- NAI_DOUB_ESC_OVERFLOW_FLG

- BGO_SING_ESC_OVERFLOW_FLG

- BGO_DOUB_ESC_OVERFLOW_FLG

- UNF_XRAY_BIN_OVERFLOW_FLG

- MG_BIN_OVERFLOW_FLG

- AL_BIN_OVERFLOW_FLG

- ACT_BIN_OVERFLOW_FLG

- GAMMA_RAY_HEAT_ON_OFF

- GAMMA_RAY_HEAT_MODE

- BGO_LEVEL_1_SAFING_FLG

- BGO_LEVEL_2_SAFING_FLG

- Spare3

- XRAY_INTEGR_TIME

- HVPS_ON_OFF

- PIN_SENSOR_MASK_FLG

- GAS_SENSOR_MASK_FLG

- AL_SENSOR_MASK_FLG

- MG_SENSOR_MASK_FLG

- UNF_SENSOR_MASK_FLG

- BGO_SENSOR_MASK_FLG

- NAI_SENSOR_MASK_FLG

- UNF_XRAY_RISE_TIME_THRESH

- MG_RISE_TIME_THRESH

- AL_RISE_TIME_THRESH

- GAS_RISE_TIME_THRESH

- UNF_RISE_TIME_VALID_THRESH 			Downlinked value. No ground conversion

specified in NEAR XFRS DPU Software Requirements Specifications JHU/APL 7358-9002 Version 7 is performed.

- MG_RISE_TIME_VALID_THRESH  				- “ -

- AL_RISE_TIME_VALID_THRESH  				- “ -

- GAS_RISE_TIME_VALID_THRESH  			- “ -

- GAMMA_RAY_INTEGR_TIME

- GAMMA_SING_ESC_WIND_CENTER			- “ -

- GAMMA_SING_ESC_WIND_WIDTH

- GAMMA_DOUB_ESC_WIND_CENTER			- “ -

- GAMMA_DOUB_ESC_WIND_WIDTH

- NAI_LOW_LEVEL_AMPL_THRESH 	  	Ground conversion is: (downlinked value * 3.413 +

3.0), which is different from specified in NEAR XFRS DPU Software Requirements Specifications JHU/APL 7358-9002 Version 7

- BGO_LOW_LEVEL_AMPL_THRESH  			- “ -

- UNF_LOW_LEVEL_AMPL_THRESH

- MG_LOW_LEVEL_AMPL_THRESH

- AL_LOW_LEVEL_AMPL_THRESH

- GAS_LOW_LEVEL_AMPL_THRESH

- PIN_LOW_LEVEL_AMPL_THRESH

- GAMMA_RAY_SENSOR_TEMP

- NAI_LEVEL_1_SAFING_FLG

- UNF_LEVEL_1_SAFING_FLG

- MG_LEVEL_1_SAFING_FLG

- AL_LEVEL_1_SAFING_FLG

- GAS_LEVEL_1_SAFING_FLG

- PIN_LEVEL_1_SAFING_FLG

- NAI_LEVEL_2_SAFING_FLG

- UNF_LEVEL_2_SAFING_FLG

- MG_LEVEL_2_SAFING_FLG

- AL_LEVEL_2_SAFING_FLG

- GAS_LEVEL_2_SAFING_FLG

- PIN_LEVEL_2_SAFING_FLG

- XRAY_SENS_SAFING_REST_LEVEL

- GAMMA_SENS_SAFING_REST_LEVEL

- XRAY_SAFING_REST_MAX_RETRY

- GAMMA_SAFING_REST_MAX_RETRY

- GAMMA_HEATER_DUTY_CYCLE

- GAMMA_SENS_SAFING_THRESH

- HVPS_CURRENT

- BGO_PROTON_FLARE_THRESH

- VERSION_NUMBER 								Version 7

- CLEAR_SPECTRUM_0 							Version 7

- CLEAR_HW_RAW 								Version 7

- CLEAR_HW_RISE_TIME_REJECT						Version 7

- CLEAR_FALSE_RISE_TIME_REJECT 					Version 7

- CLEAR_TRUE_RISE_TIME_REJECT 					Version 7

- CLEAR_HIGH_ENERGY 							Version 7

- MG_SPECTRUM_0 								Version 7

- MG_HW_RAW 									Version 7

- MG_HW_RISE_TIME_REJECT 						Version 7

- MG_FALSE_RISE_TIME_REJECT 						Version 7

- MG_TRUE_RISE_TIME_REJECT 						Version 7

- MG_HIGH_ENERGY 								Version 7

- AL_SPECTRUM_0 								Version 7

- AL_HW_RAW 									Version 7

- AL_HW_RISE_TIME_REJECT 						Version 7

- AL_FALSE_RISE_TIME_REJECT 						Version 7

- AL_TRUE_RISE_TIME_REJECT 						Version 7

- AL_HIGH_ENERGY 								Version 7

- SOLAR_SPECTRUM_0 							Version 7

- SOLAR_HW_RAW 								Version 7

- SOLAR_HW_RISE_TIME_REJECT 						Version 7

- SOLAR_FALSE_RISE_TIME_REJECT 					Version 7

- SOLAR_TRUE_RISE_TIME_REJECT 					Version 7

- SOLAR_HIGH_ENERGY 							Version 7

### E.7. NEAR XRS Derived Engineering (DER ENG) Parameters:  28*(Real*4), 1*(Integer*4), 4*(Integer*4)

These parameters are not stored in the University of Arizona  RDBMS but derived at a query time.

# “Not valid” – is designated as = -1

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- UNF_LIVE_TIME

- MG_LIVE_TIME

- AL_LIVE_TIME

- SOLAR_LIVE_TIME

- UNF_VALID_CHANNEL_HI

17   MG_VALID_CHANNEL_HI

18   AL_VALID_CHANNEL_HI

19   UNF_VALID_CHANNEL_LOW

20   MG_VALID_CHANNEL_LOW

21   AL_VALID_CHANNEL_LOW

22   UNF_RISE_TIME_CHANNEL

23   MG_RISE_TIME_CHANNEL

24   AL_RISE_TIME_CHANNEL

25   Spare

26   Spare

27   Spare

28   BAD_FLAG

29-32   QUERY_ID

E.8.  NEAR XRS Spatial (SPATIAL) Parameters:  1*(Integer*4), 38*(Real*4)

These parameters are derived in the University of Arizona at the ingest phase.

“Not valid” – is designated as = -1

0    MET

- PLATEID_BORESIGHT_INTERSECT				NOT VALID IF FOV_STATUS=0

- BINID_BORESIGHT_INTERSECT				NOT VALID IF FOV_STATUS=0

- SC_POSITION, X

- SC_POSITION, Y

- SC_POSITION, Z

- BS_VECTOR, X

- BS_VECTOR, Y

- BS_VECTOR, Z

- EARTH_POSITION, X

- EARTH_POSITION, Y

- EARTH_POSITION, Z

- SUN_POSITION, X

- SUN_POSITION, Y

- SUN_POSITION, Z

- FOV_STATUS

- DOWNLINK_STATUS

- ANGLE_SUN_SOLAR_MON

- AVG_SC_DISTANCE						NOT VALID IF FOV_STATUS=0

- SUN_DISTANCE

- AVG_EMISSION_ANGLE					NOT VALID IF FOV_STATUS=0

- AVG_INCIDENCE_ANGLE					NOT VALID IF FOV_STATUS=0,2,4

- PHASE_ANGLE                                                                        NOT VALID IF FOV_STATUS=0,2,4

- AVG_INC_EMI_COS_RATIO					NOT VALID IF FOV_STATUS=0,2,4

- SPATIAL_FLUORESCENCE_NORM				=0 IF FOV_STATUS=0,2,4

- SPATIAL_SCATTERING_NORM				=0 IF FOV_STATUS=0,2,4

- TOTAL_AREA_FOOTPRINT					NOT VALID

- TOTAL_ILLUMINATED_AREA				=0 IF FOV_STATUS=0,2,4

- TOTAL_VISIBLE_AREA					=0 IF FOV_STATUS=0

- TOTAL_EFF_AREA						NOT VALID

- TOTAL_EFF_SOLID_ANGLE					=0 IF FOV_STATUS=0

- FOOTPRINT_SOLID_ANGLE					=0 IF FOV_STATUS=0

- BORESIGHT_INTERSECTION_LAT 				NOT VALID IF FOV_STATUS=0

- BORESIGHT_INTERSECTION_LON 				NOT VALID IF FOV_STATUS=0

- BORESIGHT_INTERSECTION_R 				NOT VALID IF FOV_STATUS=0

- SPICE_KERNEL_ID

- PLATE_MODEL_ID

- PROCESSING_SOFTWARE_ID

- DATABASE_VERSION_ID

### E.9.  NEAR XRS Solar (SOLAR) Parameters: 2*(Real*4)

- SOLAR_INTEGRAL

- SOLAR_LEVEL

#### F. Listings of Parameters for XRS Level–2 Summary Record

FORMAT  3 - List of RDBMS parameters for Summarized XRS Level-2 record (output)

###### F.1. – F.8.   NEAR XRS Summarized Spectra:  8 spectra*256 channels*(Real*4)

0 – 255  Unfiltered Calibrated* Summed spectrum with proportional counter GAS_PC active   (UNF, GAS_PC)

0 – 255  Mg filtered Calibrated* Summed spectrum with proportional counter GAS_PC active (Mg, GAS_PC)

0 – 255  Al filtered Calibrated* Summed spectrum with proportional counter GAS_PC active   (Al, GAS_PC)

0 – 255  GAS_PC Calibrated* Summed spectrum						        (GAS_PC)

0 – 255  Unfiltered Calibrated* Summed spectrum with proportional counter PIN active	        (UNF, PIN)

0 – 255  Mg filtered Calibrated* Summed spectrum with proportional counter PIN active          (MG, PIN)

0 – 255  Al filtered Calibrated* Summed spectrum with PIN active                                              (AL, PIN)

0 – 255  PIN Calibrated* Summed spectrum                                                                                  (PIN)

* Calibration system is not implemented currently

F.9.  NEAR XRS Summarized Engineering (ENGINEERING) Parameters:  48*(Real*4)

- Spare

- Spare

- Spare

- Spare

- Spare

5    Spare

6    TOTAL_INTEG_TIME_UNF_GAS

7    TOTAL_INTEG_TIME_MG_GAS

8    TOTAL_INTEG_TIME_AL_GAS

9    TOTAL_INTEG_TIME_GAS

10   TOTAL_LIVE_TIME_UNF_GAS

11   TOTAL_LIVE_TIME_MG_GAS

12   TOTAL_LIVE_TIME_AL_GAS

13   TOTAL_LIVE_TIME_GAS

14   UNF_VALID_CHANNEL_HI_GAS

15   MG_VALID_CHANNEL_HI_GAS

16   AL_VALID_CHANNEL_HI_GAS

17   UNF_VALID_CHANNEL_LOW_GAS

18   MG_VALID_CHANNEL_LOW_GAS

19   AL_VALID_CHANNEL_LOW_GAS

20   UNF_INVALID_CHANNEL_HI_GAS

21   MG_INVALID_CHANNEL_HI_GAS

22   AL_INVALID_CHANNEL_HI_GAS

23   UNF_INVALID_CHANNEL_LOW_GAS

24   MG_INVALID_CHANNEL_LOW_GAS

25   AL_INVALID_CHANNEL_LOW_GAS

26   NUMBER_MET_GAS

27   TOTAL_INTEG_TIME_UNF_PIN

28   TOTAL_INTEG_TIME_MG_PIN

29   TOTAL_INTEG_TIME_AL_PIN

30   TOTAL_INTEG_TIME_PIN

31   TOTAL_LIVE_TIME_UNF_PIN

- TOTAL_LIVE_TIME_MG_PIN

- TOTAL_LIVE_TIME_AL_PIN

- TOTAL_LIVE_TIME_PIN

- UNF_VALID_CHANNEL_HI_PIN

36   MG_VALID_CHANNEL_HI_PIN

37   AL_VALID_CHANNEL_HI_PIN

38   UNF_VALID_CHANNEL_LOW_PIN

39   MG_VALID_CHANNEL_LOW_PIN

40   AL_VALID_CHANNEL_LOW_PIN

41   UNF_INVALID_CHANNEL_HI_PIN

42   MG_INVALID_CHANNEL_HI_PIN

43   AL_INVALID_CHANNEL_HI_PIN

44   UNF_INVALID_CHANNEL_LOW_PIN

45   MG_INVALID_CHANNEL_LOW_PIN

46   AL_INVALID_CHANNEL_LOW_PIN

47   NUMBER_MET_PIN

F.10.   NEAR XRS Summarized Spatial (SPATIAL) Parameters:  17*(Real*4)

- AVG_SC_DISTANCE_GAS 			NOT VALID IF FOV_STATUS=0

- AVG_EMI_ANGLE_GAS 			NOT VALID IF FOV_STATUS=0

- AVG_INC_ANGLE_GAS 			NOT VALID IF FOV_STATUS=0,2,4

- AVG_PHASE_GAS 		     	NOT VALID IF FOV_STATUS=0,2,4

- AVG_INC_EMI_COS_RATIO_GAS 		NOT VALID IF FOV_STATUS=0,2,4

- AVG_EFF_SOLID_ANGLE_GAS		=0 IF FOV_STATUS=0

- TOTAL_WEIGHT_FLUOR_GAS		=0 IF FOV_STATUS=0,2,4

- TOTAL_WEIGHT_SCAT_GAS			=0 IF FOV_STATUS=0,2,4

- AVG_SC_DISTANCE_PIN 			NOT VALID IF FOV_STATUS=0

- AVG_EMI_ANGLE_PIN 			NOT VALID IF FOV_STATUS=0

- AVG_INC_ANGLE_PIN 			NOT VALID IF FOV_STATUS=0,2,4

- AVG_PHASE_PIN           			NOT VALID IF FOV_STATUS=0,2,4

- AVG_INC_EMI_COS_RATIO_PIN 		NOT VALID IF FOV_STATUS=0,2,4

- AVG_EFF_SOLID_ANGLE_PIN		=0 IF FOV_STATUS=0

- TOTAL_WEIGHT_FLUOR_PIN		=0 IF FOV_STATUS=0,2,4

- TOTAL_WEIGHT_SCAT_PIN			=0 IF FOV_STATUS=0,2,4

- ESTIMATED_COVERED_AREA		=0 IF FOV_STATUS=0

F.11.   NEAR XRS Summarized Solar (SOLAR) Parameters:  4*(Real*4)

1   AVERAGE_SOLAR_INTEGRAL_GAS

2   SOLAR_INTEGRAL_VARIANCE_GAS

3   AVERAGE_SOLAR_INTEGRAL_PIN

4   SOLAR_INTEGRAL_VARIANCE_PIN

G.  Listing of Parameters for XRS Fractional Footprint Accounting System

Internal to the University of Arizona RDBMS information for accounting purposes only.  This information will not be output at the query time.

- MET						INTEGER*4

- FOOTPRINT_BIN_TOTAL_AREA		REAL*4 			NOT VALID

- FOOTPRINT_BIN_ILLUM_AREA		REAL*4			=0 IF FOV_STATUS=2,4

- FOOTPRINT_BIN_VISIBLE_AREA		REAL*4

- FOOTPRINT_BIN_EFF_AREA			REAL*4			NOT VALID

- TOTAL_EFF_SOLID_ANGLE_FRACTION	REAL*4

- SPECTRUM_FRACTION			REAL*4		NOT VALID IF FOV_STATUS=2,4

- BIN_ID						REAL*4

### Numerical Specifications for XRS Level–2 Integral Records

FORMATS  1,2 – Numerical specifications for Single Raw and Calibrated XRS Level-2 record (output)

###### H.1. – H.4.   NEAR XRS Spectra:  4 spectra*256 channels*(Real*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.

## H.5.   NEAR XRS Science Housekeeping (SCI HK) Parameters:  53*(Integer*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.

## H.6.   NEAR XRS Engineering Housekeeping (ENG HK) Parameters: 163*(REAL*4)

# The following parameters are described in detail in the NEAR XGRS DPU Software Requirements Specification JHU/APL Version 7 7358-9002.   Version 7 of the software was uploaded in May 1999.

H.7.   NEAR XRS Derived Engineering (DER ENG) Parameters:  28*(Real*4), 1*(Integer*4), 1*(4*Integer*4)

NOTE: *’ed items are pending science team approval and development.

PARAMETER NAME                           		UNITS

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- Spare

- UNF_LIVE_TIME                                                Unitless

UNF_LIVE_TIME =

UNF_XRAY_VALID_EVENTS(SCI_HK:27)/(UNF_XRAY_RAW_EVENT_RATE(SCI_HK:22)  -

0.037 * XTALK_COUNTER(SCI_HK:36))

- MG_LIVE_TIME                                                  Unitless

MG_LIVE_TIME =

MG_XRAY_VALID_EVENTS(SCI_HK:28)/(MG_XRAY_RAW_EVENT_RATE(SCI_HK:23)  -

0.025 * XTALK_COUNTER(SCI_HK:36))

- AL_LIVE_TIME                                                   Unitless

AL_LIVE_TIME =

AL_XRAY_VALID_EVENTS(SCI_HK:29)/(AL_XRAY_RAW_EVENT_RATE(SCI_HK:24)  -

0.031 * XTALK_COUNTER(SCI_HK:36))

- SOLAR_LIVE_TIME                                          Unitless

IF Active Solar Monitor is GAS

SOLAR_LIVE_TIME =

GAS_XRAY_VALID_EVENTS(SCI_HK:30) / GAS_XRAY_RAW_EVENT_RATE(SCI_HK:25)

IF Active Solar Monitor is PIN

SOLAR_LIVE_TIME =

PIN_SOLAR_PROCESS_EVENTS(SCI_HK:35) / PIN_XRAY_RAW_EVENT_RATE(SCI_HK:26)

16  UNF_VALID_CHANNEL_HI   	         Channel			Highest channel of the spectrum in original

(no calibration) or new (after calibration) energy scale.

=N(253), if N(253)<=255

=255,    if N(253)>255,

N(253)=253 for non-calibrated spectra.

N(253): channel number in the

new energy scale, corresponding

to 253 channel of the original spectrum, for calibrated spectra.

N(253)=253⋅REALGAIN...+REALZERO...−ZEROSTANDARD...GAINSTANDARD...

17  MG_VALID_CHANNEL_HI	    	- “ -					- “ -

18  AL_VALID_CHANNEL_HI      		- “ -					- “ -

19  UNF_VALID_CHANNEL_LOW	- “ –		    	Lowest channel of the spectrum in original

(no calibration) or new (after calibration) energy scale.

=N(LLD), if N(LLD)>=0

=0,    if N(LLD)<0,

N(LLD)=(Eng_HK:112) or 10, whatever is higher, for non-calibrated spectra.

N(LLD): channel number in the

new energy scale, corresponding

to low level discriminator setting of the original spectrum LLD=(Eng_HK:112) or 10, whatever is higher, for calibrated spectra.

N(LLD)=LLD⋅REALGAIN...+REALZERO...−ZEROSTANDARD...GAINSTANDARD...

20  MG_VALID_CHANNEL_LOW 	- “ -			- “ -  N(LLD) corresponds to Eng_HK:113

or 10, whatever is higher.

21  AL_VALID_CHANNEL_LOW 		- “ -			- “ -  N(LLD) corresponds to Eng_HK:114

or 10, whatever is higher.

22  UNF_RISE_TIME_CHANNEL		- “ –		    	Rise time threshold channel in the new

(after calibration) energy scale.

=N(RT)=(Eng_HK:101) for non-calibrated spectra.

=N(RT): channel number in the

new energy scale, corresponding

to a rise time discriminator setting in the original spectrum (Eng_HK:101), for calibrated spectra.

N(RT)=(Eng_HK:101)⋅REALGAIN...+REALZERO...−ZEROSTANDARD...GAINSTANDARD...

23  MG_RISE_TIME_CHANNEL	 	- “ -			- “ -  N(RT) corresponds to Eng_HK:102

24  AL_RISE_TIME_CHANNEL	 	- “ -			- “ -  N(RT) corresponds to Eng_HK:103

25  Spare

26  Spare

27  Spare

28  BAD_FLAG							Provided by GSFC

29-32  QUERY_ID

H.8.  NEAR XRS Spatial (SPATIAL) Parameters:  1*(Integer*4), 38*(Real*4)

All spatial derivations, unless specified differently, are made for the middle of integration period.

*  ET :  ephemeris time corresponding to clock fixed mission elapsed time in the middle of the integration

period.  Measuring units are seconds before January 1, 2000.

PARAMETER NAME           UNITS

0  MET				Seconds		Mission Elapsed Time at start of integration.

1  PlateID_Boresight_Intersect  	Plate Index   		ID of the plate that boresight intersects

in the middle of integration period

2  BinID_Boresight_Intersect    	Bin   Index   		ID of the bin that boresight intersects

in the middle of integration period

3  SC_Position, X              		Coord X, km    	Position of spacecraft (ET*) in Erosfixed

4  SC_Position, Y              		Coord Y, km    	coord. system (ET*).

5  SC_Position, Z              		Coord Z, km

6  BS_Vector, X                		Vector X, normal    	Boresight vector of XRS (ET*) in Erosfixed

7  BS_Vector, Y                		Vector Y, normal    	coord. system (ET*).   (Instr. Pointing Direction)

8  BS_Vector, Z                		Vector Z, normal

9  Earth_Position, X          		Coord X, km    	Position of the Earth (ET*) in Erosfixed

10 Earth_position, Y           		Coord Y, km    	coord. system (ET*).

11 Earth_Position, Z           		Coord Z, km

12 Sun_Position, X             		Coord X, km    	Position of the Sun in Erosfixed coord.

13 Sun_Position, Y             		Coord Y, km    	system (ET*), corrected for light

14 Sun_Position, Z             		Coord Z, km    	time Sun-Eros (ET*- LTSun-Eros).

15 FOV_Status                  		Status value   		= 0: 	Field of view (FOV) is

completely off the asteroid.

= 1: 	The field of view is totally on the

asteroid and at least part of the footprint is lit by the sun.

= 2: 	The field of view is totally on the

asteroid and ALL of the plates are dark. (Asteroid darkside)

= 3: 	Part of the field of view is off the

asteroid and at least one plate is lit by the sun.

= 4: 	Part of the field of view is off the

asteroid and all plates are dark.

(Asteroid darkside)

For derivation see Appendix 2

16 Downlink_Status            		         Status Value    	= 0:      No downlink

= 1:      Yes downlink (The angle between

antenna direction and vector

spacecraft(ET*)→Earth(ET*) is less than 2

degrees)

17 Angle_Sun_Solar_Mon          		Degrees       		Angle between Solar Monitor Normal(ET*)

and vector spacecraft(ET*)→Sun(ET*-LTSun-NEAR)

18 Avg_SC_Distance		 		Kilometers     		Average distance from the spacecraft

to the surface of the asteroid contributing to the spectrum.

Approximation for Collimator_Fct can be found in Appendix 2.

- illuminated and visible plates,                    footprint,                if FOV_STATUS=1,3- visible plates,               if FOV_STATUS=2,4(Fractions of the plates are considered)⋅Collimator_Fct);Plate_Solid_Angle=|Cos(emi_angle)|⋅Plate_Area/SC_Distance2SC_Distance= Distance from the spacecraft to the midpoint of the plate

19 Sun_Distance                			Kilometers     		Sun_Position vector magnitude.

20 Avg_Emission_Angle           		Degrees       		Average emission angle (between normal to

the surface and direction from the surface to the spacecraft).

Approximation for Collimator_Fct is given in Appendix 2.

- illuminated and visible plates,               footprint,                if FOV_STATUS=1,3- visible plates,               if FOV_STATUS=2,4(Fractions of the plates are considered)⋅CollimatorFct);PlateSolidAngle :   definition is given in specifications for parameter 18                                    of the present section.

21 Avg_Incidence_Angle          		Degrees 		Average incidence angle (between normal to

the surface and direction from the surface to the sun).

Approximation for Collimator_Fct is given in Appendix 2.

illuminated and visibleplates              footprint,(Fractionsoftheplatesareconsidered)⋅CollimatorFct);PlateSolidAngle  :   definition is given in specifications for parameter 18                                    of the present section.

22  Phase_Angle       		 		Degrees	 	Phase angle (between

direction from the sun to the surface and direction from the surface to the spacecraft).

23 Avg_Inc_Emi_Cos_Ratio        		Unitless 		Average ratio of the cosine of incidence

angle to the cosine of emission angle

Approximation for Collimator_Fct is given in Appendix 2.

illuminated and visibleplates              footprint,(Fractions of the plates are considered)⋅CollimatorFct);PlateSolidAngle :   definition is given in specifications for parameter 18                                    of the present section.

24 Spatial_Fluorescence_Norm    		Steradians       	 	Spatial normalization coefficient

for fluorescence flux.  It relates the fluorescence flux at the detector to the fluorescence intensity from the asteroid surface in the direction normal to the surface under the condition that  incidence angle =0.

For complete definition see Appendix 1.

Approximations for Angular_Fluor_Fct and Collimator_Fct  are given in Appendix 2.

illuminated and visible plates               footprint,(Fractions of the plates are considered)Cos(emiangle))PlateSolidAngle :   definition is given in specifications for parameter 18                                    of the present section.

25 Spatial_Scattering_Norm      		Steradians 	 	Spatial normalization coefficient

for scattering.  It relates the scattering flux  at the detector to the intensity of scattering from the asteroid surface under the condition that incidence angle = emission angle =  0

For complete definition see Appendix 1.

Approximations for Angular_Scatter_Fct and Collimator_Fct are given in Appendix 2.

illuminated and visible plates               footprint,(Fractions of the plates are considered)Cos(emiangle))PlateSolidAngle :   definition is given in specifications for parameter 18                                    of the present section.

26 Total_Area_Footprint         		Sq. Kilometers     	NOT VALID  (= -1)

27 Total_Illuminated_Area       		Sq. Kilometers            Total area of  plates in the

footprint contributing to

the spectrum  (lit by the sun and visible from the spacecraft)

illuminated and visible plates                footprint,(Fractions of the plates are considered)=footprint(PlateArea∗VISAMT∗LITAMT),            where   VISAMT: fraction of the plate that is located within the field of view                                   and is visible to the spacecraft.,                  LITAMT: fraction of VISAMT lit by the sun.

28 Total_Visible_Area		  	Sq. Kilometers     	Total area of plates in the

footprint visible from the spacecraft

visibleplatesfootprint,footprint(PlateArea∗VISAMT),                                where VISAMT: fraction of the plate that is located within the field of view                                                       and is visible to the spacecraft.

29 Total_Eff_Area	    	        		Sq. Kilometers     	NOT VALID  (= -1)

30 Total_Eff_Solid_Angle         		Unitless		Overall solid angle of all plates

in the footprint contributing to the spectrum (lit by the sun and visible from the spacecraft) and weighted by the response function of the collimator, relatively the instrument full field of view.

- illuminated and visible plates,               footprint,                if FOV_STATUS=1,3- visible plates,               if FOV_STATUS=2,4(Fractions of the plates are considered)FOVSolidAngle;FOVSolidAngle=0.005828 steradians :    XRS field of view  (16-sided polygon)EffPlateSolidAngle=PlateSolidAngle⋅CollimatorFctPlateSolidAngle :   definition is given in specifications for parameter 18                                    of the present section.Approximation for Coliimator_Fct is given in Appendix 2

31 Footprint_Solid_Angle	   		Unitless          		Overall solid angle of all plates in the

footprint as seen from the spacecraft, relatively the instrument full field of view.

* Plate solid angle is modified by plate   fraction in FOV

visibleplates   footprint,(Fractions of the plates are considered)FOVSolidAngleFOVSolidAngle : definition is given in specifications                                 for parameter 30 of the present section

32 Boresight_Intersection_Lat	   	Degrees		Latitude of boresight intersection

in the middle of integration period

Coordinate transformation Asteroid Body Fixed, Cartesian to

Asteroid Body Fixed, Spherical. Intersection point (x,y,z) of

boresight with asteroid surface converted to Latitude.

LAT = Degrees(ARC_SIN(Z/R))             (R solved for in 34)

33 Boresight_Intersection_Lon	   	Degrees		Longitude of boresight intersection

in the middle of integration period

Coordinate transformation Asteroid Body Fixed, Cartesian to

Asteroid Body Fixed, Spherical. Intersection point (x,y,z) of

boresight with asteroid surface converted to Longitude.

1.    LON = -1.*Degrees(ARC_TAN(Y/X))

2.    LON = 270				for x = 0 AND y > 0

3.    LON = 90					for x = 0 AND y < 0

34 Boresight_Intersection_R	   		Kilometers		Radius-vector of boresight intersection

in the middle of integration period

Coordinate transformation Asteroid Body Fixed, Cartesian to

Asteroid Body Fixed, Spherical. Intersection point (x,y,z) of

boresight with asteroid surface converted to Radius from

center of asteroid mass.

R =    SQRT( X^2 + Y^2 + Z^2)         (R is needed in sol’n 32)

35 SPICE_Kernel_ID             			Version Number		Specified by NAIF

36 Plate_Model_ID              			Version Number

37 Processing_Software_ID      		Version Number

38 DataBase_Version_ID         		Version Number

H.9.  NEAR XRS Solar (SOLAR) Parameters: 2*(Real*4)

PARAMETER NAME                          	   UNITS

- SOLAR_INTEGRAL 			Counts/Second		Solar monitor counting rate,

corrected for the radiation incidence angle and normalized to 1.4 Astronomical Unit distance to the Sun.   Reflects changes in solar x-ray flux.

IF   Active  Solar  Monitor   is   PIN       THENSolarIntegral==I=32253SolarSp(I)⋅SunDistance2CosAngleSolMon⋅SolarLiveTime⋅ActvSolMonIntegTime⋅D02;where        D0=1.4 AU =1.4⋅1.4959787⋅108  km

IF   Active  Solar  Monitor   is   GAS       THENSolarIntegral==I=19253SolarSp(I)⋅SunDistance2CosAngleSolMon⋅SolarLiveTime⋅ActvSolMonIntegTime⋅D02;where        D0=1.4 AU =1.4⋅1.4959787⋅108  km

1	 SOLAR_LEVEL       Set correspondingly to 10 solar activity ranges provided by GSFC,

Control parameter: (Solar_Integral*300)

Takes values 0 .. 9.

V = Solar_Integral * 300

For PIN – detector:   Example:   V values will be modified and subject to change:  Dynamically loaded runtime.

IF  V< 3500  THEN   SOLAR_LEVEL = 0

IF V>= 3500  AND   V < 5000   THEN   SOLAR_LEVEL = 1

IF V>= 5000  AND   V < 7000   THEN   SOLAR_LEVEL = 2

IF V>= 7000  AND   V < 11000   THEN   SOLAR_LEVEL = 3

IF  V >= 11000  AND   V < 20000   THEN   SOLAR_LEVEL = 4

IF  V >= 20000  AND   V < 40000   THEN   SOLAR_LEVEL = 5

IF  V >= 40000  AND   V < 80000   THEN   SOLAR_LEVEL = 6

IF  V >= 80000  AND   V < 200000   THEN   SOLAR_LEVEL = 7

IF  V >= 200000  AND   V < 1750000   THEN   SOLAR_LEVEL = 8

IF  V >= 1750000 THEN   SOLAR_LEVEL = 9

For GAS – detector:

IF V < 16000  THEN   SOLAR_LEVEL = 0

IF V >= 16000  AND   V < 32000   THEN   SOLAR_LEVEL = 1

IF  V >= 32000  AND   V < 63000   THEN   SOLAR_LEVEL = 2

IF  V >= 63000  AND   V < 100000   THEN   SOLAR_LEVEL = 3

IF  V >= 100000  AND   V < 105000   THEN   SOLAR_LEVEL = 4

IF  V >= 105000  AND   V < 120000   THEN   SOLAR_LEVEL = 5

IF  V >= 120000  AND   V < 140000   THEN   SOLAR_LEVEL = 6

IF  V >= 140000  AND   V < 180000   THEN   SOLAR_LEVEL = 7

IF  V >= 180000  AND   V < 1900000   THEN   SOLAR_LEVEL = 8

IF  V >= 1900000 THEN   SOLAR_LEVEL = 9

#### I. Numerical Specifications for XRS Level–2 Summary Record

FORMAT  3 – Numerical specifications for Summarized XRS Level-2 record (output)

###### I.1.    NEAR XRS Summarized Unfiltered Spectrum with GAS_PC Active (UNF,GAS_PC):  256 Channels*(Real*4)

Measuring units:   Counts/Channel

GAS activeMET with I=0...255− channelAVGLIVETIME(MET)=(ALLIVETIME⋅ALmon_integtime++MGLIVETIME⋅MGmon_integtime+UNFLIVETIME⋅UNFmon_integtime)3

###### I.2.    NEAR XRS Summarized MG Spectrum with GAS_PC Active (MG,GAS_PC):  256 Channels*(Real*4)

Measuring units:   Counts/Channel

GAS activeMET with I=0...255− channelAVGLIVETIME(MET):  definition is given in specifications for section I.1.

###### I.3.    NEAR XRS Summarized AL Spectrum with GAS_PC Active (AL,GAS_PC):  256 Channels*(Real*4)

Measuring units:   Counts/Channel

GAS activeMET with I=0...255− channelAVGLIVETIME(MET):  definition is given in specifications for section I.1.

###### I.4.    NEAR XRS Summarized GAS_PC  Spectrum  (GAS_PC):  256 Channels*(Real*4)

Measuring units:   Counts/Channel

GASPC(I)=      =MET with GAS activeSOLAR(I,MET)⋅AVGLIVETIMECosAngleSunSolarMon⋅SOLARLIVETIME⋅ActiveMonIntegTime;I=0...255− channelAVGLIVETIME(MET):  definition is given in specifications for section I.1.

###### I.5.    NEAR XRS Summarized Unfiltered Spectrum with PIN Active (UNF,PIN):  256 Channels*(Real*4)

Measuring units:   Counts/Channel

PIN activeMET with I=0...255− channelAVGLIVETIME(MET):  definition is given in specifications for section I.1.

###### I.6.    NEAR XRS Summarized MG Spectrum with PIN Active (MG,PIN):  256 Channels*(Real*4)

Measuring units:   Counts/Channel

PIN activeMET with I=0...255− channelAVGLIVETIME(MET):  definition is given in specifications for section I.1.

###### I.7.    NEAR XRS Summarized AL Spectrum with PIN active (AL,PIN):  256 Channels*(Real*4)

Measuring units:   Counts/Channel

PIN activeMET with I=0...255− channelAVGLIVETIME(MET):  definition is given in specifications for section I.1.

###### I.8.    NEAR XRS Summarized PIN Spectrum (PIN):  256 Channels*(Real*4)

Measuring units:   Counts/Channel

PIN(I)=     =MET with PIN activeSOLAR(I,MET)⋅AVGLIVETIMECosAngleSunSolarMon⋅SOLARLIVETIME⋅ActiveMonIntegTime;I=0...255:    channelPIN(I)=0,         I=0..9,254,255AVGLIVETIME(MET):  definition is given in specifications for section I.1.

I.9.  NEAR XRS Summarized Engineering (ENGINEERING) Parameters:  48*(Real*4)

PARAMETER NAME                                  	   UNITS

0	Spare

1	Spare

2     	Spare

3     	Spare

4	Spare

5     	Spare

6	TOTAL_INTEG_TIME_UNF_GAS 	   	Seconds   METwithGASactiveUnfMonIntegTime(MET)

7	TOTAL_INTEG_TIME_MG_GAS 	   	Seconds   METwithGASactiveMgMonIntegTime(MET)

8	TOTAL_INTEG_TIME_AL_GAS  	   	Seconds   METwithGASactiveAlMonIntegTime(MET)

9     	TOTAL_INTEG_TIME_GAS  	         	Seconds   METwithGASactiveActiveMonIntegTime(MET)

10	TOTAL_LIVE_TIME_UNF_GAS          	Seconds

MET with GAS active[UNFLIVETIME(MET)⋅UNFMonIntegTime]

11	TOTAL_LIVE_TIME_MG_GAS		Seconds

MET with GAS active[MGLIVETIME(MET)⋅MGMonIntegTime]

12	TOTAL_LIVE_TIME_AL_GAS		Seconds

MET with GAS active[ALLIVETIME(MET)⋅ALMonIntegTime]

13    	TOTAL_LIVE_TIME_GAS              		Seconds

MET with GAS active[SOLARLIVETIME(MET)⋅ActiveMonIntegTime]

14	UNF_VALID_CHANNEL_HI_GAS    	Channel	Highest channel of summed spectrum

uncorrupted by energy scale adjustments.

Lowest UNF_VALID_CHANNEL_HI among all MET records in a sum.

15    	MG_VALID_CHANNEL_HI_GAS	    	Channel	- “ - Lowest MG_VALID_CHANNEL_HI

16	AL_VALID_CHANNEL_HI_GAS  	   	Channel	 - ‘ - Lowest AL_VALID_CHANNEL_HI

17	UNF_VALID_CHANNEL_LOW_GAS   	Channel	Lowest channel of summed spectrum

uncorrupted by energy scale adjustments or LLD variations.

Highest UNF_VALID_CHANNEL_LOW among all MET records in a sum.

18	MG_VALID_CHANNEL_LOW_GAS    	Channel        - ‘ - Highest MG_VALID_CHANNEL_LOW

19	AL_VALID_CHANNEL_LOW_GAS    	Channel        - ‘ - Highest AL_VALID_CHANNEL_LOW

20	UNF_INVALID_CHANNEL_HI_GAS  	Channel	Highest channel of rise-time transition

region in the summed spectrum formed as a result of energy scale or rise time settings variations.

Highest UNF_RISE_TIME_CHANNEL among all MET records in a sum.

21	MG_INVALID_CHANNEL_HI_GAS   	Channel	- ‘ -Highest MG_RISE_TIME_CHANNEL

22	AL_INVALID_CHANNEL_HI_GAS   	Channel	- ‘ - Highest AL_RISE_TIME_CHANNEL

23	UNF_INVALID_CHANNEL_LOW_GAS 	Channel	Lowest channel of rise-time transition

region in the summed spectrum formed as a result of energy scale or rise time settings variations.

Lowest UNF_RISE_TIME_CHANNEL among all MET records in a sum.

24	MG_INVALID_CHANNEL_LOW_GAS  	Channel	- ‘ - Lowest MG_RISE_TIME_CHANNEL

25	AL_INVALID_CHANNEL_LOW_GAS  	Channel	- ‘ - Lowest AL_RISE_TIME_CHANNEL

26        Number_MET_GAS        Number of MET records in the query when GAS solar monitor was active

27	TOTAL_INTEG_TIME_UNF_PIN          	Seconds    METwithPINactiveUnfMonIntegTime(MET)

28	TOTAL_INTEG_TIME_MG_PIN		Seconds    METwithPINactiveMgMonIntegTime(MET)

29	TOTAL_INTEG_TIME_AL_PIN 	    	Seconds     METwithPINactiveAlMonIntegTime(MET)

30    	TOTAL_INTEG_TIME_PIN  	          	Seconds METwithPINactiveActiveMonIntegTime(MET)

31	TOTAL_LIVE_TIME_UNF_PIN           	Seconds

MET with PIN active[UNFLIVETIME(MET)⋅UNFMonIntegTime]

32	TOTAL_LIVE_TIME_MG_PIN		Seconds

MET with PIN active[MGLIVETIME(MET)⋅MGMonIntegTime]

33	TOTAL_LIVE_TIME_AL_PIN		 Seconds

MET with PIN active[ALLIVETIME(MET)⋅ALMonIntegTime]

34    	TOTAL_LIVE_TIME_PIN               		 Seconds

MET with PIN active[SOLARLIVETIME(MET)⋅ActiveMonIntegTime]

35	UNF_VALID_CHANNEL_HI_PIN 	    	Channel	Highest channel of summed spectrum

uncorrupted by energy scale adjustments.

Lowest UNF_VALID_CHANNEL_HI among all MET records in a sum.

36    	MG_VALID_CHANNEL_HI_PIN	   	Channel	- “ - Lowest MG_VALID_CHANNEL_HI

37	AL_VALID_CHANNEL_HI_PIN    	    	Channel	- “ - Lowest AL_VALID_CHANNEL_HI

38	UNF_VALID_CHANNEL_LOW_PIN   	Channel	Lowest channel of summed spectrum

uncorrupted by energy scale adjustments or LLD variations.

Highest UNF_VALID_CHANNEL_LOW among all MET records in a sum.

39	MG_VALID_CHANNEL_LOW_PIN    	Channel        - “ - Highest MG_VALID_CHANNEL_LOW

40	AL_VALID_CHANNEL_LOW_PIN    	Channel        - “ - Highest AL_VALID_CHANNEL_LOW

41	UNF_INVALID_CHANNEL_HI_PIN  	Channel	Highest channel of rise-time transition

region in the summed spectrum formed as a result of energy scale or rise time settings variations.

Highest UNF_RISE_TIME_CHANNEL among all MET records in a sum.

42	MG_INVALID_CHANNEL_HI_PIN    	Channel	- ‘ - Highest MG_RISE_TIME_CHANNEL

43	AL_INVALID_CHANNEL_HI_PIN	    	Channel	- ‘ - Highest AL_RISE_TIME_CHANNEL

44	UNF_INVALID_CHANNEL_LOW_PIN 	Channel	Lowest channel of rise-time transition

region in the summed spectrum formed as a result of energy scale or rise time settings variations.

Lowest UNF_RISE_TIME_CHANNEL among all MET records in a sum.

45	MG_INVALID_CHANNEL_LOW_PIN  	Channel	- ‘ - Lowest MG_RISE_TIME_CHANNEL

46	AL_INVALID_CHANNEL_LOW_PIN  	Channel	- ‘ - Lowest AL_RISE_TIME_CHANNEL

47        Number_MET_PIN         Number of MET records in the query when PIN solar monitor was active

I.10.   NEAR XRS Summarized Spatial (SPATIAL) Parameters:  17*(Real*4)

PARAMETER NAME                         UNITS

0	AVG_SC_DISTANCE_GAS		  Kilometers		Distance from the spacecraft to the surface

of the asteroid averaged over MET records in a sum with proportional counter active.

Weight(MET)=TotalEffSolidAngle(MET)⋅AVGLIVETIME(MET)AVGLIVETIME(MET)=(ALLIVETIME⋅ALmon_integtime++MGLIVETIME⋅MGmon_integtime+UNFLIVETIME⋅UNFmon_integtime)3                                                                    AvgSCDISTANCEGAS=MET with GASactive[AvgSCDistance(MET)⋅Weight(MET))]MET with GASactive[Weight(MET)]

1	AVG_EMI_ANGLE_GAS 		    Degrees 		Emission angle averaged over MET records

in a sum with proportional counter active.

AVGEMIANGLEGAS=MET with GASactive[AvgEmissionAngle(MET)⋅Weight(MET))]MET with GASactive[Weight(MET)]Weight(MET) :   definition is given in specifications for parameter 0                          of the present section.

2	AVG_INC_ANGLE_GAS		    Degrees 		Incidence angle averaged over MET records

in a sum with proportional counter active.

AVGINCANGLEGAS=MET with GASactive[AvgIncidenceAngle(MET)⋅Weight(MET))]MET with GASactive[Weight(MET)]Weight(MET) :   definition is given in specifications for parameter 0                          of the present section.

3	AVG_PHASE	_GAS 	  		  Degrees 		Phase angle

averaged over MET records in a sum

with proportional counter active.

AvgPhaseGAS=MET with GASactive[PhaseAngle(MET)⋅Weight(MET))]MET with GASactive[Weight(MET)]Weight(MET) :   definition is given in specifications for parameter 0                          of the present section.

4	AVG_INC_EMI_COS_RATIO_GAS          Unitless 	Ratio of cosines of incidence and emission

angles averaged over MET records in a sum with proportional counter active.

AvgIncEmiCosRatioGAS=                                                =MET with GASactive[AvgIncEmiCosRatio(MET)⋅Weight(MET))]MET with GASactive[Weight(MET)]Weight(MET) :   definition is given in specifications for parameter 0                          of the present section.

5	AVG_EFF_SOLID_ANGLE_GAS	            Unitless 	Total effective solid angle averaged over

MET records in a sum with proportional

counter active.

MET with GAS activeTotalEffSolidAngle(MET)⋅AVGLIVETIME(MET)MET with GAS activeAVGLIVETIME(MET)AVGLIVETIME(MET):   definition is given in specifications for parameter 0                                              of the present section.

6     Total_Weight_Fluor_GAS             	Steradians*Counts    	Normalization coefficient providing the link

between the summed fluorescence spectrum and the intensity of fluorescence from the asteroid surface in the direction normal to the surface under the condition that incidence angle = 0 (for MET’s with proportional counter active).

For complete definition see Appendix 1.

TOTAL_WEIGHT_FLUOR_GAS==MET with GAS active(Solar_Integral⋅D02⋅AVGLIVETIME⋅SpatialFluorescenceNormSun_Distance2),where        D0=1.4 AU =1.4⋅1.4959787⋅108  kmAVGLIVETIME(MET) :    definition is given in specifications for parameter 0                                               of the present section.

7     Total_Weight_Scat_GAS              	Steradians*Counts	Normalization coefficient providing the

link between the summed spectrum of scattering and the intensity of scattering from the asteroid surface under the condition that  incidence angle = emission angle = half-sum of incidence and emission angles averaged over all MET records participating in the sum (for MET’s with proportional counter active).   For complete definition see Appendix 1.

TOTAL_WEIGHT_SCAT_GAS==MET with GAS active(Solar_Integral⋅D02⋅AVGLIVETIME⋅SpatialScatteringNormSun_Distance2),where        D0=1.4 AU =1.4⋅1.4959787⋅108  kmAVGLIVETIME(MET) :    definition is given in specifications for parameter 0                                               of the present section.

8       AVG_SC_DISTANCE_PIN		     Kilometers 		Distance from the spacecraft to the surface

of the asteroid averaged over MET records in a sum with PIN monitor active.

AvgSCDISTANCEPIN=MET with PIN active[AvgSCDistance(MET)⋅Weight(MET))]MET with PIN active[Weight(MET)]Weight(MET) :    definition is given in specifications for parameter 0                          of the present section.

9	AVG_EMI_ANGLE_PIN 		      	Degrees	Emission angle averaged over MET records

in a sum with PIN monitor active.

AVGEMIANGLEPIN=MET with PIN active[AvgEmissionAngle(MET)⋅Weight(MET))]MET with PIN active[Weight(MET)]Weight(MET) :    definition is given in specifications for parameter 0 of the present section.

10	AVG_INC_ANGLE_PIN		            Degrees	Incidence angle averaged over MET records

in a sum with PIN monitor active.

AVGINCANGLEPIN=MET with PIN active[AvgIncidenceAngle(MET)⋅Weight(MET))]MET with PIN active[Weight(MET)]Weight(MET) :    definition is given in specifications for parameter 0 of the present section.

11	AVG_PHASE_PIN 	         			Degrees	Phase angle

averaged over MET records in a sum

with PIN monitor active.

AvgPhasePIN=MET with PIN active[PhaseAngle(MET)⋅Weight(MET))]MET with PIN active[Weight(MET)]Weight(MET) :    definition is given in specifications for parameter 0 of the present section.

12	AVG_INC_EMI_COS_RATIO_PIN          	Unitless	Ratio of cosines of incidence and emission

angles averaged over MET records in a sum

with PIN monitor active.

AvgIncEmiCosRatioPIN=                                                =MET with PIN active[AvgIncEmiCosRatio(MET)⋅Weight(MET))]MET with PIN active[Weight(MET)]Weight(MET) :   definition is given in specifications for parameter 0 of the present section.

13	AVG_EFF_SOLID_ANGLE_PIN	      Unitless 		Total effective solid angle averaged over

MET records in a sum with PIN monitor

active.

MET with PIN activeTotalEffSolidAngle(MET)⋅AVGLIVETIME(MET)MET with PIN activeAVGLIVETIME(MET)AVGLIVETIME(MET) :    definition is given in specifications for parameter 0                                              of the present section.

14    Total_Weight_Fluor_PIN            	Steradians*Counts	Normalization coefficient providing the link

between the summed fluorescence spectrum and the intensity of fluorescence from the asteroid surface in the direction normal to the surface under the condition that incidence angle = 0 (for MET’s with PIN monitor active).

For complete definition see Appendix 1.

MET with PIN active(Solar_Integral⋅D02⋅AVGLIVETIME⋅SpatialFluorescenceNormSun_Distance2),where        D0=1.4 AU =1.4⋅1.4959787⋅108  kmAVGLIVETIME(MET):    definition is given in specifications for parameter 0                                               of the present section.

15    Total_Weight_Scat_PIN             	Steradians*Counts	Normalization coefficient providing the

link between the summed spectrum of scattering and the intensity of scattering from the asteroid surface under the condition that incidence angle = emission angle = half-sum of incidence and emission angles averaged over all MET records participating in the sum (for MET’s with PIN monitor active).   For complete definition see Appendix 1.

MET with PIN active(Solar_Integral⋅D02⋅AVGLIVETIME⋅SpatialScatteringNormSun_Distance2),where        D0=1.4 AU =1.4⋅1.4959787⋅108  kmAVGLIVETIME(MET):     definition is given in specifications for parameter 0                                              of the present section.

16	ESTIMATED_COVERED_AREA	 Sq. Kilometers 	Estimated area of the asteroid surface

covered by MET records in a sum

Bins in a query regionBinArea            For a spatial query

Unique bins covered by all footprints in a sumBinArea   For a temporal query

I.11.  NEAR XRS Summarized Solar (SOLAR) Parameters:  4*(Real*4)

PARAMETER NAME                      UNITS

1        Average_Solar_Integral_GAS       	    Counts       MET with GAS activeSolarIntegral(MET)NumberMETGAS

2       Solar_Integral_Variance_GAS 	   	    Sq. Counts

METwithGASactive(SolarIntegral(MET)−AverageSolarIntegralGAS)2NumberMETGAS−1

3        Average_Solar_Integral_PIN       	      Counts       MET with PIN activeSolarIntegral(MET)NumberMETPIN

4        Solar_Integral_Variance_PIN       	   Sq. Counts

METwithPINactive(SolarIntegral(MET)−AverageSolarIntegralPIN)2NumberMETPIN−1

J.  Numerical Specifications for XRS Fractional Footprint Accounting System

PARAMETER NAME                      UNITS

0  	MET 					Seconds		Mission Elapsed Time at start of integartion

1  	Footprint_Bin_Total_Area	        Sq. Kilometers 		NOT VALID (= -1)

2  	Footprint_Bin_Illum_Area	        Sq. Kilometers		Total area of all plates in the footprint

that belong to the bin, contributing into the spectrum (lit by the sun and visible from the spacecraft)

illuminatedby  the Sun and visible from the s/c                plates of the footprint, that belong to the BIN,                     (Fractions of the plates are considered)			 Fractions are considered as in H8 param.29

3  	Footprint_Bin_Visible_Area	        Sq. Kilometers		Total area of all plates in the

footprint that belong to the bin, visible from the spacecraft

visible from the spacecraft                plates of the footprint, that belong to the BIN,                     (Fractions of the plates are considered) 			Fractions are considered as in H8 param.28

4  	Footprint_Bin_Eff_Area	        Sq. Kilometers		NOT VALID (= -1)

5  	Total_Eff_Solid_Angle_Fraction      Unitless		Fraction of the total effective

solid angle that can be attributed to the bin.

- illuminated and visible, if FOV_STATUS=1,3  plates of the footprint.-visible, if FOV_STATUS=2,4(Fractions of the plates in terms of illumination and visibility are considered)EffPlateSolidAngle=PlateSolidAngle⋅CollimatorFctPlateSolidAngle=|Cos(emiangle)|⋅PlateAreaSCDistance2SCDistance= Distance from the spacecraft to the midpoint of the plateApproximation for Coliimator_Fct is given in Appendix 2

6  	Spectrum_Fraction			Unitless		Fraction of the spectrum that can

be attributed to the bin.

Approximations for Angular_Scatter_Fct and Collimator_Fct are given in Appendix 2.

illuminated and visible plates                footprint,(Fractions of the plates are considered)Cos(emiangle))PlateSolidAngle :    definition is given in specifications for parameter 5                                    of the present section.

7.       BINID					Number		Bin identifier as generated by the

Bin definition system. Each bin is unique.

APPENDIX 1.        		Definitions for:

Collimator_Fct;

Spatial_Fluorescence_Norm;

Angular_Fluor_Fct;

Total_Weight_Fluor;

Spatial_Scattering_Norm;

Angular_Scatter_Fct;

Total_Weight_Scatt.

Njfluor[counts]=Tlive[s]⋅Sdetector[cm2]⋅effx[countsphotons]⋅FluorFluxj[photons(s⋅cmdetector2)]FluorFluxj[photonss⋅cmdetector2]=I0[solar photonss⋅cm2]⋅footprintFj(α,β)4π[photonssr⋅solarphoton⋅cm2cmsurface2]⋅fcollCosβdΩ[sr⋅cmsurface2cmdetector2],where      Njfluor - number of counts at the XRS detector in j energy fluorescent line              Tlive- live time of the surface detector              Sdetector- area of the surface detector              effx- intrinsic efficiency of surface detector              FluorFluxj- fluorescence flux in line j              dΩ - solid angle of an area  on the asteroid surface as seen from the spacecraft              fcoll-   angular response function of XRS instrument (response, normalized to 1  at normal direction)              α - incidence angle (from the normal to the surface)              β  - emission angle ( from the normal to the surface)              I0− solar radiation intensity in a given energy range  (a...b) at the location of the asteroid             Fj(α,β)4π− fluorescence intensity from the surface of the asteroid per unit of solar radiation intensity in the energy                                  range (a...b) at the location of the asteroid              I0=abdIdλdλ=SolarIntegral[counts(n1..n2)s]⋅D02[km2]SunDistance2[km2]⋅AS[cm2]⋅effS[countsphotons]     [solar photonss⋅cm2],         where                                                                                                                 dIdλ[solar photons s⋅cm2⋅A  o]-differential solar spectrum;                                                                          a,b - wave lengths, corresponding to n1 and n2 channels                                                                         AS - area of solar monitor                                                                          effS - intrinsic efficiency of solar monitor                                                                          D0=1.4AU=1.4⋅1.4959787⋅108km              Fj(α,β)4π=1I0⋅14π⋅ϖj⋅gj⋅rj−1rj⋅Cj⋅λminλedgedIdλ⋅Cosα⋅μj(λ)⋅dλiCi[μi(λ)+μi(λj)CosαCosβ]     [photonssr⋅solarphoton⋅cm2cmsurface2]i - all elements of surface compositionj- fluorescing elementsϖj- Kα fluorescence yield for element jgj- Kα electron transfer probability for element jrj-  K absorption jump ratio for element j Cj-relative concentration (weight fraction) of element jμj- mass attenuation coefficient for fluorescence line j

FluorFluxj=I0⋅Fj(0,0)4π⋅footprintFj(α,β)Fj(0,0)⋅fcollCosβdΩ=(I0⋅Fj(0,0)4π)[photonssr⋅s⋅cmsurface2]⋅SpatialFluorescenceNorm,where     Spatial_Fluorescence_Norm=footprintFj(α,β)Fj(0,0)⋅fcollCosβ dΩ       [sr⋅cmsurface2cmdetector2],            AngularFluorFct=Fj(α,β)Fj(0,0)=λminλedgedIdλ⋅Cosα⋅μj(λ)⋅dλiCi[μi(λ)+μi(λj)CosαCosβ]    λminλedgedIdλ⋅μj(λ)⋅dλiCi[μi(λ)+μi(λj)]            Collimator_Fct = fcoll

Angular_Fluor_Fct:

Ratio of intensity of fluorescence (photons/(s*sr*cm2surface) from the surface

to the intensity of fluorescence from the same surface illuminated with the

same solar flux with the incidence angle = 0 in the direction of the normal to the

surface (emission angle = 0), which is a weak function of composition, energy and solar activity.

Spatial_Fluorescence_Norm:

Ratio of the fluorescence flux at the detector (photons/(s*cm2detector) from the

whole footprint at given orbital configuration to the intensity of

fluorescence from the asteroid surface (photons/(s*sr*cm2surface) under the same

solar flux and 0 incidence and emission angles.

METNjfluor=Sdetector⋅effx⋅Fj(0,0)4π⋅METTlive⋅I0⋅Spatial_Fluorescence_Norm=SdetectorAS⋅effxeffS⋅Fj(0,0)4π⋅⋅METTlive⋅SolarIntegral⋅D02[km2]SunDistance2[km2]⋅Spatial_Fluorescence_Norm;D0=1.4AU=1.4⋅1.4959787⋅108kmMETNjfluor=SdetectorAS⋅effxeffS⋅Fj(0,0)4π⋅TotalWeightFluor,    where                     Total_Weight_Fluor=METTlive⋅SolarIntegral⋅D02[km2]SunDistance2[km2]⋅Spatial_Fluorescence_Norm

Nscat(E)[countskeV]=Tlive[s]⋅Sdetector[cm2]⋅effx[countsphotons]⋅ScattFlux(E)[photons(s⋅keV⋅cmdetector2)]ScattFlux(E)[photonss⋅keV⋅cmdetector2]=I0[solar photonss⋅cm2]⋅footprintF(E,α,β,ϑ)[photonskeV⋅sr⋅solarphoton⋅cm2cmsurface2]⋅fcollCosβdΩ[sr⋅cmsurface2cmdetector2],where      Nscat(E) - number of counts at the XRS detector per 1 keV from scattering gamma radiation              F(E,α,β,ϑ) - scattering intensity from the surface per unit of solar radiation intensity in the (a...b) energy                                range at the location of the asteroid              ScattFlux(E)-flux of scattered radiation               ϑ - phase angle                Θ   -  solid angle of detection                                          F(E,α,β,ϑ)=              =1I0⋅i[CiAi⋅NAV⋅dIdE(E)⋅Cosα⋅dσicoh(E,ϑ)dΘiCi⋅μi(E)⋅(1+CosαCosβ)]+i[CiAi⋅NAV⋅dIdE(Ep)⋅Cosα⋅dσiincoh(E,ϑ)dΘiCi⋅(μi(Ep)+μi(E)⋅CosαCosβ)]                         Ai-atomic weight                         NAV-Avogadro number                         dσicoh(E,ϑ)dΘ- differential cross-section for coherent scattering                          dσiincoh(E,ϑ)dΘ-differential cross-section for incoherent scattering                          Ep-primary photon energyScattFlux(E)=I0⋅footprintF(E,0,0,ϑ)⋅F(E,α,β,ϑ)F(E,0,0,ϑ)⋅fcollCosβdΩ≈≈(I0⋅F(E,0,0,ϑ)   [photonss⋅sr⋅keV⋅cmsurface2]   ⋅SpatialScatteringNorm,where     Spatial_Scattering_Norm=footprintF(E,α,β,ϑ)F(E,0,0,ϑ)⋅fcollCosβdΩ      [sr⋅cmsurface2cmdetector2],            AngularScatterFct=F(E,α,β,ϑ)F(E,0,0,ϑ)

Angular_Scatter_Fct:

Ratio of intensity of scattering (photons/(s*keV*sr*cm2surface) from the surface  with a given incidence and emission angles,  and , to the intensity of scattering from the same surface with the incidence angle = emission angle =  0, which is a weak function of composition, energy and solar activity.

Spatial_Scattering_Norm:

Ratio of the scattering flux at the detector (photons/(s*keV*cm2detector) from the whole footprint at given orbital configuration to the scattering intensity of 1 cm2 of the asteroid surface (photons/(s*keV*sr*cm2surface) under the same solar flux when incidence angle = emission angle = 0.

METNscat(E)≈Sdetector⋅effx⋅F(E,0,0,<ϑ>)⋅METTlive⋅I0⋅Spatial_Scattering_Norm=SdetectorAS⋅⋅effxeffS⋅F(E,0,0,<ϑ>)⋅METTlive⋅SolarIntegral⋅D02[km2]SunDistance2[km2]⋅SpatialScattering_Norm;D0=1.4AU=1.4⋅1.4959787⋅108kmMETNscat(E)≈SdetectorAS⋅effxeffS⋅F(E,0,0,<ϑ>)⋅TotalWeightScatt,    where                     Total_Weight_Scatt=METTlive⋅SolarIntegral⋅D02[km2]SunDistance2[km2]⋅Spatial_Scattering_Norm

APPENDIX 2.       Approximations for:

Collimator_Fct;

Angular_Fluor_Fct;

Angular_Scatter_Fct;

FOV_Status

1)  Collimator_Fct = 0.97*exp(-0.5*((-0.02)/1.42)2)+0.028-0.11*  +0.0256* 2,

where – the angle between the boresight and the vector directed from the spacecraft to the midpoint of

the plate.

Collimator_Fct = 0, when > 3.4 degrees.

Figure 1.   Angular response of the XRS collimator;

measurements vs. approximation

- When XRS field of view is represented with the 16 sided polygon if all 16 sides are located on the asteroid  FOV_STATUS = 1 or 2 depending on the illumination status.   If less than 16 sides are located on the asteroid  FOV_STATUS = 3 or 4 depending on the illumination status.   If no sides are located on the asteroid FOV_STATUS = 0.

- Angular_Fluor_Fct   and   Angular_Scatter_Fct  –  surfaces:

AngularFluorFct(α,β)=i,jT(i,j)⋅(α)j(β)i;            α,β∈[0, 90) Degrees,                                                                                         except when α>65 AND β>65 degreesAngularFluorFct(α,β)=i, jT1(i,j)⋅((α−65)∗5)j((β−65)∗5)i;   when α> 65 AND β>65 degrees                                                                                                                                                                              If  AngularFluorFct(α,β)< 0.0, assume AngularFluorFct(α,β)=0.0                                           AngularFluorFct(90,β)=0.0    for  all  β                                           AngularFluorFct(α,90)=0.0    for  all  αAngularScatterFct(α,β)=2⋅Cos(α)(1+Cos(α)Cos(β));          α,β∈[0, 90] DegreesAngularScatterFct(90,90)=0.0                                                                                                                  α - incidence angle, counted from the normal to the surface, in Degreesβ - emission angle, counted from the normal to the surface, in Degrees

T(0,0)= 1.00172E+00	T(2,0)=-3.26751E-05	T(4,0)=2.42909E-08

T(0,1)=-5.64297E-04	T(2,1)=-8.04678E-06	T(4,1)=-4.14045E-09

T(0,2)=-1.73585E-05	T(2,2)=7.96751E-07	T(4,2)=4.06218E-10

T(0,3)=-2.17047E-06	T(2,3)=-2.74497E-08	T(4,3)=-1.41884E-11

T(0,4)=3.15083E-08	T(2,4)=4.04218E-10	T(4,4)=2.09849E-13

T(0,5)=-2.18812E-10	T(2,5)=-2.05539E-12	T(4,5)=-1.07269E-15

T(1,0)=-4.38375E-04	T(3,0)=-1.70545E-06	T(5,0)=-1.77067E-10

T(1,1)=8.20107E-05	T(3,1)=2.81293E-07	T(5,1)=2.10402E-11

T(1,2)=-7.99650E-06	T(3,2)=-2.74913E-08	T(5,2)=-2.07651E-12

T(1,3)=2.79160E-07	T(3,3)=9.59744E-10	T(5,3)=7.25547E-14

T(1,4)=-4.09545E-09	T(3,4)=-1.41387E-11	T(5,4)=-1.07835E-15

T(1,5)=2.07066E-11	T(3,5)=7.19243E-14	T(5,5)=5.55545E-18

T1(0,0)=4.22939E-01	T1(2,0)=-2.58902E-06	T1(4,0)=1.23481E-09

T1(0,1)=-1.63307E-03	T1(2,1)=-8.19620E-07	T1(4,1)=-2.18860E-10

T1(0,2)=-1.27089E-06	T1(2,2)=6.04984E-08	T1(4,2)=1.58527E-11

T1(0,3)=-1.81205E-07	T1(2,3)=-1.50287E-09	T1(4,3)=-3.97157E-13

T1(0,4)=1.61853E-09	T1(2,4)=1.58143E-11	T1(4,4)=4.21262E-15

T1(0,5)=-7.84793E-12	T1(2,5)=-5.75453E-14	T1(4,5)=-1.54659E-17

T1(1,0)=-1.67373E-03	T1(3,0)=-1.46340E-07	T1(5,0)=-6.17352E-12

T1(1,1)=2.35748E-05	T1(3,1)=2.10009E-08	T1(5,1)=7.78907E-13

T1(1,2)=-8.08433E-07	T1(3,2)=-1.50273E-09	T1(5,2)=-5.79691E-14

T1(1,3)=2.07575E-08	T1(3,3)=3.75675E-11	T1(5,3)=1.45610E-15

T1(1,4)=-2.15323E-10	T1(3,4)=-3.96237E-13	T1(5,4)=-1.55376E-17

T1(1,5)=7.61606E-13	T1(3,5)=1.44604E-15	T1(5,5)=5.75357E-20

Figure 2    Angular_Fluor_Fct(

APPENDIX 3    Location of XRS Level-2 Parameters in the U of A Database

## E.5.   NEAR XRS Science Housekeeping (SCI HK) Parameters:  53*(Integer*4)

| #### No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) X-Ray Spectrometer (XRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 0 | MET | ACQ_MET_TIME | All tables | All databases |
| 1 | UTC | ACQ_UTC_TIME | x_unf_spec_tab, x_mg_spec_tab, x_al_spec_tab, x_solar_spec_tab | near_xrs_db |
|  |  |  | xrs_hk_tab | near_hk_db |
| 2 | DQI | DQI | index_tab | near_xrs_db |
| 3 | TIME_TAG | ACQ_MET_TIME | All tables | All databases |
| 4 | UNF_MON_INTEG_TIME | INT_TIME | x_unf_spec_tab | near_xrs_db |
| 5 | MG_MON_INTEG_TIME | INT_TIME | x_mg_spec_tab | near_xrs_db |
| 6 | AL_MON_INTEG_TIME | INT_TIME | x_al_spec_tab | near_xrs_db |
| 7 | ACTV_SOL_MON_INTEG_TIME | INT_TIME | x_solar_spec_tab | near_xrs_db |
| 8 | UNF_XRAY_1_SAFE_FLG | LVL1_SENSOR_SAFING_FLAG | x_unf_spec_tab | near_xrs_db |
| 9 | MG_XRAY_1_SAFE_FLG | LVL1_SENSOR_SAFING_FLAG | x_mg_spec_tab | near_xrs_db |
| 10 | AL_XRAY_1_SAFE_FLG | LVL1_SENSOR_SAFING_FLAG | x_al_spec_tab | near_xrs_db |
| 11 | GAS_XRAY_1_SAFE_FLG | # GAS_LVL1_SENSOR_SAFING_FLAG | x_solar_spec_tab | near_xrs_db |
| 12 | PIN_XRAY_1_SAFE_FLG | PIN_LVL1_SENSOR_SAFING_FLAG | x_solar_spec_tab | near_xrs_db |
| 13 | UNF_XRAY_2_SAFE_FLG | LVL2_SENSOR_SAFING_FLAG | x_unf_spec_tab | near_xrs_db |
| 14 | MG_XRAY_2_SAFE_FLG | LVL2_SENSOR_SAFING_FLAG | x_mg_spec_tab | near_xrs_db |
| 15 | AL_XRAY_2_SAFE_FLG | LVL2_SENSOR_SAFING_FLAG | x_al_spec_tab | near_xrs_db |
| 16 | GAS_XRAY_2_SAFE_FLG | GAS_LVL2_SENSOR_SAFING_FLAG | x_solar_spec_tab | near_xrs_db |
| 17 | PIN_XRAY_2_SAFE_FLG | PIN_LVL2_SENSOR_SAFING_FLAG | x_solar_spec_tab | near_xrs_db |
| 18 | UNF_XRAY_OVERFLOW_FLG | SPEC_BOF | x_unf_spec_tab | near_xrs_db |
| 19 | MG_XRAY_OVERFLOW_FLG | SPEC_BOF | x_mg_spec_tab | near_xrs_db |
| 20 | AL_XRAY_OVERFLOW_FLG | SPEC_BOF | x_al_spec_tab | near_xrs_db |
| 21 | ACTIVE_XRAY_OVERFLOW_FLG | SPEC_BOF | x_solar_spec_tab | near_xrs_db |
| 22 | UNF_XRAY_RAW_EVENT_RATE | RAW_EVENT_RATE | x_unf_spec_tab | near_xrs_db |
| 23 | MG_XRAY_RAW_EVENT_RATE | RAW_EVENT_RATE | x_mg_spec_tab | near_xrs_db |
| 24 | AL_XRAY_RAW_EVENT_RATE | RAW_EVENT_RATE | x_al_spec_tab | near_xrs_db |
| 25 | GAS_XRAY_RAW_EVENT_RATE | GAS_RAW_EVENT_RATE | x_solar_spec_tab | near_xrs_db |
| 26 | PIN_XRAY_RAW_EVENT_RATE | PIN_RAW_EVENT_RATE | x_solar_spec_tab | near_xrs_db |
| 27 | UNF_XRAY_VALID_EVENTS | NUM_VALID_PROCESSED_EVENTS | x_unf_spec_tab | near_xrs_db |
| 28 | MG_XRAY_VALID_EVENTS | NUM_VALID_PROCESSED_EVENTS | x_mg_spec_tab | near_xrs_db |
| 29 | AL_XRAY_VALID_EVENTS | NUM_VALID_PROCESSED_EVENTS | x_al_spec_tab | near_xrs_db |
| 30 | GAS_XRAY_VALID_EVENTS | GAS_VALID_PROCESSED_EVENTS | x_solar_spec_tab | near_xrs_db |
| 31 | UNF_XRAY_EVENTS_REJ_BY_RT | NUM_EVENTS_REJ_BY_RTD | x_unf_spec_tab | near_xrs_db |
| 32 | MG_XRAY_EVENTS_REJ_BY_RT | NUM_EVENTS_REJ_BY_RTD | x_mg_spec_tab | near_xrs_db |
| 33 | AL_XRAY_EVENTS_REJ_BY_RT | NUM_EVENTS_REJ_BY_RTD | x_al_spec_tab | near_xrs_db |
| 34 | GAS_XRAY_EVENTS_REJ_BY_RT | NUM_EVENTS_REJ_BY_RTD | x_solar_spec_tab | near_xrs_db |
| 35 | PIN_SOLAR_PROCESS_EVENT | PIN_SOLAR_PROCESSED_EVENTS | x_unf_spec_tab | near_xrs_db |
| 36 | XTALK_COUNTER | XTALK_COUNTER | x_mg_spec_tab | near_xrs_db |
| 37 | CMD_OPCODE | # CMD_OPCODE | x_al_spec_tab | near_xrs_db |
| 38 | CMD_ARGUMENT | CMD_ARGUMENT | x_al_spec_tab | near_xrs_db |
| 39 | CMDS_RECEIVED | CMDS_RECD | x_solar_spec_tab | near_xrs_db |
| 40 | PROBLEM_COUNTER | PROBLEM_COUNTER | x_solar_spec_tab | near_xrs_db |
| 41 | PROBLEM_CODE | PROBLEM_CODE | x_solar_spec_tab | near_xrs_db |
| 42 | UNF_XRAY_DC_OFFSET | DC_OFFSET_EST | x_unf_spec_tab | near_xrs_db |
| 43 | MG_XRAY_DC_OFFSET | DC_OFFSET_EST | x_mg_spec_tab | near_xrs_db |
| 44 | AL_XRAY_DC_OFFSET | DC_OFFSET_EST | x_al_spec_tab | near_xrs_db |
| 45 | GAS_XRAY_DC_OFFSET | GAS_DC_OFFSET_EST | x_solar_spec_tab | near_xrs_db |
| 46 | PIN_DC_OFFSET | PIN_DC_OFFSET_EST | x_solar_spec_tab | near_xrs_db |
| 47 | UNF_STAND_DEV | STD_DEV | x_unf_spec_tab | near_xrs_db |
| 48 | MG_STAND_DEV | STD_DEV | x_mg_spec_tab | near_xrs_db |
| 49 | AL_STAND_DEV | STD_DEV | x_al_spec_tab | near_xrs_db |
| 50 | GAS_STAND_DEV | GAS_STD_DEV | x_solar_spec_tab | near_xrs_db |
| 51 | PIN_STAND_DEV | PIN_STD_DEV | x_solar_spec_tab | near_xrs_db |
| 52 | XRAY_DATA_QUALITY_BIT | XRAY_DQI | x_unf_spec_tab | near_xrs_db |

## E.6. NEAR XRS Engineering Housekeeping (ENG HK) Parameters: 163*(REAL*4)

| #### No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) X-Ray Spectrometer (XRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 0 | DETECTOR_ELECTR_CURR | DETECTOR_ELECTR_CURR | xrs_hk_tab | near_hk_db |
| 1 | XRAY_PIN_DIODE_FET_BIAS | XRAY_PIN_DIODE_FET_BIAS | xrs_hk_tab | near_hk_db |
| 2 | HVCE_TEMPERATURE | HVCE_TEMPERATURE | xrs_hk_tab | near_hk_db |
| 3 | UNF_XRAY_HVPS_VOLT | UNF_XRAY_HVPS_VOLT | xrs_hk_tab | near_hk_db |
| 4 | MG_XRAY_HVPS_VOLT | MG_XRAY_HVPS_VOLT | xrs_hk_tab | near_hk_db |
| 5 | AL_XRAY_HVPS_VOLT | AL_XRAY_HVPS_VOLT | xrs_hk_tab | near_hk_db |
| 6 | XRAY_GAS_HVPS_VOLT | XRAY_GAS_HVPS_VOLT | xrs_hk_tab | near_hk_db |
| 7 | XRAY_PIN_HVPS_VOLT | XRAY_PIN_HVPS_VOLT | xrs_hk_tab | near_hk_db |
| 8 | NAI_HVPS_VOLT | NAI_HVPS_VOLT | xrs_hk_tab | near_hk_db |
| 9 | BGO_HVPS_VOLT | BGO_HVPS_VOLT | xrs_hk_tab | near_hk_db |
| 10 | DPU_DC_DC_TEMP | DPU_DC_DC_TEMP | xrs_hk_tab | near_hk_db |
| 11 | XGRS_DC_DC_TEMP | XGRS_DC_DC_TEMP | xrs_hk_tab | near_hk_db |
| 12 | UNF_XRAY_HVPS_TEMP | UNF_XRAY_HVPS_TEMP | xrs_hk_tab | near_hk_db |
| 13 | MG_XRAY_HVPS_TEMP | MG_XRAY_HVPS_TEMP | xrs_hk_tab | near_hk_db |
| 14 | AL_XRAY_HVPS_TEMP | AL_XRAY_HVPS_TEMP | xrs_hk_tab | near_hk_db |
| 15 | XRAY_GAS_HVPS_TEMP | XRAY_GAS_HVPS_TEMP | xrs_hk_tab | near_hk_db |
| 16 | XRAY_PIN_HVPS_TEMP | XRAY_PIN_HVPS_TEMP | xrs_hk_tab | near_hk_db |
| 17 | NAI_HVPS_TEMP | NAI_HVPS_TEMP | xrs_hk_tab | near_hk_db |
| 18 | BGO_HVPS_TEMP | BGO_HVPS_TEMP | xrs_hk_tab | near_hk_db |
| 19 | TEC_CURRENT | TEC_CURRENT | xrs_hk_tab | near_hk_db |
| 20 | NAI_HVPS_VOLT_CMD | NAI_HVPS_VOLT_CMD | xrs_hk_tab | near_hk_db |
| 21 | BGO_HVPS_VOLT_CMD | BGO_HVPS_VOLT_CMD | xrs_hk_tab | near_hk_db |
| 22 | UNF_HVPS_VOLT_CMD | UNF_HVPS_VOLT_CMD | xrs_hk_tab | near_hk_db |
| 23 | MG_HVPS_VOLT_CMD | MG_HVPS_VOLT_CMD | xrs_hk_tab | near_hk_db |
| 24 | AL_HVPS_VOLT_CMD | AL_HVPS_VOLT_CMD | xrs_hk_tab | near_hk_db |
| 25 | HVPS_POWER_ON_OFF | HVPS_POWER_ON_OFF | xrs_hk_tab | near_hk_db |
| 26 | XRAY_CALIB_HOME_FLG | XRAY_CALIB_HOME_FLG | xrs_hk_tab | near_hk_db |
| 27 | XRAY_CALIB_MOTOR_ON_OFF | XRAY_CALIB_MOTOR_ON_OFF | xrs_hk_tab | near_hk_db |
| 28 | XRAY_CALIB_MOTOR_ENABLE | XRAY_CALIB_MOTOR_ENABLE | xrs_hk_tab | near_hk_db |
| 29 | GAS_HVPS_VOLT_CMD | GAS_HVPS_VOLT_CMD | xrs_hk_tab | near_hk_db |
| 30 | PIN_HVPS_VOLT_CMD | PIN_HVPS_VOLT_CMD | xrs_hk_tab | near_hk_db |
| 31 | GAMMA_RAY_HEATER_CURR | GAMMA_RAY_HEATER_CURR | xrs_hk_tab | near_hk_db |
| 32 | Spare0 | Spare0 | xrs_hk_tab | near_hk_db |
| 33 | XRAY_CALIB_MOTOR_DIR | XRAY_CALIB_MOTOR_DIR | xrs_hk_tab | near_hk_db |
| 34 | XRAY_CALIB_MOTOR_CURR | XRAY_CALIB_MOTOR_CURR | xrs_hk_tab | near_hk_db |
| 35 | XRAY_CALIB_MOTOR_GOAL | XRAY_CALIB_MOTOR_GOAL | xrs_hk_tab | near_hk_db |
| 36 | XRAY_CALIB_MOTOR_POS | XRAY_CALIB_MOTOR_POS | xrs_hk_tab | near_hk_db |
| 37 | XRAY_CALIB_MOTOR_FID_SENS | XRAY_CALIB_MOTOR_FID_SENS | xrs_hk_tab | near_hk_db |
| 38 | XRAY_CALIB_MOTOR_FID_BRIT | XRAY_CALIB_MOTOR_FID_BRIT | xrs_hk_tab | near_hk_db |
| 39 | XRAY_MOTOR_IN_CALIB_POSITION | XRAY_MOTOR_IN_CALIB_POSITION | xrs_hk_tab | near_hk_db |
| 40 | XRAY_MOTOR_IN_NORM_POSITION | XRAY_MOTOR_IN_NORM_POSITION | xrs_hk_tab | near_hk_db |
| 41 | ACTIVE_SOLAR_TOGGLE_MODE | ACTIVE_SOLAR_TOGGLE_MODE | xrs_hk_tab | near_hk_db |
| 42 | XRAY_CALIB_MAX_STEPS | XRAY_CALIB_MAX_STEPS | xrs_hk_tab | near_hk_db |
| 43 | spare1 | spare1 | xrs_hk_tab | near_hk_db |
| 44 | TEC_ENABLE | TEC_ENABLE | xrs_hk_tab | near_hk_db |
| 45 | XRAY_CALIB_CUM_MOTOR_STPS | XRAY_CALIB_CUM_MOTOR_STPS | xrs_hk_tab | near_hk_db |
| 46 | GRAY_BURST_THRESH_VAL | GRAY_BURST_THRESH_VAL | xrs_hk_tab | near_hk_db |
| 47 | XRAY_SAFING_THRESH | XRAY_SAFING_THRESH | xrs_hk_tab | near_hk_db |
| 48 | XRAY_CALIB_MOTOR_DIAG | XRAY_CALIB_MOTOR_DIAG | xrs_hk_tab | near_hk_db |
| 49 | TEC_POWER_ON_OFF | TEC_POWER_ON_OFF | xrs_hk_tab | near_hk_db |
| 50 | TEC_MODE | TEC_MODE | xrs_hk_tab | near_hk_db |
| 51 | TEC_COOL_HEAT_FLG | TEC_COOL_HEAT_FLG | xrs_hk_tab | near_hk_db |
| 52 | TEC_TEMPERATURE | TEC_TEMPERATURE | xrs_hk_tab | near_hk_db |
| 53 | TEC_COOL_MODE_TEMP_UP_LMT | TEC_COOL_MODE_TEMP_UP_LMT | xrs_hk_tab | near_hk_db |
| 54 | TEC_COOL_MODE_TEMP_LOW_LMT | TEC_COOL_MODE_TEMP_LOW_LMT | xrs_hk_tab | near_hk_db |
| 55 | TEC_HEAT_MODE_TEMP_UP_LMT | TEC_HEAT_MODE_TEMP_UP_LMT | xrs_hk_tab | near_hk_db |
| 56 | TEC_HEAT_MODE_TEMP_LOW_LMT | TEC_HEAT_MODE_TEMP_LOW_LMT | xrs_hk_tab | near_hk_db |
| 57 | GAMMA_RAY_TEMP_HYSTERESIS | GAMMA_RAY_TEMP_HYSTERESIS | xrs_hk_tab | near_hk_db |
| 58 | GAMMA_RAY_HEAT_TEMP_LOW_LMT | GAMMA_RAY_HEAT_TEMP_LOW_LMT | xrs_hk_tab | near_hk_db |
| 59 | FULL_GAMMA_RAY_SCIENCE_MODE | FULL_GAMMA_RAY_SCIENCE_MODE | xrs_hk_tab | near_hk_db |
| 60 | FULL_XRAY_SCIENCE_MODE | FULL_XRAY_SCIENCE_MODE | xrs_hk_tab | near_hk_db |
| 61 | SUMM_GRAY_XRAY_SCIENCE_MODE | SUMM_GRAY_XRAY_SCIENCE_MODE | xrs_hk_tab | near_hk_db |
| 62 | GRAY_BURST_SCI_REC_MODE | GRAY_BURST_SCI_REC_MODE | xrs_hk_tab | near_hk_db |
| 63 | HVCE_TEMP_SET_POINT | HVCE_TEMP_SET_POINT | xrs_hk_tab | near_hk_db |
| 64 | CMDED_END_XRAY_INTEG_PER | CMDED_END_XRAY_INTEG_PER | xrs_hk_tab | near_hk_db |
| 65 | CMDED_GAMMA_RAY_INTEG_PER | CMDED_GAMMA_RAY_INTEG_PER | xrs_hk_tab | near_hk_db |
| 66 | BGO_PROTON_FLARE_FLG | BGO_PROTON_FLARE_FLG | xrs_hk_tab | near_hk_db |
| 67 | GAMMA_RAY_BIN_OVER_FLG_CMD | GAMMA_RAY_BIN_OVER_FLG_CMD | xrs_hk_tab | near_hk_db |
| 68 | UNF_XRAY_BIN_OVER_FLG_CMD | UNF_XRAY_BIN_OVER_FLG_CMD | xrs_hk_tab | near_hk_db |
| 69 | MG_BIN_OVER_FLG_CMD | MG_BIN_OVER_FLG_CMD | xrs_hk_tab | near_hk_db |
| 70 | AL_BIN_OVER_FLG_CMD | AL_BIN_OVER_FLG_CMD | xrs_hk_tab | near_hk_db |
| 71 | ACT_BIN_OVER_FLG_CMD | ACT_BIN_OVER_FLG_CMD | xrs_hk_tab | near_hk_db |
| 72 | NAI_BIN_OVERFLOW_FLG | NAI_BIN_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 73 | BGO_BIN_OVERFLOW_FLG | BGO_BIN_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 74 | NAI_ANTICOIN_OVERFLOW_FLG | NAI_ANTICOIN_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 75 | NAI_SING_ESC_OVERFLOW_FLG | NAI_SING_ESC_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 76 | NAI_DOUB_ESC_OVERFLOW_FLG | NAI_DOUB_ESC_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 77 | BGO_SING_ESC_OVERFLOW_FLG | BGO_SING_ESC_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 78 | BGO_DOUB_ESC_OVERFLOW_FLG | BGO_DOUB_ESC_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 79 | UNF_XRAY_BIN_OVERFLOW_FLG | UNF_XRAY_BIN_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 80 | MG_BIN_OVERFLOW_FLG | MG_BIN_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 81 | AL_BIN_OVERFLOW_FLG | AL_BIN_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 82 | ACT_BIN_OVERFLOW_FLG | ACT_BIN_OVERFLOW_FLG | xrs_hk_tab | near_hk_db |
| 83 | GAMMA_RAY_HEAT_ON_OFF | GAMMA_RAY_HEAT_ON_OFF | xrs_hk_tab | near_hk_db |
| 84 | GAMMA_RAY_HEAT_MODE | GAMMA_RAY_HEAT_MODE | xrs_hk_tab | near_hk_db |
| 85 | BGO_LEVEL_1_SAFING_FLG | BGO_LEVEL_1_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 86 | BGO_LEVEL_2_SAFING_FLG | BGO_LEVEL_2_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 87 | spare3 | spare3 | xrs_hk_tab | near_hk_db |
| 88 | XRAY_INTEGR_TIME | XRAY_INTEGR_TIME | xrs_hk_tab | near_hk_db |
| 89 | HVPS_ON_OFF | HVPS_ON_OFF | xrs_hk_tab | near_hk_db |
| 90 | PIN_SENSOR_MASK_FLG | PIN_SENSOR_MASK_FLG | xrs_hk_tab | near_hk_db |
| 91 | GAS_SENSOR_MASK_FLG | GAS_SENSOR_MASK_FLG | xrs_hk_tab | near_hk_db |
| 92 | AL_SENSOR_MASK_FLG | AL_SENSOR_MASK_FLG | xrs_hk_tab | near_hk_db |
| 93 | MG_SENSOR_MASK_FLG | MG_SENSOR_MASK_FLG | xrs_hk_tab | near_hk_db |
| 94 | UNF_SENSOR_MASK_FLG | UNF_SENSOR_MASK_FLG | xrs_hk_tab | near_hk_db |
| 95 | BGO_SENSOR_MASK_FLG | BGO_SENSOR_MASK_FLG | xrs_hk_tab | near_hk_db |
| 96 | NAI_SENSOR_MASK_FLG | NAI_SENSOR_MASK_FLG | xrs_hk_tab | near_hk_db |
| 97 | UNF_XRAY_RISE_TIME_THRESH | UNF_XRAY_RISE_TIME_THRESH | xrs_hk_tab | near_hk_db |
| 98 | MG_RISE_TIME_THRESH | MG_RISE_TIME_THRESH | xrs_hk_tab | near_hk_db |
| 99 | AL_RISE_TIME_THRESH | AL_RISE_TIME_THRESH | xrs_hk_tab | near_hk_db |
| 100 | GAS_RISE_TIME_THRESH | GAS_RISE_TIME_THRESH | xrs_hk_tab | near_hk_db |
| 101 | UNF_RISE_TIME_VALID_THRESH | UNF_RISE_TIME_VALID_THRESH | xrs_hk_tab | near_hk_db |
| 102 | MG_RISE_TIME_VALID_THRESH | MG_RISE_TIME_VALID_THRESH | xrs_hk_tab | near_hk_db |
| 103 | AL_RISE_TIME_VALID_THRESH | AL_RISE_TIME_VALID_THRESH | xrs_hk_tab | near_hk_db |
| 104 | GAS_RISE_TIME_VALID_THRESH | GAS_RISE_TIME_VALID_THRESH | xrs_hk_tab | near_hk_db |
| 105 | GAMMA_RAY_INTEGR_TIME | GAMMA_RAY_INTEGR_TIME | xrs_hk_tab | near_hk_db |
| 106 | GAMMA_SING_ESC_WIND_CENTER | GAMMA_SING_ESC_WIND_CENTER | xrs_hk_tab | near_hk_db |
| 017 | GAMMA_SING_ESC_WIND_WIDTH | GAMMA_SING_ESC_WIND_WIDTH | xrs_hk_tab | near_hk_db |
| 108 | GAMMA_DOUB_ESC_WIND_CENTER | GAMMA_DOUB_ESC_WIND_CENTER | xrs_hk_tab | near_hk_db |
| 109 | GAMMA_DOUB_ESC_WIND_WIDTH | GAMMA_DOUB_ESC_WIND_WIDTH | xrs_hk_tab | near_hk_db |
| 110 | NAI_LOW_LEVEL_AMPL_THRESH | NAI_LOW_LEVEL_AMPL_THRESH | xrs_hk_tab | near_hk_db |
| 111 | BGO_LOW_LEVEL_AMPL_THRESH | BGO_LOW_LEVEL_AMPL_THRESH | xrs_hk_tab | near_hk_db |
| 112 | UNF_LOW_LEVEL_AMPL_THRESH | UNF_LOW_LEVEL_AMPL_THRESH | xrs_hk_tab | near_hk_db |
| 113 | MG_LOW_LEVEL_AMPL_THRESH | MG_LOW_LEVEL_AMPL_THRESH | xrs_hk_tab | near_hk_db |
| 114 | AL_LOW_LEVEL_AMPL_THRESH | AL_LOW_LEVEL_AMPL_THRESH | xrs_hk_tab | near_hk_db |
| 115 | GAS_LOW_LEVEL_AMPL_THRESH | GAS_LOW_LEVEL_AMPL_THRESH | xrs_hk_tab | near_hk_db |
| 116 | PIN_LOW_LEVEL_AMPL_THRESH | PIN_LOW_LEVEL_AMPL_THRESH | xrs_hk_tab | near_hk_db |
| 117 | GAMMA_RAY_SENSOR_TEMP | GAMMA_RAY_SENSOR_TEMP | xrs_hk_tab | near_hk_db |
| 118 | NAI_LEVEL_1_SAFING_FLG | NAI_LEVEL_1_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 119 | UNF_LEVEL_1_SAFING_FLG | UNF_LEVEL_1_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 120 | MG_LEVEL_1_SAFING_FLG | MG_LEVEL_1_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 121 | AL_LEVEL_1_SAFING_FLG | AL_LEVEL_1_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 122 | GAS_LEVEL_1_SAFING_FLG | GAS_LEVEL_1_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 123 | PIN_LEVEL_1_SAFING_FLG | PIN_LEVEL_1_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 124 | NAI_LEVEL_2_SAFING_FLG | NAI_LEVEL_2_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 125 | UNF_LEVEL_2_SAFING_FLG | UNF_LEVEL_2_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 126 | MG_LEVEL_2_SAFING_FLG | MG_LEVEL_2_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 127 | AL_LEVEL_2_SAFING_FLG | AL_LEVEL_2_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 128 | GAS_LEVEL_2_SAFING_FLG | GAS_LEVEL_2_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 129 | PIN_LEVEL_2_SAFING_FLG | PIN_LEVEL_2_SAFING_FLG | xrs_hk_tab | near_hk_db |
| 130 | XRAY_SENS_SAFING_REST_LEVEL | XRAY_SENS_SAFING_REST_LEVEL | xrs_hk_tab | near_hk_db |
| 131 | GAMMA_SENS_SAFING_REST_LEVEL | GAMMA_SENS_SAFING_REST_LEVEL | xrs_hk_tab | near_hk_db |
| 132 | XRAY_SAFING_REST_MAX_RETRY | XRAY_SAFING_REST_MAX_RETRY | xrs_hk_tab | near_hk_db |
| 133 | GAMMA_SAFING_REST_MAX_RETRY | GAMMA_SAFING_REST_MAX_RETRY | xrs_hk_tab | near_hk_db |
| 134 | GAMMA_HEATER_DUTY_CYCLE | GAMMA_HEATER_DUTY_CYCLE | xrs_hk_tab | near_hk_db |
| 135 | GAMMA_SENS_SAFING_THRESH | GAMMA_SENS_SAFING_THRESH | xrs_hk_tab | near_hk_db |
| 136 | HVPS_CURRENT | HVPS_CURRENT | xrs_hk_tab | near_hk_db |
| 137 | BGO_PROTON_FLARE_THRESH | BGO_PROTON_FLARE_THRESH | xrs_hk_tab | near_hk_db |
| 138 | VERSION_NUMBER | VERSION_NUMBER | xrs_hk_tab | near_hk_db |
| 139 | CLEAR_SPECTRUM_0 | SPECTRUM_0 | x_unf_spec_tab | near_xrs_db |
| 140 | CLEAR_HW_RAW | HW_RAW | x_unf_spec_tab | near_xrs_db |
| 141 | CLEAR_HW_RISE_TIME_REJECT | HW_RISE_TIME_REJ | x_unf_spec_tab | near_xrs_db |
| 142 | CLEAR_FALSE_RISE_TIME_REJECT | FALSE_RISE_TIME_REJ | x_unf_spec_tab | near_xrs_db |
| 143 | CLEAR_TRUE_RISE_TIME_REJECT | TRUE_RISE_TIME_REJ | x_unf_spec_tab | near_xrs_db |
| 144 | CLEAR_HIGH_ENERGY | HIGH_ENERGY | x_unf_spec_tab | near_xrs_db |
| 145 | MG_SPECTRUM_0 | SPECTRUM_0 | x_mg_spec_tab | near_xrs_db |
| 146 | MG_HW_RAW | HW_RAW | x_mg_spec_tab | near_xrs_db |
| 147 | MG_HW_RISE_TIME_REJECT | HW_RISE_TIME_REJ | x_mg_spec_tab | near_xrs_db |
| 148 | MG_FALSE_RISE_TIME_REJECT | FALSE_RISE_TIME_REJ | x_mg_spec_tab | near_xrs_db |
| 149 | MG_TRUE_RISE_TIME_REJECT | TRUE_RISE_TIME_REJ | x_mg_spec_tab | near_xrs_db |
| 150 | MG_HIGH_ENERGY | HIGH_ENERGY | x_mg_spec_tab | near_xrs_db |
| 151 | AL_SPECTRUM_0 | SPECTRUM_0 | x_al_spec_tab | near_xrs_db |
| 152 | AL_HW_RAW | HW_RAW | x_al_spec_tab | near_xrs_db |
| 153 | AL_HW_RISE_TIME_REJECT | HW_RISE_TIME_REJ | x_al_spec_tab | near_xrs_db |
| 154 | AL_FALSE_RISE_TIME_REJECT | FALSE_RISE_TIME_REJ | x_al_spec_tab | near_xrs_db |
| 155 | AL_TRUE_RISE_TIME_REJECT | TRUE_RISE_TIME_REJ | x_al_spec_tab | near_xrs_db |
| 156 | AL_HIGH_ENERGY | HIGH_ENERGY | x_al_spec_tab | near_xrs_db |
| 157 | SOLAR_SPECTRUM_0 | SPECTRUM_0 | x_solar_spec_tab | near_xrs_db |
| 158 | SOLAR_HW_RAW | HW_RAW | x_solar_spec_tab | near_xrs_db |
| 159 | SOLAR_HW_RISE_TIME_REJECT | HW_RISE_TIME_REJ | x_solar_spec_tab | near_xrs_db |
| 160 | SOLAR_FALSE_RISE_TIME_REJECT | FALSE_RISE_TIME_REJ | x_solar_spec_tab | near_xrs_db |
| 161 | SOLAR_TRUE_RISE_TIME_REJECT | TRUE_RISE_TIME_REJ | x_solar_spec_tab | near_xrs_db |
| 162 | SOLAR_HIGH_ENERGY | HIGH_ENERGY | x_solar_spec_tab | near_xrs_db |

### E.7. NEAR XRS Derived Engineering (DER ENG) Parameters:  28*(Real*4), 1*(Integer*4), 4*(Integer*4)

| #### No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) X-Ray Spectrometer (XRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 6 | REAL_GAIN_UNF | UNFIL_GAIN | xrs_calib_tab | near_xrs_db |
| 7 | REAL_GAIN_MG | MGFIL_GAIN | xrs_calib_tab | near_xrs_db |
| 8 | REAL_GAIN_AL | ALFIL_GAIN | xrs_calib_tab | near_xrs_db |
| 9 | REAL_ZERO_UNF | UNF_ZERO | xrs_calib_tab | near_xrs_db |
| 10 | REAL_ZERO_MG | MGFIL_ZERO | xrs_calib_tab | near_xrs_db |
| 11 | REAL_ZERO_AL | ALFIL_ZERO | xrs_calib_tab | near_xrs_db |
| 28 | BAD_FLAG | BAD_CODE | xrs_calib_tab | near_xrs_db |

E.8.  NEAR XRS Spatial (SPATIAL) Parameters:  1*(Integer*4), 38*(Real*4)

| No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) X-Ray Spectrometer (XRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 0 | #### MET | ACQ_MET_TIME | s_xrs_spatial_tab | near_spatial_db |
| 1 | #### PLATEID_BORESITE_INTERSECT | BSIGHT_PLATE_ID | s_xrs_spatial_tab | near_spatial_db |
| 2 | #### BINID_BORESIGHT_INTERSECT | BSIGHT_BIN_ID | s_xrs_spatial_tab | near_spatial_db |
| 3 | #### SC_POSITION, X | SC_POS_X | s_xrs_spatial_tab | near_spatial_db |
| 4 | #### SC_POSITION, Y | SC_POS_Y | s_xrs_spatial_tab | near_spatial_db |
| 5 | #### SC_POSITION, Z | SC_POS_Z | s_xrs_spatial_tab | near_spatial_db |
| 6 | #### BS_VECTOR, X | BSIGHT_X | s_xrs_spatial_tab | near_spatial_db |
| 7 | #### BS_VECTOR, Y | BSIGHT_Y | s_xrs_spatial_tab | near_spatial_db |
| 8 | #### BS_VECTOR, Z | BSIGHT_Z | s_xrs_spatial_tab | near_spatial_db |
| 9 | #### EARTH_POSITION,X | EARTH_POS_X | s_xrs_spatial_tab | near_spatial_db |
| 10 | #### EARTH_POSITION,Y | EARTH_POS_Y | s_xrs_spatial_tab | near_spatial_db |
| 11 | #### EARTH_POSITION,Z | EARTH_POS_Z | s_xrs_spatial_tab | near_spatial_db |
| 12 | #### SUN_POSITION,X | SUN_POS_X | s_xrs_spatial_tab | near_spatial_db |
| 13 | #### SUN_POSITION,Y | SUN_POS_Y | s_xrs_spatial_tab | near_spatial_db |
| 14 | #### SUN_POSITION,Z | SUN_POS_Z | s_xrs_spatial_tab | near_spatial_db |
| 15 | #### FOV_STATUS | FOV_STATUS | s_xrs_spatial_tab | near_spatial_db |
| 16 | #### DOWNLINK_STATUS | DOWNLINK_FLAG | s_xrs_spatial_tab | near_spatial_db |
| 17 | #### ANGLE_SUN_SOLAR_MON | SOLAR_MON_ANGLE | s_xrs_spatial_tab | near_spatial_db |
| 18 | #### AVG_SC_DISTANCE | AVG_SC_DISTANCE | s_xrs_spatial_tab | near_spatial_db |
| 19 | #### SUN_DISTANCE | SUN_DISTANCE | s_xrs_spatial_tab | near_spatial_db |
| 20 | #### AVG_EMISSION_ANGLE | AVG_EMI_ANGLE | s_xrs_spatial_tab | near_spatial_db |
| 21 | #### AVG_INCIDENCE_ANGLE | AVG_INC_ANGLE | s_xrs_spatial_tab | near_spatial_db |
| 22 | #### PHASE_ANGLE | PHASE_ANGLE | s_xrs_spatial_tab | near_spatial_db |
| 23 | #### AVG_INC_EMI_RATIO | AVG_INC_EMI_COS_RATIO | s_xrs_spatial_tab | near_spatial_db |
| 24 | #### SPATIAL_FLUORESCENCE_NORM | SPATIAL_FLUORESCENCE | s_xrs_spatial_tab | near_spatial_db |
| 25 | #### SPATIAL_SCATTERING_NORM | SPATIAL_SCATTERING_NORM | s_xrs_spatial_tab | near_spatial_db |
| 26 | #### TOTAL_AREA_FOOTPRINT | TOT_AREA | s_xrs_spatial_tab | near_spatial_db |
| 27 | #### TOTAL_ILLUMINATED_AREA | TOT_ILLUM_AREA | s_xrs_spatial_tab | near_spatial_db |
| 28 | #### TOTAL_VISIBLE_AREA | TOT_VIS_AREA | s_xrs_spatial_tab | near_spatial_db |
| 29 | #### TOTAL_EFF_AREA | TOT_EFF_AREA | s_xrs_spatial_tab | near_spatial_db |
| 30 | #### TOTAL_EFF_SOLID_ANGLE | TOT_EFF_SOLID_ANGLE | s_xrs_spatial_tab | near_spatial_db |
| 31 | #### FOOTPRINT_SOLID_ANGLE | TOT_SOLID_ANGLE | s_xrs_spatial_tab | near_spatial_db |
| 32 | #### BORESIGHT_INTERSECTION_LAT | BSIGHT_INTSECT_LAT | s_xrs_spatial_tab | near_spatial_db |
| 33 | #### BORESIGHT_INTERSECTION_LON | BSIGHT_INTSECT_LON | s_xrs_spatial_tab | near_spatial_db |
| 34 | #### BORESIGHT_INTERSECTION_R | BSIGHT_INTSECT_R | s_xrs_spatial_tab | near_spatial_db |
| 35 | #### SPICE_KERNEL_ID | SPICE_ID | s_xrs_spatial_tab | near_spatial_db |
| 36 | #### PLATE_MODEL_ID | PLATE_MODEL_ID | s_xrs_spatial_tab | near_spatial_db |
| 37 | #### PROCESSING_SOFTWARE_ID | SPATIAL_SOFTWARE_ID | s_xrs_spatial_tab | near_spatial_db |
| 38 | #### DATABASE_VERSION_ID | DB_VERSION | s_xrs_spatial_tab | near_spatial_db |

### E.9.  NEAR XRS Solar (SOLAR) Parameters: 2*(Real*4)

| No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) X-Ray Spectrometer (XRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 0 | SOLAR_INTEGRAL | SOLAR | x_solar_spec_tab | near_xrs_db |

G.  Listing of Parameters for XRS Fractional Footprint Accounting System

| No | #### PARAMETER NAME IN “Requirements Specifications For Level-2 Parameters Required Stored or Derived by The Near Earth Redezvous (NEAR) X-Ray Spectrometer (XRS) Ground System” | # PARAMETER NAME IN # UA DATABASE | TABLE NAME | DATABASE NAME |
| --- | --- | --- | --- | --- |
| 0 | #### MET | ACQ_MET_TIME | s_xrs_overlap_tab | near_spatial_db |
| 1 | #### FOOTPRINT_BIN_TOTAL_AREA | TOT_AREA | s_xrs_overlap_tab | near_spatial_db |
| 2 | #### FOOTPRINT_BIN_ILLUM_AREA | TOT_ILLUM_AREA | s_xrs_overlap_tab | near_spatial_db |
| 3 | #### FOOTPRINT_BIN_VISIBLE_AREA | TOT_VIS_AREA | s_xrs_overlap_tab | near_spatial_db |
| 4 | #### FOOTPRINT_BIN_EFF_AREA | TOT_EFF_AREA | s_xrs_overlap_tab | near_spatial_db |
| 5 | #### TOTAL_EFF_SOLID_ANGLE_FRACTION | SOLID_ANG_FRACTION | s_xrs_overlap_tab | near_spatial_db |
| 6 | #### SPECTRUM_FRACTION | SPECTRUM_FRACTION | s_xrs_overlap_tab | near_spatial_db |
| 7 | #### BINID | BIN_ID | s_xrs_overlap_tab | near_spatial_db |

NEAR XRS Requirements Specs Revision Notes

NEAR XGRS Team Meeting

September 15, 1999

(Version 7 Upgrades)

XRS Document Revisions:

1. Pg 3      Removed 2 Eng_hk redundant parameters from record:

Formats 1 and 2:

Eng_HK changed from 165 to 163 total parameters.

Two parameters:

gray_burst_thresh_val:   Partition Offset (46,139) kept at 46

gray_burst_sci_rec_mode: positions (62,140) kept at 62

Related: Forced changes in E.6 title (pg. 7) and list

(Pg. 10), H.6 (title) pg.15

Modified SCIHK also for new V7 total parameters

2. Pg 6-7    Added 9 SCI_HK parameters E.5 and labeled for

version 7 sofware upgrade.

3. Pg 7-10   Added 25 ENG_HK parameters E.6 and labeled for

version 7 software upgrade.

4. Pg 10     E.7 BADID and QUERYID is a total 5*(I*4) field in Title

5. Pg 14     Added  7 BINID to Fractional Footprint Accounting (Section G)

6. Pg 15     Modified Live Time equations for MG,AL,UNF,Solar   (Derived Eng) R.

Starr

7. Pg 17     Bsite vectors changed from km to unitless

8. Pg 22     Added comment * Plate solid angle modified by plate fraction

in FOV. (JSB)

9. Pg 24     Modified Equation for V.

Removed factor 3*SQRT(Solar_Integral*300)

Added Line:

V values will be modified and subject to change:  -->

Dynamically loaded at Runtime.

10. Pg 37    Added elaboration of 7 BINID to section J.

Fractional Footprint Accounting System

11. Pg 42    Added Comment * Currently testing method using fractional

FOV boundaries.

12. Pg 43 	 Modified Angular_Fluor_Fct and Angular_Scat_Fct derivations

October 18, 1999  Following revision is suggested.

1.  Pg.3		Format 1: tailing parameter is added to the description of valid

Derived Eng. Record parameters.

2.  Pg.11	E.7.: Paramaters 22-24 are renamed.

Parameters 25-27 are removed.

Parameter 25 is set as TAILING_PARAMETER.

Parameter 26-27: spare.

3. Pg.11-12 E.8.: Parameters 18,20,30 are made valid for FOV_STATUS=2,4 (Darkside)

4. Pg.13-14	F.10: Param.0-1,8-9  “not valid IF FOV_STATUS=0” added,

Param.2-4,10-12 “not valid IF FOV_STATUS=0,2,4” added,

Param.5,13,16  “=0 IF FOV_STATUS=0” added,

Param.6-7,14-15 “=0 IF FOV_STATUS=0,2,4” added

5. Pg.14	  G.:	Param.5: “=0 IF FOV_STATUS=2,4” is removed in acc. with note 3.

6. Pg.15-16	H.7.: In description of param.12-15 eng.param.names are adjusted.

7. Pg.16	H.7.: Param.16,19-21 definition is modified in accordance with new

spectrum limits. Derivation algorith is added.

8. Pg.16	H.7.: Param.22-24 are redefined and renamed accordingly to revision

note 2.

Param.25 is set as TAILING_PARAMETER.

9. Pg.17	H.7.:	Param.26-27 are removed, now spare.

10. Pg.17	H.8.: In param.3-14 description light time correction NEAR-Eros is

removed.

11. Pg.18,21 H.8.: Param.18,20,30 are redefined to characterise Darkside as well.

12. Pg.19-22 H.8.: Param.21-25,28-31: Added comment “Fractions of the plates are

considered”.

13. Pg.22	H.8.: Param.33 – specifications for Boresight_Intersection_Lon are

changed.

14. Pg.23	H.9.: For param.0 upper limit of integration is changed to 253 in

accordance with new spectrum limits.

15.Pg.28,29	I.9.: Insignificant changes in description of param.14-19,35-40

16.Pg.28-30 I.9.: Param.20-25,41-46 description is modified in accordance with

modifications to param.22-24 of Der.Eng. (revision notes 2,8).

17.Pg.30,33 I.10: Param.0,8 description is modified in accordance with new

definitions of param.18 of Spatial record (revision note 11).

18. Pg.36-37  J.: Param.3-6: Added comment “Fractions of the plates are

considered”.

19. Pg.37	  J.: Param.5 is redefined in accordance with the param.30 of Spatial

record (revision note 11).

20. Pg.42  App.2: New FOV_STATUS specifications.

21. Pg.43  App.2: Figure 2 “Footprint_Solid_Angle variations” is removed

Figure 2 “Angular_Fluor_Fct()”  is added.

22. Pg.44		Revision notes are added

23. Pg.45		Pending issues are added

24. Pg.i-ii		Table of contents is added

January 25, 2000    Following revision is suggested.

0. C: Nomenclature for Format 1-3 is changed

- E6, multiple parameters:  NaL is changed to NaI

- G, param.6: "=0 if FOV_STATUS=2,4" is replaced by "not valid if FOV_STATUS=2,4"

- H8, param.26-29: description of fractions added

- H8, param.32-33: “Degrees” added

- J, param.1-4: description of fractions added

- Appendix 1.:  some symbols definitions added

April 6-7, 2000.  Following revision is suggested.

1. E5, param.42-51:	 	description is clarified

2. E6, param.101-104,106,108,110,111:  description is clarified.

3. E7, H7,  param.16-24:  	changed to valid for noncalibrated data.

4. E8, H8, param.26,27: 	set to –1 all the time.

5. G, J, param.1,2:    		set to –1 all the time.

6. H10, param.16:          	description is clarified.

- I9, param.14-25,35-46:  	changed to valid for noncalibrated data.

8. I9, param.0-5:  		make it real gain/zero until calibration is

implemented

9. Appendix 3 added

January 12, 2001 Revision

- Revisions of scattering treatment in Appendix 1

- E8, H8, parameter 22		redefined to PHASE_ANGLE

- F10, I10, parameters 3, 11	redefined to AVG_PHASE

4.  H8, param 33  definition is changed

Revision Notes Continued…..

May 25, 2001 Revision

- Changed all Level-1 references in Document to Level 2

- E.7, H.7 (Derived Engineering) changed all references

To (Real and Standard) Gain, Zeros in Level-2 record to

Spare.  Calibration system was never implemented in UA

XRS ground system.  Parameters changed = (0-11)

- F.9, I.9 (Derived Engineering, Summary Record) changed all

References to (Standard) Gain, Zeros in record to

Spare.  Calibration system was never implemented in UA XRS

Ground system.  Parameters changed = (0-5)

- Took out references to Tailing Parameter.  The system was never

implemented as part of the quicklook system.

NEAR XRS Glossary and Acronym List

AL		X-ray Spectrometer: Aluminum Detector

ANTICO	Gamma-ray Spectrometer: Anti-Coincidence

APL		Applied Physics Laboratory

AVG		Average

BGO		Bismuth Germanate

CH		Channel

CMD		Command

CTP		Command and Telemetry Processing

DER		Derived Engineering

DPU		Data Processing Unit

DSN		Deep Space Network

DQI		Data Quality Index

ENG		Engineering

ESC		Escape

ET		Ephemeris Time

FCT		Function

FLG		Flag

FOV		Field of View

FTP		File Transfer Protocol

GCR		Galactic Cosmic Rays

GAS PC	Gas Proportional Counter Detector

GRS		Gamma Ray Spectrometer

GSFC		Goddard Space Flight Center

LT		Light Time

HDF		Hierarchical Data Format

HK		Space Craft or Instrument House Keeping Parameters

HVPS		High Voltage Power Supply

I*4		Signed Integer 4 byte parameter = 32 bits

JPL		Jet Propulsion Laboratory

keV		Kilo Electron Volts

LLD		Lower Level Discriminator

MET		Mission Elapsed Time

MeV		Mega Electron Volts

MG		X ray Spectrometer: Magnesium Detector

NAI		Sodium Iodide

NAIF		Nasa Ancillary Information Facility

NASA		National Aeronautics and Space Administration

NC		Neutron Capture Radiation

NR		Natural Radioactivity Radiation

NEAR		Near Earth Asteroid Rendezvous

POS		Position

PIN		X-ray Spectrometer:  Silicon PIN Detector

QueryID	Query IDentifier file served by Relational Database Management

System

R*4	Floating Point 4 byte Parameter = 32 bits

RDBMS	Relational Database Management System

RT	Rise Time

SC		Space Craft

SCI		Science

NEAR XRS Glossary and Acronym List Cont.

SDC		Applied Physics Laboratory Science Data Center

SPICE	Spacecraft – Planetary – Instrument – C-matrix – Ephemerides

TBD		To Be Decided

UA		University of Arizona

UA-LPL	University of Arizona – Lunar and Planetary Laboratory

UA-RDBMS	University of Arizona – Relational Database Management System

UNF		X-ray Spectrometer:  Unfiltered Detector

WWW		World Wide Web

XDR		eXternal Data Representation

XGRS		X-ray and Gamma-ray Spectrometer

XRS		X-ray Spectrometer