***** File IMSHERS.DOC

EXPLANATION OF THE IMS/HERS DATA FORMATS

This document describes the format of the Giotto Ion Mass

Spectrometer/High Energy Range Spectrometer (IMS/HERS) data

submitted to the International Halley Watch Data Archive.  A

description of the experiment has been published in Balsiger et al.,

ESA-SP 1077, pp 129-148, European Space Agency,     March, 1986

(republished in J. Phys. E: Sci. Instrum., 20,  759, 1987.)

For proton and alpha-particle data, the ASCII files can be

read with the following FORTRAN statements:

OPEN(1,FILE='filename',STATUS='OLD',READONLY)

READ(1,2)IDAY,IUT,DECHR,VX,VY,VZ,DEN,TLOG

2   FORMAT(1X,I3,I8,F8.3,3F7.0,F7.1,F7.2,5X)!for protons

2   FORMAT(1X,I3,I8,F8.3,3F7.0,F7.2,F7.2,5X)!for "alphas"

On the tape as provided to the International Halley Watch, this

file and three other files were supplied with 60 byte records

and a block size of 600 bytes.   The last data file (M/Q>12)

was subsequently broken into 10 pieces as listed in Appendix A.

For heavier ions, the following FORTRAN statements should be used:

OPEN(1,FILE='filename',STATUS='OLD',READONLY)

READ(1,2)IDAY,IUT,ISEC,DECHR,VX,VY,VZ,DEN,TLOG

2       FORMAT(1X,I3,I8,I6,F8.3,3F7.0,F6.1,F7.2)

Where the parameters have the following meanings:

IDAY = Day of year  (Day 72 = March 13, 1986)

IUT = Spacecraft event time in hhmmss at the center of the

measurement interval

ISEC = Length of averaging interval in seconds (Heavy ions only)

DECHR = IUT converted to decimal hours

VX = x component of velocity, in km/s, in HSE coordinates

VY = y component of velocity, in km/s, in HSE coordinates

VZ = z component of velocity, in km/s, in HSE coordinates

DEN = density, in particles/cm**3

TLOG = Log (base 10) of temperature (K)

HSE coordinates are Halley-relative Solar Ecliptic

coordinates

Vx is the velocity component toward the Sun

Vz is the velocity component toward the north ecliptic pole

Vy forms a right-handed system

The temperature is defined by 3nkT = m(v - <v>)**2

It must be emphasized that the IMS/HERS instrument could not

detect ions with spacecraft-relative velocity vectors within

15 degrees of the ram direction.  The method of data

reduction assumed spherical shell distributions, and

corrected for the portion of a spherical shell outside the

field of view of the experiment.  However, if a spherical

shell fell entirely within the blind area, no correction

could be made.  These unmeasured cold ions are an important or

the major component of the ion density after ~2345UT, March 13.

The following files are included in the data set.

PROTONS.IHW

ALPHAS.IHW

HEAVY.IHW

For protons, the data start at 1830 UT on March 13 and

extend to the time of instrument failure at 0003 UT on

March 14.  For alphas, the data also start at 1830 UT on

March 13 but end at 2330 UT, March 13, after which it became

difficult to separate He++ ions from the H2+ (see Fuselier

et al., Geophysical Research Letters, Vol. 15, pages 549-

552,1988). For heavier ions, we give data only when the

fluxes were large enough to permit calculation of density

and temperature.

Until 2348 UT on March 13, proton spectra were obtained

every 4th spacecraft spin (i.e., one measurement every 16

seconds) and the data are given at this time resolution,

with some gaps corresponding to missing or erroneous data.

After 2348 UT, the data mode changed such that 8 spins

(32 s) were required to obtain a proton spectrum. The proton

parameters were calculated using a moment summing technique.

The estimated densities include corrections for protons

outside the field of view of the experiment; the corrections

were based on the assumption that the proton distribution

was isotropic.

Alpha spectra were also obtained every 4th spacecraft spin

and four consecutive spectra have been summed before

calculation of the physical parameters.  The time resolution

of the "alpha" data is therefore 64 s. The estimates of the

alpha parameters were obtained by fitting to an isotropic

distribution, and the estimated densities include

corrections for alphas outside the field of view of the

experiment.

The heavier ion spectra are at nonconstant intervals because

it was possible to obtain estimates more frequently when

fluxes were higher. Data are given only for those intervals

and those species for which there were at least 8 counts in

the appropriate part of phase  space. Note that the

components of velocity are independent of M/Q for any given

interval. This results from the method of calculation.  For

each interval, the water-group ions (M/Q = 16-18) were used

to determine the velocity that gave the best (least-squares)

fit of the ion distributions to an isotropic distribution.

For each time interval, the best-fit water-group vector

velocity was then used to transform the measured velocity

distribution for each ion species into a moving coordinate

system in which the density and temperature were calculated,

again assuming isotropy. The estimated densities include

corrections for ions outside the field of view of the

experiment.

To convert from time to cometocentric distance:

Distance (km) = (24.05 - dechr) * 3600 * 68.4

For further information, contact one of the following

people at:

Mail Stop 169-506,

Jet Propulsion Laboratory,

Pasadena, CA 91109,

USA:

Bruce E. Goldstein, Telephone 818-354-7366

Raymond Goldstein, 818-354-0241

Marcia Neugebauer, 818-354-2005

*******************************************************************

Appendix A

Note: a corrected file of "HEAVY" densities was sent on April 3, 1991.

These data have been reformatted into separate files.   A complete list

of files, including protons and alphas, follows:

Filename            Content

IMSHER01            protons

IMSHER02            alphas

IMSHER03            M/Q = 12

IMSHER04                  13

IMSHER05                  14

IMSHER06                  15

IMSHER07                  16

IMSHER08                  17

IMSHER09                  18

IMSHER10                  28

IMSHER11                  29

IMSHER12                  32

*******************************************************************