***** File PIA_MODE.DOC

Mission: GIOTTO

Instrument: PIA

Submission: MPI Text Files

Original Format

The data for these files were received as a series of FITS files, each of

which consisted of spectra of a single mode.  Amplitude-triggered samples

in the spectra were indicated by an "X" following the sample.

Reformatting at SBN

The following changes were made to the data:

o  A program was written to parse the headers and spectra and convert

the ASCII values to binary form.  Parameter values were ordered to

facilitate comparison between the MPI and IKI submissions for the

VEGA missions, which used the same instrument (PUMA).

o  Flagged amplitudes, followed by an "X" in the text file, were

indicated in the binary file as a additional byte set equal to 1

for flagged amplitudes, 0 otherwise.  This is the same system used

for the IKI VEGA submissions.

o  All spectra of the same mode were collected into a single, fixed-

length table.

o  The original order of the spectra in the FITS files, as well as the

order of the text files themselves, was preserved in the combined

mode tables.  This order was assumed to be chronological.

Known Anomalies/Problems:

o  Little or no explanation was available for some of the parameters in

the spectrum headers.  This is indicated in the DESCRIPTION part of

the PDS label for these datasets.

o  It has not yet been possible to translate the times in this file,

which are apparently in units of spacecraft clock time, to UT.

o  In the received FITS file numbered 40, the final spectrum in the

file (sequence number 2991, event number 46698, time 48110) was

truncated.  It is not clear if only this last spectrum was lost

or if other spectra might have followed since the NAXIS1 and NAXIS2

values from the header indicate more data.  The incomplete spectrum

was omitted entirely from the binary table.

o  Two mode 1 spectra were found which contained an odd number of bytes

in the spectrum.  In mode 1 the number of bytes should be even, as

each sample in the spectrum consists of a time sample byte and an

amplitude sample byte.  In these cases the single byte was assumed to

be a time value and a null amplitude byte was appended to the spectru

.

o  In working with these files it was noted that many spectra exhibited

strange features, including:

- parameter values but no spectrum bytes (i.e., zero-length

spectra)

- repeating time values of 255 (the maximum possible time)

- repeating amplitude values of 127 (the maximum possible)

- repeating values of 32 (ASCII code for blank)

o  There was a failure in the instrument during the fly-by.  The

following is taken directly from the FITS file header which was

attached to the text files:

"During the actual flyby two out of six multiplier-amplifiers

failed, yielding incorrect amplitude information in the

mass spectra channel; to obtain partial correction for

amplitude, use F(a), where

F(0)       = 0           F(20)       = 49

F(1...4)   = a*2 + 21    F(21)       = 52

F(5...8)   = a + 25      F(22)       = 57

F(9...14)  = a + 26      F(23...25)  = 3*(a-23) + 62

F(15,16)   = a + 27      F(26...83)  = a + 4a

F(17...19) = a + 2b      F(84...127) = 128

NOTE: Despite this correction the amplitude ranges 1...22 and

around 51...65 are incomplete."

No further documentation of this problem was available.  Specifi-

cally, it seems likely the "2b" in the equation for F(17...19) was

intended to be "28".