***** File TNM.DOC

MISSION: VEGA-1

INSTRUMENT: TUNDE-M

solid state detector to measure high energy ions and electrons

Mass: 2.2 kg

Dimensions: 15x16.5x15 cm

Date built: 1985

Manufacturer: Central Research Institute for Physics, Budapest,

Hungary

TUNDE-M consists of two particle telescopes: T1 is viewing at 55 deg to the

east of the Sun, T2 looks at 90 deg to the east of the Sun, both in the

ecliptic plane.

The scientific objective was to study the high energy (from about 50 keV

to 700 keV) ion environment of comet Halley. Ions (probably mostly water

group) were successfully observed (for details, see Somogyi et al. Nature

321, 285, 1986; Gribov B.E. et al., Astronomy and Astrophysics 187, 293

1987; Kecskemety K. et al., J. Geophys. Res. 94, 185, 1989; Somogyi A.J.

et al., in: "Comet Halley - Investigations, Results, Interpretations",

Ellis Horwood Limited, Chichester, England, Vol. 1, 203, 1990; and

Kecskemety K. and T.E. Cravens, J. Geophys. Res. 97, 2891, 1992.)

Calibrations: electronic calibrations  were  performed  only,  the

results were used to determine the relative width of the ion energy

channels and from this to obtain the real values of the energy

limits of the channels starting from the nominal values.

The instrument was mounted to point in a fixed direction relative to the

spacecraft.

Principal investigator: Prof. A.J. Somogyi.

Coinvestigators:

G. Erdos, J. Ero jr., M. Farago, T.I. Gombosi, K. Kecskemety, A. Kondor,

T. Kovacs jr., Gy. Kozma, L. Lohonyai, R. Redl, J. Szabo, L. Szabo, K. Szego,

I. Szentpetery, A. Szepesvary, I. T.-Szucs, M. Tatrallyay, A. Varga,

J. Windberg, A. Zarandy (Central Research Institute for Physics,

Budapest, Hungary);

R.Redl (Technical University, Budapest, Hungary);

T.E. Cravens (Space Physics Research Laboratory, University of

Michigan, Ann Arbor, USA);

A. Dyachkov, I.N. Klimenko, A.P. Remizov, M.I. Verigin, G.A. Vladimirova

(Space Research Institute, Moscow, USSR);

E. Keppler, A.K. Richter (Max-Planck-Institute fur Aeronomie,

Katlenburg-Lindau, FRG);

Yu.I. Logachev, V.G. Stolpovskii (Moscow State University, Moscow, USSR)

R. Marsden, K.-P. Wenzel (Space Science Dept., ESA, ESTEC, Noordwijk,

The Netherlands).

Instrument Section/ Modes of Operation

TUNDE-M consists of two particle telescopes: T1 is viewing at 55 deg to the

east of the Sun, T2 looks at 90 deg to the east of the Sun, both in the

ecliptic plane. The telescopes consist of two circular surface barrier

semiconductor detectors with an Al layer of 15x10^-6 g/cm2 coated on the

front detector (A) and an anticoincidence scintillation detector MCP.

Low energy electrons (below about 160 keV) were deflected with a magnet.

Field of view (half cone): 25 deg. Geometric factor: 0.25 cm^2 sr.

Particles stopping in the thin front detector were not identified (they

are referred as ion channels), those reaching the thick detector were

identified via the dE/E method.

T1 was working all along the mission, T2 was rendered inoperative before

the Halley encounter.

Modes of operation: Trassa-1 and Trassa-2 (cruise phase) and BRL (fast

telemetry, started at 04:17:03 UT, March 6, 1986, ended at 08:24:53 UT,

same day). Data were gathered in 66 channels, some referred as ions,

others as identified particles, both between preset energy limits (given

at the data set description).

Data rate: 1 frame of 66 real numbers in every 4 s (BRL mode).

The measurement was continuous, with an integration time of 4 s.

Other technical details can be found in:

Somogyi A.J. et al., in: Proc. of Int. Conf. on Cometary Exploration,

Budapest 1982 (ed. T.I. Gombosi), vol. 3, p351, 1983 and Somogyi A.J. et

al., in: "Field, particle and wave experiments on cometary missions"

(eds. K. Schwingenschuh and W. Riedler, Austrian Academy of Sciences,

Proc. of International Workshop, Graz, Austria 1985, p237, 1986.

Details concerning scientific result were published in:

Somogyi et al. Nature 321, 285, 1986; Gribov B.E. et al., Astronomy and

Astrophysics 187, 293 1987; Kecskemety K. et al., J. Geophys. Res. 94,

185, 1989; Somogyi A.J. et al., in: "Comet Halley - Investigations,

Results, Interpretations", Ellis Horwood Limited, Chichester, England,

Vol. 1, 203, 1990; and Kecskemety K. and T.E. Cravens, J. Geophys. Res.

97, 2891, 1992.

DATA SET

The data were measured during the BRL seance on 6 March 1986.

The data are listed as 62 records long blocks (time and

61 energy channels), blocks were obtained in every 4 s.

The structure of the blocks is the following:

1st record - time in sec at the end of integration. To obtain the spacecraft

time in UT, one has to add 4h 16m 59s to that number.

2nd to 62nd records (integer numbers): counts/4 s in the energy channels

listed below (in channels 5 to 57 - from 6th to 58th record - the energies

depend on the identity of the particles, they are given below both

for protons and oxygen ions in keVs):

rec  2  protons     E = 3.2-4.5 MeV

rec  3  protons     E = 4.5-13 MeV

rec  4  ions 20- 30 keV (noisy)

rec  5  ions 30- 40 keV (noisy)

nominal      proton        oxygen

energy       energy        energy

rec  6  ions 40- 50       49- 57        96-106

rec  7       50- 60       57- 70       106-125

rec  8       60- 70       70- 80       125-139

rec  9       70- 80       80- 91       139-153

rec 10       80- 90       91-100       153-164

rec 11       90-100      100-108       164-174

rec 12      100-110      108-117       174-184

rec 13      110-120      117-126       185-196

rec 14      120-130      126-136       196-209

rec 15      130-140      136-146       209-221

rec 16      140-150      146-160       221-238

rec 17      150-160      160-165       238-245

rec 18      160-170      165-172       245-254

rec 19      170-180      172-182       254-265

rec 20      180-190      182-188       265-274

rec 21      190-200      188-197       274-284

rec 22      200-210      197-212       284-302

rec 23      210-220      212-216       302-306

rec 24      220-230      216-231       306-324

rec 25      230-240      231-241       324-335

rec 26      240-250      241-260       335-342

rec 27      250-260      246-265       342-357

rec 28      260-270      260-272       357-366

rec 29      270-280      267-280       366-381

rec 30      280-290      280-286       381-388

rec 31      290-300      286-295       388-398

rec 32      300-310      295-304       398-408

rec 33      310-320      304-312       408-417

rec 34      320-330      312-324       417-431

rec 35      330-340      324-335       431-443

rec 36      340-350      335-349       443-458

rec 37      350-360      349-353       458-462

rec 38      360-370      353-360       462-471

rec 39      370-380      360-374       471-486

rec 40      380-390      374-383       486-496

rec 41      390-400      383-392       496-505

rec 42      400-410      392-402       505-516

rec 43      410-420      402-413       516-529

rec 44      420-430      413-425       529-542

rec 45      430-440      425-433       542-550

rec 46      440-450      433-444       550-562

rec 47      450-460      444-450       562-568

rec 48      460-470      450-467       568-586

rec 49      470-480      467-476       586-596

rec 50      480-490      476-487       596-607

rec 51      490-510      487-502       607-623

rec 52      510-160      502-527       623-650

rec 53      530-170      527-548       650-674

rec 54      550-180      548-568       674-695

rec 55      570-190      568-591       695-720

rec 56      590-100      591-608       720-738

rec 57      610-630      608-633       738-766

rec 58      630-3000     633-3000      738-3000

rec 59      electrons   E = 160-300 keV

rec 60      alpha       E = 3-13 MeV/n

rec 61      control

rec 62    alpha or heavier   > 13 MeV

No processing has been applied to get the data other than computing

counts from their m2**n form, where m is a <64 integer.

The data set is continuous with one major data gap as given below:

Time:

CD data file           end of frame (UT)

first              8                     4:17:07

last before

data gap        2293                     4:55:11

first           4446                     5:31:05

last           14779                     8:23:18