***** File TVS_PROC.TXT

December 1, 1994

The TVS data from KFKI came in various stages.  The initial submission

from KFKI was completely replaced in the fall 1992.   At that time,

additional processed data with aspect and range corrections were included.

The original filenaming scheme was preserved except for changing the

leading filename character and expressing the aspect ratio as "a".

Mission  data description                 KFKI      New       LPL-NO

Vega 1   original 63 images               w01114    T01114    1114

filtered 32 images               w02114    T02114    2114

filtered with 4/3 aspect corr    w211443   T2114A

filtered, aspect and range corr  w211443r  T2114AR

sharpened 32 images              w09114    T09114    9114

sharpened with 4/3 aspect corr   w911443   T9114A

sharpened, aspect and range corr w911443r  T9114AR

flat field                       w02769    T02769    2769

Vega 2   original 11 images               w01174    T01174    1174

filtered 11 images               w11174    T11174    2174

flat fields                      w01301    T01301

Vega 1 Data:

Later submissions included the images from March 5 (60) and March 7 (186)

before and after the encounter period (March 6 (7:21:00.9), 1986).  These

data were given a file description to indicate if they were PRE or POST

encounter.    The data is included in its original form but with a new

filenaming scheme (TPREnnnn and TPOSnnnn), unique to the data set.  The data

were sent as a separate image file (8 bit) with an ascii description of

the header information.   An IHW-FITS file was created from this

data in the style of the previous submissions, and approximate times

was calculated from the TONBHRDW value provided with the data.  Since

this is a relative value, the switch-on time was taken from a preprint

by Laszlo Varhalmi (KFKI) and a previous communication with KFKI also

gave the state vector for each spacecraft during the encounter.

Note that the switch-on times are then given  as UT = Moscow time - 3 hours

and a "light travel time" must be subtracted to match published values.

The algorithm for the midpoint of any exposure was calculated as follows:

UT (midpoint) = UT0+5.5+TONBHRDW/4.766485-T(exp)-T(loc)+T(exp)/2  (second).

Note the clock count rate comes from a fit of times to original 63 images.

We have taken the 5.5 s correction from Varhalmi as due to a memory readout;

another correction of 1.6 s called T (loc) was needed if the image was less

than the full array which is a process time to find and transmit the data.

For Vega 1, the assumed switch-on times were March 5, 5h48m8s and for

March 7, 7h 58m57s; these values are given as COMMENTs in the FITS header.

The state vectors for Vega 1 in equatorial XYZ coordinates are as follows:

X=-51480466.2   Y=-106677747.1   Z=+4023544.7   (in km)

V(X)=+33.4265   V(Y)=-10.6418    V(Z)=-2.7059   (in km/sec)

It was asuumed these were barycentric coordinates and the linear velocities

were used to estimate the coordinates for the new dates and times.

Appropriate values from the Nautical Almanac for 1986 were used to make the

earth correction and eventually estimate the light travel time correction.

It was determined that the calculated UT times are consistent with the

published values within 3 seconds; as such we leave the TIME-OBS keyword

value (midpoint day fraction required by the IHW) as 4 significant digits.

Unfortunately, in the processing it was discovered that 3 images in POST

submissions from Vega 1 had inconsistent descriptions and were rejected.

Also, an error in the times provided in the revised FITS headers for

the original 24 images (Nos 1-20, 25, 33, 39, 61) was corected at SBN.

In one case, T01111, the FITS header had incorrect values for the exposure

and it is not clear that the data is correctly matched with the header.

Vega 2 Data:

Later submissions included the images from March 7 (51) and March 10 (189)

before and after the encounter period (March 9 (7:20:54.8), 1986).  These

data were given a file description to indicate if they were PRE or

POST encounter.    The data, in its original form, is included but with

a new filenaming scheme which was unique to the data set.  The data

were sent as a separate image file (8 bit) with an ascii description of

the header information.   An IHW-FITS file was created from this

data in the style of the previous submissions, and approximate times

was calculated from the TONBHRDW value provided with the data.  Since

this is a relative value, the switch-on time was taken from a preprint

by Laszlo Varhalmi (KFKI) and a previous communication with KFKI also

gave the state vector for each spacecraft during the encounter.

Note that the switch-on times are then given  as UT = Moscow time - 3 hours.

and a "light travel time" must be subtracted to match published times.

The algorithm for the midpoint of any exposure was calculated as follows:

UT (midpoint) = UT0+5.5+TONBHRDW/4.766485-T(exp)-T(loc)+T(exp)/2  (second).

Note the clock count rate is assumed to be the same as for Vega 1 data.

We have taken the 5.5 s correction from Varhalmi as due to a memory readout;

another correction of 1.6 s called T (loc) was needed if the image was less

than the full array which is a process time to find and transmit the data.

For Vega 2, the assumed switch-on times (UT0) were March 7, 3h47m25s and for

March 10, 5h48m14s; these values are given as COMMENTs in the FITS header.

The state vectors for Vega 2 in equatorial XYZ coordinates are as follows:

X=-63031872.6   Y=-107676032.6   Z=+1048764.   (in km)

V(X)=+31.6486   V(Y)=-11.6669    V(Z)=-1.8879   (in km/sec)

It was asuumed these were barycentric coordinates and the linear velocities

were used to estimate the coordinates for the new dates and times.

Appropriate values from the Nautical Almanac for 1986 were used to make the

earth correction and eventually subtract the light travel time correction.

It was determined that the calculated UT times are consistent with the

published values within 3 seconds; as such we leave the TIME-OBS keyword

value (midpoint day fraction required by the IHW) as 4 significant digits.

For two of the filtered files, the submitted FITS header had errors

in the TONBHRDW values; these were eliminated.  In the case of 5 other

files (1198, 1202, 1206, 1210, 1214), no FITS information was sent; the

FITS header was constructed from the corresponding original data file

except that a CHECKSUM value was added to identify the image correctly.

Abergel, J., Bertaux, G., Avanessov, G.A., Tarnopolsky, V.I., and

Zhulov, B.S., 1987, Appl. Opt., 26, No.26, 4457-4468.

Kondor, A., 1990, MPAE-W-100-90-04.

Merenyi, E., Foldy, C., Szego, K., Toth, I. and Kondor, A., 1990, Icarus,

86, 9-20.