***** File MODE_TAB.DOC

Mission: VEGA 2

Instrument: PUMA

Submission: MPI Text Files

Original Format

The data for these files were received as a series of text files, each of

which consisted of spectra of a single mode.  Amplitude-triggered samples

in the spectra were indicated by an "X" following the sample.

Reformatting at SBN

The following changes were made to the data:

o  A program was written to parse the headers and spectra and convert

the ASCII values to binary form.  Parameter values were ordered to

facilitate comparison between the MPI and IKI submissions.

o  Flagged amplitudes, followed by an "X" in the text file, were

indicated in the binary file as a additional byte set equal to 1

for flagged amplitudes, 0 otherwise.  This is the same system used

for the IKI submission.

o  All spectra of the same mode were collected into a single, fixed-

length table.

o  The original order of the spectra in the text files, as well as the

order of the text files themselves, was preserved in the combined

mode tables.  This order was assumed to be chronological.

Known Anomalies/Problems:

o  Little or no explanation was available for some of the parameters in

the spectrum headers.  This is indicated in the DESCRIPTION part of

the PDS label for these datasets.

o  There are more spectra in this submission than IKI submission.

While there is a large amount of overlap between the IKI and MPI

submissions, each contains spectra not included in the other.

o  It has not yet been possible to translate the times in this file,

which are apparently in units of spacecraft clock time, to UT.

o  A number of mode 1, 2, 3  and test spectra were found which contained

an odd number of bytes in the spectrum.  In these modes the number of

bytes should be even, as each sample in the spectrum consists of a

time sample byte and an amplitude sample byte.  In these cases the

single byte was assumed to be a time value and a null amplitude byte

was appended to the spectrum.

o  Although the text files were each supposed to contain spectra of only

a single mode, in several files of test mode 3 spectra there were one

or more spectra of some other mode.  It was not possible to unambigu-

ously assign locations for these spectra within the the other mode

tables, so they were omitted from those tables.  The text forms of

these spectra may be found in the file "MISFITSP.TXT".

o  In working with these files it was noted that many spectra exhibited

strange features, including:

- parameter values but no spectrum bytes (i.e., zero-length

spectra)

- repeating time values of 255 (the maximum possible time)

- repeating amplitude values of 127 (the maximum possible)