***** File DUCMA.DOC

VEGA DATA DOCUMENTATION

A description of the experiment is in the following references:

Simpson, J. A., Rabinowitz, D. and Tuzzolino, A. J. 1989, "I. PVDF

detector signal dependence on mass and velocity for penetrating

particles", Nucl. Instr. Meth., A279, 611-624.

Simpson, J. A. and Tuzzolino, A. J. 1989, "II. Instruments for

measurement of particle trajectory, velocity, and mass", Nucl.

Instr. Meth., A279, 625-639.

The Vega data consists of 4 files, two files each for Vega-1 and Vega-2.

1) Vega 1 dsfast17 file:

The dsfast17 file includes all the fast telemetry data (data obtained

every two seconds).  The fast telemetry data covers the entire

Halley's comet encounter, lasting approximately four hours.

The telemetry noise events have not been removed

from the data. Each readout includes; 1)the time at which data

were obtained in seconds (given in time since the Vega launch

date), and the Blissi clock described in the technical manual,

2) Two eight integer status commands also described in

the Ducma Technical Manual, 3) two temperatures, the detector

temperature and the electronic temperature explained

in the technical manual, 4) the dust events given as ten

integers which represent counts in each of four different

thresholds, M4-M1, as well as the anticoincident counts, M4NV-M1NV,

and the two V detector counts, VL and VH, which register

noise events. More detailed explanations may be found in the

Ducma technical manual on file at the Small Bodies Node of PDS.

A sample data line appears as follows:

Ducma     Blissi     Command     Status      Detector    Elect.

clock     clock                               Temp.      Temp.

38252812     1      10100100    11011001     -18.01      30.86

M4    M3      M2    M1    M4NV   M3NV   M2NV   M1NV    VL     VH

0     0       1     2       0      0      0      0      0      0

2) Vega 1 Dvega101 file:

Dvega101 file includes the encounter data without any information

about status commands and temperature variations.

The noise spikes have been removed. This file is designed for easy data

analysis and lists simply the time of an event in seconds and the total

counts in each threshold, M4-M1, the anticoincident events, M4NV-M1NV,

and the noise events, VL and VH, all obtained during that time interval.

The masses associated with each threshold follow: M1: 1.5 x 10**-13 g,

M2: 5.6 x 10**-11 g, M3: 1.6 x 10**-9 g, and M4: 4.0 x 10**-8 g.

3) Vega 2 dsfast27 file:

This file is the entire fast telemetry data for Vega-2 .  Its format is

similar to sfast17, except Vega-2 obtained data every second instead of

every two seconds.

4) Vega 2 Dvg2cln1 file:

This file is the encounter data for Vega-2. Its format is similar to

Dvega101. Note the accidental calibration and noise spikes have been

removed.  Again Vega-2 obtained data every second.

Below are two charts to help in the conversion of BLISSI clock times for

Vega1 and Vega2 to DUCMA clock times and 'calendar time'.

DUCMA times are given in seconds after launch

BLISSI times are given in seconds

Vega1

DUCMA start time: Dec. 15 1984 10:33:37

from file dsfast17.tab

------------+-----------+--------------------------+--------------------+

DUCMA time |BLISSI time|    DUCMA Time to CA 	   |      Real time	|

(seconds) | (seconds)	| (seconds) |(DD hh:mm:ss) | (YY-MM-DDThh:mm:ss)|

------------+-----------+-----------+--------------+--------------------+

37910088  |    179	|   612424  | 07 02:07:04  |  86-02-27T05:13:02	|

37917104  |   7195	|   605408  | 07 00:10:08  |  86-02-27T07:09:58	|

38344948  |      8	|   177564  | 02 01:19:24  |  86-03-04T06:00:42	|

38352136  |   7196	|   170376  | 01 23:19:36  |  86-03-04T08:00:30	|

38431348  |     13	|    91164  | 01 01:19:24  |  86-03-05T06:00:42	|

38438524  |   7191	|    83988  | 00 23:19:48  |  86-03-05T08:00:18	|

38511536  |      2	|    10976  | 00 03:02:56  |  86-03-06T04:17:10	|

* 38522512  |  10976 	|        0  | 00 00:00:00  |  86-03-06T07:20:06 |

38526412  |  14878	|     3900  | 00 01:05:00  |  86-03-06T08:25:06	|

38612184  |    202	|    89672  | 01 00:54:32  |  86-03-07T08:14:38	|

38619168  |   7188	|    96656  | 01 02:50:56  |  86-03-07T10:11:02	|

------------+-----------+-----------+--------------+--------------------+

Vega2

DUCMA start time: Dec. 21 1984 10:29:46

from file dsfast27.tab

------------+-----------+--------------------------+--------------------+

DUCMA time |BLISSI time|    DUCMA Time to CA 	   |      Real time	|

(seconds) | (seconds)	| (seconds) |(DD hh:mm:ss) | (YY-MM-DDThh:mm:ss)|

------------+-----------+-----------+--------------+--------------------+

37478720  |   49156   |   785332  | 09 02:08:52  |  86-02-28T05:11:08	|

@ 37483380  |   53818 	|   780672  | 09 00:51:12  |  86-02-28T06:28:48	|

@ 37483380  |   37435 	|   780672  | 09 00:51:12  |  86-02-28T06:28:48	|

37483384  |   37436   |   780668  | 09 00:51:08  |  86-02-28T06:28:52	|

37486588  |   38595	|   777464  | 08 23:57:44  |  86-02-28T07:22:16	|

38079280  |       1	|   184772  | 02 03:19:32  |  86-03-07T04:00:28	|

38086472  |    7191	|   177580  | 02 01:19:40  |  86-03-07T06:00:20	|

38173960  |       5	|    90092  | 01 01:01:32  |  86-03-08T06:18:28	|

38181148  |    7194	|    82904  | 00 23:01:44  |  86-03-08T08:18:16	|

38252812  |       1	|    11240  | 00 03:07:20  |  86-03-09T04:12:40	|

* 38264052  |   11242   |        0  | 00 00:00:00  |  86-03-09T07:20:00 |

38268860  |   16049	|     4808  | 00 01:20:08  |  86-03-09T08:40:08	|

------------+-----------+-----------+--------------+--------------------+

*-times of Closest Approach (CA)

@-BLISSI clock apparently reset from 53818 to 37435 during 4 second

DUCMA time period beginning at 37483380 seconds, resulting

in two very different BLISSI clock values 'mapping' to the

same DUCMA time and Real time.