***** File SP2.DOC

TABLES

OF DUST PARTICLE SPATIAL AND MASS DISTRIBUTIONS IN COMET P/HALLEY

FROM SP-2 EXPERIMENT ON BOARD VEGA 1 AND VEGA 2 SPACECRAFT

1. THE SP-2 EXPERIMENT

The SP-2 experiment on Vega spacecraft was intended for studying

the spatial and mass distributions of dust particles in the

cometary coma over the mass range 1. x 10**-16 to 1. x 10**-6 g. Covering

such a broad mass range was made possible by using sensors of two

types, namely, impact plasma and acoustic.

Experiment tasks and measurement procedures

Dust particle velocities in the coma are small compared to the

encounter velocity. In the cometary nucleus reference system the

encounter pattern envisages the flyby module passage through the cloud

of motionless dust particles, with the velocity V along the trajectory

with the minimum distance from the nucleus, Ro.

At every trajectory point with the distance R to the nucleus,

where the density of dust particles with masses larger than some

specified value m, is n(R) (> m), a surface with area S normal to the

velocity vector V, is subjected to dN/dt(> m) = nR (> m) SV impacts with

dust particles per unit time. Thus, by measuring this value for

different R and m we can determine n(R) (> m), (the value we are

interested in). This direct measurement method obviously makes it

possible not only to determine the general dependence of dust particle

densities on their distance to the nucleus, as well as their mass

distribution, but also to detect variations of these characteristics,

due to the non-stationary processes in the nucleus (explosion-induced

shedding of outer layers, rotation and so on).

Instrument components

The SP-2 system consists of impact detectors of two types,

acoustic and ionization. A general view of the instrument with block

diagram is given in 'Venus-Halley Mission' (Balekanov 1985). The basic

acoustic detector is a thin circular membrane (500 cm**2 in area) with

three piezoelectric elements. While a particle impacts the membrane

the wave due to the flexural strain propagates. The force acting upon

a piezoelectric element and proportional to the acceleration vertical

component generates an electric signal. The acoustic detector is

sensitive to the momentum imparted to the membrane during the

impact. The hyper-velocity impact of a dust particle is accompanied by

a micro-explosion that results in forming the crater out of which a

dense highly-ionized plasma is ejected. The value of recoil momentum

associated with this ejection may be sufficiently higher than that of

intrinsic momentum of the dust particle P = MV. The numeric

simulation of the impact yields the estimates showing that at 80 km/s

the recoil momentum may be higher than the initial momentum P = MV by

an order of magnitude. The accurate estimation of K-factor prevents a

serious problem in calibrating such kind of detectors.

The piezoelectric microphone output signal enters the

preamplifier. Because of the very short impact duration the

signal-power frequency spectrum extends to the high ultrasonic

frequency region. A high-frequency signal component (about ~ 160 kHz)

is amplified by the narrow-band amplifier and detected. At the output

the signal dynamic range ~5x10**6 is divided into 16 channels with

constant relative-variation of the threshold delta/m = 2.82 that allows

measuring the particle mass spectrum n ( > m) at 16 equi-spaced points

on the log m scale. Two detectors and two identical independent

detecting circuits are used to make measurements more reliable. The

third central piezoelement controls the in flight calibration. It

regularly receives a calibrated electric pulse converted by the

detector into the mechanical pulse to simulate the impact with a

particle of certain mass.

According to the preliminary estimates obtained from the

well-known Comet dust envelope models the counting rate of small dust

particles may be very high. When a particle impacts the piezoelectric

element, the latter is subjected not only to the forward wave caused

by strain but also to the waves reflected from the membrane edges. Due

to this effect the output signal is considerably delayed in time and

thereby the impact counting rate will be limited to < 50 to 100 1/s.

It means that the instrument's time resolution can prove insufficient

and the saturation may be reached long before the closest approach to

the Comet. To avoid this the wave reflections from the membrane edge

are suppressed by the damper inserted along the membrane perimeters,

which has an output signal of rather short duration (not more than 1

ms) due to the efficient suppression of reflections.

The acoustic detector is mounted on the instrument's body via

three acoustic insulators to protect the detector against mechanical

noises.

The dust particle ionization detectors are placed in four

cylinders served as the load-bearing structural elements. The

detectors are identical and arranged in pairs. The detectors of each

pair are connected to two individual measuring systems to make

measurements more reliable. A dust particle passes through the set of

deflecting electrodes and grids which protect the detector inner pan

from the exposure to the surrounding plasma and electromagnetic

fields, and hits the lower electrode surface. The highly ionized dense

plasma cloud is expanded in the lower chamber, then the charges of

opposite sign are separated by the electric field and collected by the

chamber electrodes to which the potential 2kV is applied.

The instrument measures the total charge of electrons collected on

the lower electrode.

The published results of the laboratory studies of the hyper-

velocity impact show that the charge Q ~ [m**(alpha)][V**(beta)] where

alpha ~ 1, beta ~ 3. With the constant velocity of particles, as in

our case, the total charge is proportional to their mass.

The dynamic range of the ionization detectors is 10**5. It is

divided into six output channels to measure a dust-particle mass

spectrum at six points on the mass scale with a factor of an order of

magnitude. The SP-2 instrument was not intended to realize the maximum

possible sensitivity of the ionization technique. Smallest dust

particles are measured with the VEGA SP-1 instrument.

The regions of mass measurements with the acoustic and ionization

detectors are overlapped appreciably that should allow comparing the

data independently obtained on micron and submicron dust particles.

Such a procedure may reduce uncertainty due to calibration errors and

permit a more reliable comparison between the data about the particles

which dominate the solar light scattering and the results of remote

photometry and polarimetry of the Comet.

SP-2 performance characteristics

The threshold sensitivity of the dust particle acoustic detector

in terms of the transferred momentum is given by Pmin ~ 2x10**-5

gcm/s.  Taking into account that the recoil momentum may exceed the

particle momentum mV by 5 to 10 times, the minimum mass of recorded

particles is < 3x10**-13 g.  The measurement range n (> m)x3x10**-13

to ~ 2x10**-6 g is divided into 16 channels. With the relative width

delta/m ~ 2.82, sensitive area S = 500 cm**2, accumulation time for one

spectrum is 1 s, which corresponds to the spatial resolution ~ 80 km

over the flyby trajectory. The counting rate of small mass particles

is about 4095 1/s.

The ionization detector is capable of recording particles with

mass 3x10**-16 to 3x10**-11g. The number of channels is 6, with

relative width delta/m = 10, sensitive area - 40 cm**2. The limit

counting rate of small mass particles is ~ 65,500 1/s. The

accumulation time for one spectrum is 1 s.

As it was mentioned above, the detector and measuring systems of

the instrument are backed-up. The instrument represents the results of

measurements and flight tests for each cycle in a digital format of

200 bit. It may dump the data to either high-speed or low-speed link

or to both simultaneously at a rate 65,535 bit/s and 3072 bit/s,

respectively.

The instrument mass is 4 kg, power consumption, 3.5 W.

The test support equipment is used to test the instrument

functioning in all modes.

The instrument was developed by the Soviet specialists.

Balebanov, V.M., Skuridin, G.A., Vorontsova, E.V., Bassolo, V.S.(eds.):

1985, Venus-Halley Mission, Louis-Jean, Gap

Mazets, E.P., Aptekar, R.L., Golenetskii, S.V., Guryan, Y.A.,

Dyachkov, A.V., Ilyinskii, V.N., Panov, V.N., Petrov, G.G.,

Savvin, A.V., Sagdeev, R.Z., Sokolov, I.A., Khavenson, N.G.,

Shapiro, V.D., Shevchenko, V.I., 1986, Nature 321, 276

Mazets, E.P., Sagdeev, R.Z., Aptekar, R.L., Golenetskii, S.V.,

Guryan, Y.A., Dyachkov, A.V., Ilyinskii, V.N., Panov, V.N.,

Petrov, G.G., Savvin, A.V., Sokolov, I.A., Frederiks, D.D.,

Khavenson, N.G., Shapiro, V.D., Shevchenko, V.I.,

1987, Astron. Astrophys. 187, 699-704

2. OBSERVATIONS AND DESCRIPTION OF THE DATA TABLES

The flyby trajectories of the Vega spacecraft in the cometocentric

frame can be specified by the coordinates of the closest approach

to the nucleus R0(R0x,R0y,R0z) and the components of the relative

velocity V(Vx,Vy,Vz). The corresponding values for Vega1 and Vega2

were as follows:

Vega 1:   R0x = 7830 km;      R0y = 2480 km;      R0z = 3400 km;

Vx = 28.64 km/s;    Vy = -72.73 km/s;   Vz =-12.95km/s;

Vega 2:   R0x = 7100 km;      R0y = 2680 km;      R0z = 2630 km;

Vx = 30.53 km/s;    Vy = -69.44 km/s;   Vz =-11.88km/s;

If the results of the dust flux measurements are referred to the

time reckoned from the instant of closest approach, the

corresponding distances to the cometary nucleus can be found as

R=R0*sqrt(1 + (t/t0)**2), where the parameter t0 = R0/V. In the

course of measurements the numbers of particle impacts with masses

exceeding the channel threshold is counted in each pulse height

channel at 1 second intervals along the trajectory:

N(>m) = F(>m)*S*T = n(>m)*V*S*T,

where n(>m) is the number density, F(>m)=n(>m)*V the cumulative

flux, S the effective sensor area (500 cm2 for the acoustic sensor

and 28.57 cm2 for the impact plasma one), T=63/64s the accumulation

time in each measurement.

For the acoustic sensors, dead time corrections were introduced at

channel count rate N(>m) > 50/s. For the fast impact plasma sensors

such corrections were not needed.

The SP-2 data for Vega1 spacecraft are formatted into table containing

2092 records. Each record consists of 18 columns and contains time of

measurement referred to "closest approach" (where 0 sec is 4:12:38 UT

adopted from SP-1 experiment), 5 columns of impact plasma sensor data,

and 12 columns of acoustic sensor data.

The data table for Vega2 is similar but record contains 14 columns for

acoustic sensor data, total 20 columns. Total number of records is 2267.

Below are the mass threshold values (in gramms) for the table columns

described.

Column    Data        Vega1             Vega2

type

1        ---        time              time

2       Plasma    N( > 6.e-16 )     N( > 6.e-16 )

3       Plasma    N( > 6.e-15 )     N( > 6.e-15 )

4       Plasma    N( > 6.e-14 )     N( > 6.e-14 )

5       Plasma    N( > 6.e-13 )     N( > 6.e-13 )

6       Plasma    N( > 6.e-12 )     N( > 6.e-12 )

7       Acoustic  N( > 1.5e-11 )    N( > 6.6e-13 )

8       Acoustic  N( > 4.1e-11 )    N( > 1.8e-12 )

9       Acoustic  N( > 1.2e-10 )    N( > 5.2e-12 )

10      Acoustic  N( > 3.3e-10 )    N( > 1.5e-11 )

11      Acoustic  N( > 9.2e-10 )    N( > 4.1e-11 )

12      Acoustic  N( > 2.6e-9 )     N( > 1.2e-10 )

13      Acoustic  N( > 7.3e-9 )     N( > 3.3e-10 )

14      Acoustic  N( > 2.1e-8 )     N( > 9.2e-10 )

15      Acoustic  N( > 5.8e-8 )     N( > 2.6e-9 )

16      Acoustic  N( > 1.7e-7 )     N( > 7.3e-9 )

17      Acoustic  N( > 4.7e-7 )     N( > 2.1e-8 )

18      Acoustic  N( > 1.3e-6 )     N( > 5.8e-8 )

19      Acoustic  -----             N( > 1.7e-7 )

20      Acoustic  -----             N( > 4.7e-7 )