***** File EPAS.DOC

THE ISEE-3/ICE  ENERGETIC PARTICLE  ANISOTROPY SPECTROMETER (EPAS)

INTRODUCTION

The ISEE-3/ICE	spacecraft encountered comet Giacobini-Zinner (G-Z)

on 11 September 1985 and obtained detailed observations of particle

and fields  in the  cometary environment.   To date, it is the only

spacecraft to  have passed  through a  cometary  plasma  tail.	The

Energetic Particle Anisotropy Spectrometer (EPAS) on the spacecraft

provided  observations	 of  the   3-dimensional  distribution	 of

energetic (> 65 keV) cometary ions.  These ions were detected up to

4 million km from  the comet  nucleus and  the maximum	ion  fluxes

attained during  the encounter were among the highest ever observed

by the	instrument.   The  ions  originate  as	cometary  neutrals,

principally  water  molecules,	which  sublime	from  the  cometary

nucleus at  typical speeds of 1 km/s.  They are subsequently photo-

dissociated and  ionized, either by charge-exchange with solar wind

protons or  by solar  UV radiation,  on time  scales  of  around  1

million seconds,  resulting in	a cloud  of cometary ions extending

several million  km from the comet.  The ions are then picked up by

the solar  wind flow and accelerated to energies of several tens of

keV, allowing them to be detected by EPAS.  However, ions with much

higher energies, extending up to around 400 keV were also detected,

in particular in the vicinity of, and downstream from, the cometary

bow shock,  indicating that  further acceleration  must  have  been

taking place in addition to solar wind pick-up.

THE ISEE-3/ICE MISSION

The International  Sun-Earth  Explorer-3  spacecraft  (ISEE-3)	was

launched on  12 August 1978  and initially  placed in  a halo orbit

around	the  L1  libration  point,  some  240 Re  (1.5	million km)

upstream of  the Earth.  In late 1982-late 1983, the trajectory was

changed to  one which took the spacecraft into the geomagnetic tail

using a  series of  lunar-gravitationally assisted orbits.  Then in

December 1983,	ISEE-3, renamed the International Cometary Explorer

(ICE), made  a close  encounter with  the moon and was ejected from

the Earth-moon	system to  make a  pass through  the tail  of comet

Giacobini-Zinner on  11 September (DOY 254) 1985.   The  spacecraft

travelled approximately  from south  to north relative to the comet

at 21 km/s.   Closest approach to the comet nucleus (7800 km on the

tailside) occurred  at 1102 UT.  The cometary bow shock was crossed

inbound and outbound at approximately 0927 UT and 1220 UT.  Further

details of  the encounter  and initial	results  can  be  found  in

Science, 232, 1986.

The  ISEE-3/ICE	 Project Scientist  was  originally  Dr. Tycho  von

Rosenvinge  and  is currently  Dr. Keith Ogilvie, Code 692, Goddard

Space Flight Center.

THE ISEE-3/ICE ENERGETIC PARTICLE ANISOTROPY SPECTROMETER (EPAS)

The  Imperial	College,  London/   Space  Science  Dept.  of  ESA,

Noordwijk/ Space  Research Laboratory,	Utrecht, Energetic Particle

Anisotropy  Spectrometer   (EPAS)     on  the	spacecraft  (ISEE-3

experiment designation	DFH,  Principal  Investigator,	Prof.  R.J.

Hynds of  Imperial College) measures the 3-dimensional distribution

of energetic ions about the spacecraft.  It consists of a system of

three identical  semi-conductor particle  telescopes mounted on the

body of  the spacecraft  and inclined  at 30 degrees (Telescope 1),

60 degrees (Telescope  2) and  135 degrees  (Telescope	3)  to	the

spacecraft spin  axis which  is  maintained  perpendicular  to	the

ecliptic plane (to within 1 degree).  The spacecraft spin period is

3.04 s.   Each telescope  has a  conical field	of view  of 16 deg.

semi-cone angle  and a	geometrical factor  of	0.05 cm2 sr.	The

telescopes  detect   ions  (electrons  being  excluded	by  "broom"

magnets) and  measure their  total kinetic  energy (but  not  their

mass) by  each	using  a  stack  of  two  silicon  surface  barrier

detectors.   The front	detector (A) is  33 micrometers thick while

the second (B)	is 150 micrometers  thick.    Particle	counts	are

defined by  anticoincidence (A.not B),	i.e. the  ions deposit	all

their energy in the A detector and do not intercept and trigger the

B detector.  The amplitude of the signal produced in the A detector

is dependent  on the  energy deposited in the silicon, and hence on

the incident  ion energy.   This  signal is  fed  to  pulse  height

discriminators which  define 8	primary energy	channels, E1 to E8.

In addition, a further channel, E0, monitors the instrument thermal

noise but can register ions above the background if the ion flux is

sufficiently high.  No background noise counting-rate correction is

required in  any of  the primary  energy channels,  i.e. the counts

recorded are  actual particle  counts.	 The channel  energy ranges

depend slightly  on ion  mass.	 This is  due principally  to mass-

dependent energy  losses when  the ions  pass through  a thin  gold

electrode on  the front surface of the A detector.  For protons and

water-group ions  (such as  were observed at comet G-Z), the energy

ranges are:

Protons	 Water Group

(Noise) E0:  21-35	keV	 35-65	keV(approx.)

E1:  35-56	keV	 65-95	keV

E2:  56-91	keV	 95-140 keV

E3:  91-147 keV	140-205 keV

E4: 147-238 keV	205-310 keV

E5: 238-384 keV	310-480 keV

E6: 384-620 keV

E7: 620-1000 keV	(From Hynds et al.,

E8: 1000-1600 keV	Science 232, 361, 1986)

(The water-group  energy ranges  for channels  E6-E8 have  not been

determined.)

In this  G-Z data set, observations from Channels E1 to E5 only are

included since cometary ions were not detected in the higher energy

channels.    For  each	of  these  channels,  particle	counts	are

accumulated in	eight azimuthal  sectors of  45 deg. width  in each

telescope. The	sectors are numbered from 1 to 8 anticlockwise when

viewed from  above the ecliptic, with  Sector 1 centred on the sun-

Earth line, i.e. it views in the sunward direction and detects ions

streaming away	from the  Sun.	 At Giacobini-Zinner, a complete 3-

dimensional distribution  function was	obtained at  32 s intervals

during the  main encounter  phase,  changing  to  64 s	at  greater

distances e.g. after 1830 UT on 12 September outbound.

Cometary pick-up ions at large distances (millions of km) from the

nucleus are characterized by highly anisotropic distributions, with

large fluxes in the sunward viewing sectors (1,2,8).  Closer to the

comet, within  around 500,000 km  or 6 hours  of closest  approach,

broader distributions  with a  antisolar  streaming  are  observed.

Here enhanced  solar  wind  turbulence	excited  by  cometary  ions

scatters the  ions into near isotropy in the solar wind frame.	The

observed anisotropy  is then  due to  the ions	convecting with the

solar wind.

Further details  of the  EPAS instrument can be found in: Balogh et

al., IEEE Trans. on Geoscience Electronics, GE-16, No. 3, 176, 1978

and van Rooijen et al., Space Science Instrumentation, 4, 373, 1979.

Preliminary observations  from the G-Z encounter are given in Hynds

et al.,  Science, 232,	361, 1986.  The instrument configuration is

also illustrated in Richardson et al., Planetary and Space Science,

36, 1429, 1988.

Contact addresses:

Profs. R.J. Hynds/S.W.H. Cowley,	Dr. I.G. Richardson,

Blackett Laboratory,			Code 661,

Imperial College,			NASA/Goddard Space Flight

Center,

London, SW7 2BZ,			Greenbelt,

U.K.					MD 20771,

USA.

SPAN LHEAVX::RICHARDSON

Drs. K.-P. Wenzel/T.R. Sanderson,

Space Science Dept. of ESA,

2200 AG Noordwijk,

The Netherlands

SPAN ESTCS1::KWENZEL or TSANDERS

==================================================================

REFERENCES

Balogh, A., van Dijen, G., van Genechten, J., Henrion, J.,  Hynds,

R., Korfmann, G., Iversen, T., van Rooijen, J., Sanderson, T.,

Stevens, G., Wenzel, K.-P., Low energy proton experiment on ISEE-C,

IEEE Trans. Geosci. Electron., GE-16, No. 3, 176-180, July 1978.

Daly P.W., Sanderson, T.R., Wenzel, K.-P., Survey of  energetic (E

> 35 keV) ion anisotropies in the deep geomagnetic tail,  J.

Geophys. Res., 89, No. A12, 10733-10739, 1984.

Hynds, R.J., Cowley, S.W.H., Richardson, I.G., Sanderson,  T.R.,

Tranquille, C., Wenzel, K.-P., Daly, P.W., Energetic ion

observations during comet Giacobini-Zinner encounter, Adv. Space

Res.,  5, No. 12, 17-25, 1986.

Hynds, R.J., Cowley, S.W.H., Sanderson, T.R., Wenzel, K.-P.,  Van

Rooijen, J.J., Observations of energetic ions from Comet

Giacobini-Zinner, Science, 232, 361-365, Apr. 1986.

Richardson, I.G., Cowley, S.W.H., Tranquille, C.T., Sanderson,

T.R., Hynds, R.J., Wenzel, K.-P., Daly, P.W., Observations of

energetic water-group ions at comet Giacobini-Zinner: Implications

for  ion acceleration processes, Planet. Space Sci., 35, No. 10,

1323-1345,  Oct. 1987.

Richardson, I.G., Cowley, S.W.H., Moore, V.M., Staines, K.,  Hynds,

R.J., Sanderson, T.R., Wenzel, K.-P., Daly, P.W., Energy  spectra

of energetic heavy ions in the vicinity of comet

P/Giacobini-Zinner, Astron. Astrophys., 187, 276-280, Nov. 1987.

Richardson, I.G., Cowley, S.W.H., Hynds, R.J., Daly P.W.,

Sanderson, T.R., Wenzel, K.-P., properties of energetic water-group

ions in the extended pick up region surrounding comet

Giacobini-Zinner, Planet. Space Sci., 36, No. 12, 1429-1450, 1988.

Richardson, I.G., Cowley, S.W.H., Hynds, R.J., Daly, P.W.,

Sanderson, T.R., Wenzel, K.-P., Energetic cometary ion flows in the

pick-up region of Comet giacobini-Zinner, Adv. Space Res., 9,

(3)381-(3)384, Mar. 1989.

Richardson, I.G., Cowley, S.W.H., Wenzel, K.-P., Scarf, F.L.,

Plasma wave, magnetic field and energetic ion observations in the

ion  pick-up region of Comet Giacobini-Zinner, Adv. Space Res., 9,

(3)377-(3)380, Mar. 1989.

Sanderson, T.R., Reinhard, R., Wenzel, K.-P., The propagation  of

upstream protons between the earth's bow shock and ISEE 3, J.

Geophys. Res., 86, No. A6, 4425-4434, 1981.

Staines, K., Balogh, A., Cowley, S.W.H., Hynds, R.J., Yates,  T.S.,

Richardson, I.G., Sanderson, T.R., Wenzel, K.-P., McComas, D.J.,

Tsurutani, B.T., Cometary water-group ions in the region

surrounding  comet Giacobini-Zinner: Distribution functions and

bulk parameter  estimates, Planet. Space Sci., 39, No. 3, 479-506,

1991.

Stevens, G.A., van Rooijen, J.J., The distribution function of

upstream energetic particle populations, J. Geophys. Res., 86, No.

A6, 4435-4444, 1981.

Van Rooijen, J.J., van Dijen, G., Lafleur, H., Th., Lowes, P.,  A

low energy proton spectrometer for directional distribution

measurements in space, Space Science Instrumentation, 4, 373-392,

1979.

I.G. Richardson,

Code 661,

Goddard Space Flight Center,

Greenbelt, MD 20771, USA.

=================================================================

FORMAT OF ORIGINAL G-Z ENCOUNTER EPAS DATA AS PROVIDED BY I. G.

RICHARDSON

Data are  provided for	8-14 September (DOY  251-257) 1985.

The data  are time  ordered and  provided at the highest resolution

available, i.e.  32 s averages	for the main part of the encounter,

and 64 s for the outer region (after 1830 UT on 12 September).	For

each time period, the data are provided in the following format:

RECORD 1: Time record.

Year; Day of year; Hours; Minutes; Seconds.

Format:  2I4, 3I3

RECORDS 2  to 16:  Sectored data  for 3 telescopes x Channels E1 to

E5.

Telescope no.;  Channel no.; No. of spins; Counts in Sectors 1

to 8.

Format:  I2,I3,8F8.0

No. of  spins =  number of spacecraft spins (of 3.04 s period)

over which the particle counts are accumulated.

The number  of counts  in each  sector is the actual number of

counts accumulated during this number of spacecraft spins.

RECORD 17: Direction-averaged ion intensities (using 3 telescopes x

8 sectors) for Channels E1 to E5, in units of counts/(cm2-sec-sr-keV)

(assuming water group ions) and Channel E1 ion streaming direction,

relative to the spacecraft.

Direction-averaged ion intensities in Channels E1 to E5; Theta;

Phi

Format:  5E10.2, 2F7.1

Theta  ranges   from  0 degrees   (northward   streaming)	 to

180 degrees (southward).

Phi = 0 deg. corresponds  to sunward  streaming, and increases

anti-clockwise as	viewed from  above the ecliptic.  Thus, for

example, for  antisolar streaming	in the ecliptic, theta = 90

deg. and phi = 180 deg.

The ion  streaming direction  is derived  from the first-order

term of a third-order spherical harmonic fit to the log of the

sectored data  in Channel E1  (Daly et  al., J. Geophys. Res.,

89, 10733, 1984).

Variations in  the ion  streaming direction  at G-Z  and their

interpretation have  been	discussed  in  Richardson  et  al.,

Geophys. Res. Lett., 13, 415, 1986, Daly et al., Geophys. Res.

Lett. 13, 419, 1986 and Richardson et al., Planet. Space Sci.,

36, 1429, 1988.

N.B.:

1) -999 is used as a "fill" value throughout.

2) Occasionally,  an erroneous large "number of counts" is recorded

in a  sector.	An attempt  has been  made to remove these bad data

points, but a few may still remain.

===================================================================

ORIGINAL DATA SAMPLE

1985 251  0 43 22

11 10      1.      0.      0.      1.      0.      0.      1.      1.

21 10      2.      0.      0.      0.      0.      0.      2.      0.

31 10      0.      2.      0.      1.      1.      2.      0.      1.

12 10      0.      0.      0.      0.      1.      0.      1.      0.

22 10      0.      1.      0.      0.      0.      0.      1.      0.

32 10      1.      2.      0.      0.      1.      1.      0.      0.

13 10      0.      0.      1.      0.      1.      0.      1.      0.

23 10      0.      0.      0.      0.      0.      0.      0.      0.

33 10      0.      1.      0.      1.      0.      1.      0.      0.

14 10      0.      0.      0.      0.      0.      0.      0.      0.

24 10      0.      0.      0.      0.      0.      0.      0.      0.

34 10      0.      0.      0.      0.      0.      0.      0.      0.

15 10      0.      0.      0.      0.      0.      0.      0.      0.

25 10      0.      0.      0.      0.      0.      0.      0.      0.

35 10      0.      0.      0.      0.      0.      0.      0.      0.

0.11E+00  0.47E-01  0.11E-01  0.00E+00  0.00E+00 -999.0 -999.0

======================================================================

SAMPLE CALCULATION OF DIRECTION AVERAGED ION INTENSITIES

1985 251  0 43 22    Channel 1

------------------------------------------------------------------------

Tel.  Ch.  No. of              Ion Counts in Sector               Total

No.   No.  spins      1    2    3    4    5    6    7    8     counts (N)

-------------------------------------------------------------------------

1     1     10       1.   0.   0.   1.   0.   0.   1.   1.        4

2     1     10       2.   0.   0.   0.   0.   0.   2.   0.        4

3     1     10       0.   2.   0.   1.   1.   2.   0.   1.        7

-------------------------------------------------------------------------

Normalized weighting factors (W) for telescopes:

Telescope 1:  W(1) = -0.0351

Telescope 2:  W(2) = +0.6316

Telescope 3:  W(3) = +0.4036

Directional (weighted) average ion counts per telescope for channel 1:

N(1)*W(1) + N(2)*W(2) + N(3)*W(3) =

4*-0.0351 + 4*0.6316  + 7*0.4036  = 5.2112

Directional averaged ion intensities for channel 1:

wtd. avg. ion counts

Ion Intensity = -------------------------------------------------

geometric factor  * integration time * bandwidth

geometric factor = 0.05 cm2-sr (for each telescope);

integration time = No. of spins * spin period = 10*3.04 = 30.4 s

bandwidth = delta-E = 95-65 = 30 keV for channel 1 (water-gp. ions)

5.2112

Ion Intensity = -----------------  =  0.114 ions per (cm2-s-sr-keV)

0.05 * 30.4 * 30

3) In a few records at high fluxes, the Telescope 2, sector 1, data

for energy channel 1 are set to fill.  This is because an

unexpectedly high particle count caused an integer overflow

(count > 32767) during initial data processing.  Unfortunately, the

original data points cannot be recovered for this archive.  (The

corresponding direction-averaged intensities in EPSUMMRY.TAB have

been set to "fill".)

=============================================================================

These files were reformatted to allow for PDS to adequately describe the data.

The original records 1 (time), and 2-16 (sector data) were gathered into

individual ".tab" files for each appropriate date.   The record 17 data

(direction averaged ion intensities) along with the corresponding time were

collected into a separate file as a "summary table" (EPSUMMRY.TAB).

Sample of Reformatted EPAS Ion Count Data

1985 251  0 43 22 11 10      1.      0.      0.      1.      0.      0.      1.      1.

1985 251  0 43 22 21 10      2.      0.      0.      0.      0.      0.      2.      0.

1985 251  0 43 22 31 10      0.      2.      0.      1.      1.      2.      0.      1.

Sample of Reformatted Ion Intensities Summary Table

1985 257  0 43 22  1.10E-01  4.70E-02  1.10E-02  0.00E-02  0.00E+00 -999.0 -999.0

1985 257  0 43 54  1.10E-01  3.50E-02  1.20E-02  0.00E+00  0.00E+00 -999.0 -999.0

1985 257  0 44 26  0.00E+00  0.00E+00  0.00E+00  0.00E+00  0.00E+00 -999.0 -999.0