***** File MAG.DOC

I. BACKGROUND

On September 11, 1985, the International Cometary Explorer (ICE;

originally called ISEE-3) spacecraft made in situ measurements of

particles, fields and waves as it traversed the plasma tail of comet

Giacobini-Zinner (G-Z). The time of closest approach was approximately

11:02 UT and occurred at a tailward distance of 7,800 km. The spacecraft

was inside the bow wave for roughly 3 hours and in the main tail for about

10 min (Brandt et al., 1985;  Brandt et al., 1988, and references

therein).

Data from some of the instruments on the ICE spacecraft are

included on this CD-ROM archive for the period 08-14 September, 1985.

Ephemeris data giving hourly values of the ICE spacecraft trajectory

relative to Earth, Sun and G-Z are included in a separate file. Note

that the deviation of the spacecraft from uniform linear motion was

negligible during the encounter, so that interpolation of the trajectory

is straightforward.

Magnetic field data were obtained by the Jet Propulsion

Laboratory (JPL) magnetometer (Frandsen et al., 1978) which provided

three orthogonal vector measurements every 1/3 sec during the G-Z

encounter. Plasma electron measurements were supplied by the Los

Alamos National Laboratory (LANL) plasma analyzer (Bame et al., 1978)

every 24 seconds. No plasma ion measurements were made during the ICE

traversal of G-Z.

The first scientific results from the ICE encounter with comet

Giacobini-Zinner have appeared in Science (vol. 232, 1986), and further

findings of the mission are contained in the March and April 1986 issues

of Geophysical Research Letters, and in many other sources (e.g., McFadden

et al., 1987; McComas et al., 1987, and references therein).

References:

----------

Bame, S.J., J.R. Asbridge, H.E. Felthauser, J.P. Glore, H.L. Hawk, and

J. Chavez, ISEE-C solar wind plasma experiment, IEEE Trans. Geosci.

Electron., GE-16, 160-162, 1978.

Brandt, J.C., M.B. Niedner, and T. von Rosenvinge, "The ICE Project,"

Adv. Space Res., v.5, 3-16, 1985.

Brandt, J.C., "The International Cometary Explorer (ICE) Mission to

Comets Giacobini-Zinner and Halley" in "Space Missions to Halley's Comet",

p. 99-115, R. Reinhard, B. Battrick (Editors), ESA SP-1066, ESA

Publications Division, Noordwijk, The Netherlands, ISBN 0379 6566, 1986.

Brandt, J.C., R.W. Farquhar, S.P. Maran, M.B. Niedner, Jr., and T.T.

von Rosenvinge, "The International Cometary Explorer" in "Exploration

of Halley's Comet. Based on the 20th ESLAB Symposium, held in

Heidelberg, F.R.G., 27-31 October 1986," p. 969-980.

M. Grewing, F. Praderie, R. Reinhard (Editors). Springer-Verlag,

Berlin, F.R.G. 23+984 pp. ISBN 3-540-17344-7 (F.R.G.), ISBN

0-387-17344-7 (USA), 1988.

Frandsen, A.M.A., B.V. Connor, J. van Amersfoot, and E.J. Smith, The

ISEE-C vector Helium magnetometer, IEEE Trans. Geosci. Electron.,

GE-16, 195-198, 1978.

McComas, D.J., J.T. Gosling, S.J. Bame, J.A. Slavin, E.J. Smith, and

J.L. Steinberg, The Giacobini-Zinner magnetotail: Tail configuration

and current sheet, J. Geophys. Res., v.92, 1139-1152, 1987.

McFadden, L.A., M.F. A'Hearn, P.D. Feldman, H. Bohnhardt, J. Rahe,

M.C. Festou, J.C. Brandt, S.P. Maran, M.B. Niedner, A.M. Smith, and

D.G. Schleicher, Ultraviolet spectrophotometry of comet

Giacobini-Zinner during the ICE encounter, Icarus, v.69, 329-337,

1987.

_____________________________________________________________________________

-----------------------------------------------------------------------------

II. MAGNETIC FIELD DATA

These data were obtained from the JPL magnetometer experiment on ICE

(Principal Investigator: E.J. Smith; data provided by J. Wolf). The

instrument produces three, high-accuracy, triaxial measurements per

second of the  magnetic field strength in 8 ranges, i.e., +/-4 nT

(lowest full  range), 14, 42, 144, 640, 4000, 22000, and 140000 nT

(highest full range) and a sensitivity of 1/256 of each full range,

in a 0-3 Hz pass band. During the G-Z encounter the instrument range

was switched automatically between the 4 lowest ranges depending on

the field intensity, giving sensitivities of 0.015, 0.051, 0.17 and

0.57 nT respectively.

There is one ASCII data file for each day (Sept. 10-14, 1985). All the data

files have an MS-DOS file name extension of .TAB (for 'Table'). Each logical

record contains the year, day-of-year (day 1 is Jan. 1), hours, minutes,

seconds, Bx, By, Bz, and Bmagnitude in the format (1X, I2, I4, 2I3, F6.2,

4F8.3, 2X). Thus, each logical record has the following structure:

Item   Bytes        Quantity         Format

----   -----        --------         ------

0    1            blank              1X

1    2-3          Year (e.g., 85)    I2

2    4-7          Day of year        I4

3    8-10         Hour               I3

4   11-13         Minute             I3

5   14-16         Seconds            F6.2

6   20-27         Bx                 F8.3

7   28-35         By                 F8.3

8   36-43         Bz                 F8.3

9   44-51         Bmag               F8.3

10   52-53         blank              2X

(plus a carriage return and line field makes each row 55 bytes).

The time is UT and the magnetic field values are in nanoteslas.

Example: the first 2 logical records of the first day's data should appear

as follows after separation from the byte stream:

YR DAY  H  M   SEC    Bx      By       Bz     Bmag

>85 253  0  0   .04  -3.684   6.139   1.808   7.385  <

>85 253  0  0   .38  -3.598   6.156   2.001   7.406  <

The time resolution is 1/3 second from the start of Day 253 until Day 255,

18:38. At that time the bit rate dropped from 1024 to 512 bps, and the

time resolution decreased to 2/3 sec. The data coverage and approximate

file sizes are as follows:

File 1:   1985 Sept. 10, Day 253     91%    12.9 MBytes

File 2:   1985 Sept. 11, Day 254     99%    14.1 MBytes

File 3:   1985 Sept. 12, Day 255     98%    12.4 MBytes

File 4:   1985 Sept. 13, Day 256     95%     6.7 MBytes

File 5:   1985 Sept. 14, Day 257     83%     5.9 MBytes

The coordinate system for the magnetic field components is the I,S

coordinate system, defined by the JPL group before launch. I is the unit

vector in the direction of the ICE spin axis (positive in the northward

direction), and S is the unit vector from the spacecraft to the Sun.  The

z-axis is equal to I, the y-axis to the normalized cross-product I x S,

and the x-axis to Y x Z.  The I,S coordinate system is approximately the

same as the Solar Ecliptic (SE) system since the spacecraft z-axis is

maintained within half a degree of perpendicular to the Ecliptic plane.

(SE is defined the same way as GSE, but with the spacecraft [point of

observation] substituted for Earth).

While the JPL reduction program eliminates most noise spikes, there is a

small number that do appear in these data. JPL does not hand-edit the high

resolution data, since there is such a volume of it. Users should check

for noise spikes when doing averaging or other analyses. There is no

special flag for "bad" or missing data. A scan of the 5 days of high

resolution data at JPL revealed the following noise spikes:

Time              Field magnitude

----              ---------------

Day 253, 10:11:59.84        47.25

Day 254, 03:51:28.21        59.17

04:05:17.84        56.19

10:10:55.31        59.68

11:06:13.15       128.07

12:04:23.92        62.73

Day 255, 09:07:17.48        57.75

09:07:17.82       185.49

Day 256, 07:35:54.94        57.58

21:08:09.00        23.83

23:01:15.94        35.20

Day 257, 10:08:07.77        46.06

20:03:15.75        51.07

The following information (and the table of noise spikes given above) has

been provided by J. Wolf (JPL), after examining plots of the magnetic field

data during the comet encounter: The highest legitimate value of Bmagnitude

is 59.21 at 11:01:01.29 on Day 254, at the time of closest approach to the

comet. On Day 254, one should eliminate points with Bmagnitude greater

than 40 from 09:00 to 11:00 and from 11:10 to 13:00. During the interval

11:00 to 11:10, points with Bmagnitude greater than 60 should be eliminated.

For the rest of this 5 day interval, one should eliminate points with

Bmagnitude greater than 20 nT.