MISSION: VEGA-1 AND VEGA-2

INSTRUMENT: PLASMAG-1 electrostatic plasma analyzer

Mass: 9 kg

Dimensions: non applicable

Date built: 1983-84

Manufacturer: Space Research Institute, Moscow, USSR

Central Research Institute for Physics, Budapest, Hungary

Instrument overview

The PLASMAG-1 instument package included six different sensors:

A0 plasma impact detector for measuring neutral particle flux,

(manufactured by R. Grard, ESA/ESTEC, Noordwijk, The Netherlands)

A1 Faraday cup for measuring integral ion flux from solar direction,

A2 Faraday cup for measuring integral ion flux and neutral particles

from ram direction,

A3 spherical electrostatic analyzer for measuring ions from ram direction

in the energy range of 15 eV - 3.6 keV in 120 logarithmically

spaced channels,

A4 spherical electrostatic analyzer for measuring ions from solar direction

in the energy range of 60 eV - 30 keV in 60 logarithmically

spaced channels,

A5 cylindrical electrostatic analyzer for measuring ions perpendicular

to the ecliptic plane in the energy range of 2 eV - 10 keV in

30 logarithmically spaced channels.

Scientific objectives

The electrostatic ion analyzers A3 and A4 were designed to study

the change of the energy distribution of the solar wind and cometary

ions as a function of the distance from the nucleus. The

detector oriented in the ram direction could determine the mass

distribution of cometary ions close to the nucleus with a

resolution of m/dm=20-25.

The ram Faraday cup A2 had four modes of operation. Two of these

provided information on the neutral particle flow from the comet.

For further informations on instrument overview and scientific

objectives (also first results) see the following papers:

1) Gringauz et al., The Vega probe instrument for measuring

charged particles with energies less than 25 keV, in "Cometary

exploration Vol. III., ed. by T.I. Gombosi, pp. 333-350, Central Res.

Inst. Physics, Budapest, 1982.

2) Gringauz et al., First in situ plasma and neutral gas measurements

at comet Halley, Nature 321, 282-285, 1986.

3) Apathy et al., Plazmag-1 experiment, in Exploration of

Halley's comet, ESA SP-250, Vol. I., ed. by B. Battrick, E.J.

Rolfe, and R. Reinhard, pp. 65-70, Paris, 1986.

Operational considerations

The sensitivity of the electron analyzer A5 was very low.

Detectors A3 and A5 of Vega-1 were fatally damaged during the cruise phase.

Detector A3 of Vega-2 was fatally damaged 2.5 min before closest approach.

The channeltrons of A3 were saturating at higher count rates.

Calibration description

The sensors were calibrated at the Max-Planck-Institut for Aeronomy

in Katlenburg/Lindau, Germany. The viewing angle and energy resolution

of the electrostatic analyzers were determined in vacuum chamber

using monoenergetic ion and electron beams, respectively.

(For further details see reference #3 above.)

The spectrometers used Soviet-made channeltrons as particle detectors.

An in-flight calibration was performed every day to select the

optimum voltage for the channeltrons.

A post-launch calibration was performed in order to correct the saturating

count rates of detector A3 of Vega-2 using the integral fluxes provided

by the ram Faraday cup A2.

Principal investigator: K.I. Gringauz, Space Res. Inst., Moscow,

Co-principal investigator: T.I. Gombosi, Cent. Res. Inst. Physics, Budapest,

Co-investigators: A.P. Remizov, M.I. Verigin, L.I. Denchikova, A.V. Dyachkov,

I.N. Klimenko, G.A, Vladimirova, Space Res. Inst., Moscow

I. Apathy, T. Szemerey, S. Szendro, A.J. Somogyi, K. Szego,

M. Tatrallyay, A. Varga, Cent. Res. Inst. Physics, Budapest,

E. Keppler, Max-Planck-Inst. Aeronomy, Lindau, Germany.

Instrument section

Field of view of the detectors: A1: 90x90 deg (centered on solar direction),

A2: 26x26 deg (centered on ram direction),

A3: 32x12 deg (centered on ram direction),

A4: 36x32 deg (centered on solar direction),

A5: 7x7 deg (perpendicular to ecliptic plane).

Effective apperture: A1: 1.6 cm**2

A2: 0.93  cm**2

A3: 0.0193 cm**2

A4: 0.00455 cm**2

A5: 0.0036  cm**2

Energy resolution:   A3: 3.2 %

A4: 3.85%

A5: 7.5 %

One complete spectrum was taken by A3, A4, and A5 in every

second during the BRL (high telemery rate) seance.

Sampling times for each energy channel:

A3: 0.005 sec

A4: 0.005 sec

A5: 0.021 sec

During the cruise phase data were recorded by an on-board

magnetometer at lower telemetry rate.

DATA SET - VEGA-1

The data were measured during the BRL seance on March 6, 1986.

1. parameter: time in sec - end of a 10 sec averaging interval

(0 sec is 4:00 UT in spacecraft time)

2-60. parameters: count rates observed by A4 (ions from solar direction)

The energy levels for the 59 channels of detector A4 in eV:

70.4    75.7    87.1    90.6   101.2   113.5   123.2   131.6   150.5   167

2

183.0   204.2   226.2   253.4   281.6   308.0   341.4   380.2   423.3   471

7

522.7   583.4   650.3   722.5   799.0   887.9   988.2  1103.5  1223.2  1360

5

1511.8  1672.9  1859.4  2074.2  2289.8  2556.4  2824.8  3147.8  3497.1  3899

3

4331.4  4842.6  5380.3  6000.7  6651.0  7302.2  8132.1  9081.6 10137.6 11308

0

12496.0 13921.6 15576.0 17344.8 19228.0 21348.8 23830.4 26435.2 29515.2

Measurements were performed in every sec, data provided here

are 10 sec averages.

Time of first observation:  4:17:29 UT

Time of first data in file: 4:17:38 UT (1058 sec)

Time of last observation:   8:25:08 UT

Time of last data in file:  8:25:08 UT (15908 sec)

There was no longer data gap than a few seconds.

Missing or bad data: -999

Detectors A3 and A5 were fatally damaged before the encounter.

DATA SET - VEGA-2

The data were measured during the BRL seance on March 9, 1986.

1. parameter: time in sec - end of a 10 sec averaging interval

(0 sec is 4:00 UT in spacecraft time)

2-30. parameters: count rates observed by A5 (electrons)

31-90. parameters: count rates observed by A4 (ions from solar direction)

91-210. parameters: count rates observed by A3 (ions from ram direction)

The energy levels for the 29 channels of detector A5 in eV:

1.9     2.9     3.8     5.8     7.7    10.6    14.9    19.2    25.9    35

0

47.0    61.0    84.5   113.3   168.2   229.7   311.3   410.9   553.0   744

8

999.2  1349.0  1804.8  2430.2  3225.2  4364.7  5869.9  7868.0 10571.0

The energy levels for the 60 channels of detector A4 in eV:

64.0    73.8    78.7    88.6    94.3   103.3   114.0   128.7   138.6   154

2

173.8   188.6   209.1   235.3   255.0   286.2   317.3   351.0   387.0   431

3

480.5   534.6   594.5   660.9   734.7   818.4   912.7  1014.3  1123.4  1246

4

1384.2  1531.8  1689.2  1877.8  2099.2  2341.1  2606.0  2896.2  3214.4  3575

2

3985.2  4436.2  4936.4  5494.0  6117.2  6724.0  6724.0  7544.0  8388.6  9290

6

10291.0 11439.0 12783.8 14268.0 15924.4 17728.4 19647.2 21861.2 24403.2 27191

2

The energy levels for the 120 channels of detector A3 in eV:

15.6    17.2    18.9    19.7    20.5    22.1    23.0    24.6    27.1    28

3

29.5    30.8    32.0    33.2    34.4    35.7    36.9    38.1    39.4    40

6

41.8    43.5    45.1    45.9    46.7    48.0    50.0    51.3    52.5    54

9

57.4    59.4    61.5    64.0    66.8    69.7    73.0    76.7    80.8    84

9

89.4    93.9    98.4   102.9   107.4   112.3   117.7   123.0   128.7   134

5

140.6   147.2   154.2   161.5   169.3   177.1   185.3   194.3   203.8   213

2

223.0   234.1   245.6   257.5   269.8   282.9   296.4   310.0   323.9   337

8

352.2   366.5   381.3   398.5   418.2   440.3   464.1   487.9   512.5   537

1

561.7   587.9   615.8   645.3   676.5   710.9   747.8   787.2   828.2   870

8

914.3   959.4  1005.3  1052.9  1102.1  1152.1  1203.8  1262.0  1325.1  1393

2

1466.2  1539.1  1612.9  1687.6  1763.0  1845.0  1935.2  2033.6  2140.2  2248

4

2358.3  2469.8  2583.8  2702.7  2829.8  2966.8  3112.7  3268.5  3432.5  3604

7

Measurements were performed in every sec, data provided here

are 10 sec averages.

Time of first observation:  4:13:00 UT

Time of first data in file: 4:13:10 UT (790 sec)

Time of last observation:   8:40:00 UT

Time of last data in file:  8:40:00 UT (16800 sec)

There was a data gap between 7:17:30 UT (11850 sec) and 7:45:10

(13510 sec).

Missing or bad data: -99

The detector A3 was fatally damaged at 7:17:31 UT when the telemetry

connection also broke down.

At 7:45:10 UT the connection with the instrument was re-established.