***** File MODE_TAB.DOC

Mission: VEGA 1

Instrument: PUMA

Submission: MPI Text Files

Original Format

The data for these files were received as a series of text files, each of

which consisted of spectra of a single mode.  Amplitude-triggered samples

in the spectra were indicated by an "X" following the sample.

Reformatting at SBN

The following changes were made to the data:

o  A program was written to parse the headers and spectra and convert

the ASCII values to binary form.  Parameter values were ordered to

facilitate comparison between the MPI and IKI submissions.

o  Flagged amplitudes, followed by an "X" in the text file, were

indicated in the binary file as a additional byte set equal to 1

for flagged amplitudes, 0 otherwise.  This is the same system used

for the IKI submissions.

o  All spectra of the same mode were collected into a single, fixed-

length table.

o  The original order of the spectra in the text files, as well as the

order of the text files themselves, was preserved in the combined

mode tables.  This order was assumed to be chronological.

Known Anomalies/Problems:

o  Little or no explanation was available for some of the parameters in

the spectrum headers.  This is indicated in the DESCRIPTION part of

the PDS label for these datasets.

o  There are more spectra in this submission than IKI submissions.

While there is a large amount of overlap between the IKI and MPI

submissions, each contains spectra not included in the other.

o  It has not yet been possible to translate the times in this file,

which are apparently in units of spacecraft clock time, to UT.

o  A number of mode 3 test spectra were found which contained an odd

number of bytes in the spectrum.  In this mode the number of bytes

should be even, as each sample in the spectrum consists of a time

sample byte and an amplitude sample byte.  In these cases the single

byte was assumed to be a time value and a null amplitude byte was

appended to the spectrum.

o  Although the text files were each supposed to contain spectra of only

a single mode, in one file of test mode 3 spectra a single mode 1

spectrum was found.  It was not possible to unambiguously assign a

location for this spectrum within the mode 1 table, so it was omitted

from the binary mode tables.  The text form of this spectrum follows:

SEQ  3098 LLEN   211  SEN HHHHH FIS  1  TEL  4/1

ENR  3334 TIME  3341 TIL  14    TGC   6 TPOS   0

HV  3504  MO  7 CA 17 AC  2 TG  1 PM  2 COI 1/1

HVN 3596  NAC  20 NTG   3 NPM  17      TP 19

CC  78/  5/  0/191/197 IR   1 IL   0 I4   0 CCE  0

SPECTRUM NA   3 N0  65 MA  92

23    3   23    8X  25    6X  25    6X  26    6X  27    5X

31    3   33    8X  33    3X  33    7X  35   26X  35    4X

35   14X  37    3X  38   11X  40    7X  40   11X  41    4X

46    6X  47    4X  51    6X  52    8X  52    6X  58   12X

58   26X  59    6X  66    6   70    6X  71    6X  72   20X

73    3X  80    6X  80    6X  81    3X  83   18X  84    3X

92    3   94    9X  95    6X  99    6X 100   10X 101    4X

107    9X 108    6X 109    7X 110    4X 119    4  128    3

135    3  145    3  153    7X 154    5X 165    5  165   11X

167    6X 178    4  180    6X 181    6X 192   16X 192    6X

200    8X 202    6X 205    7X 206    5X 218    3  222   12X

222    7X 235    4  239   19X 239    9X 252    4  255    4

255    7X 255    6X 255    9X 255    6X 255  127  234   92X

32   99    7    0X 116   31  241    0X   0    0X  78    5X

0   63  199   67   86  120  180   87    0    8X